OPERATING & SAFETY MANUAL
ROBOTIC SEALANT DISPENSER MACHINE
OEM: Rani Enterprises  |  TIER C DRAFT

| Document Title | Operating & Safety Manual — Robotic Sealant Dispenser Machine |
| --- | --- |
| Document Number | OSM-P7-RANI-V1.0 |
| Revision / Date | V1.0 (July 2026) |
| Author / Designation | Certified Safety Professional |
| Document Status | Antigravity Sample / Draft |

TABLE OF CONTENTS

| Section | Title |
| --- | --- |
| Section 1 | 1. Front Matter & Document Control |
| Section 2 | 2. Machine Overview & Functional Description |
| Section 3 | 3. Hazard Identification & Risk Assessment Summary |
| Section 4 | 4. Lockout / Tagout (LOTO) Procedures |
| Section 5 | 5. Emergency Stop Function |
| Section 6 | 6. Operating Instructions |
| Section 7 | 7. Maintenance & Care |
| Section 8 | 8. HMI Alarms, Control Panel & Operating Modes |
| Section 9 | 9. Safety Features, Interlocks & Compliance Standards |
| Appendix A | Maintenance Log Template |
| Appendix B | LOTO Log Template |

1. Front Matter & Document Control

2. Machine Overview & Functional Description

The Rani Enterprises Robotic Sealant Dispenser Machine is a semi-automated industrial cell engineered for high-precision sealant/glue application onto automotive and industrial components. The machine integrates a Fisnar F4303N ADVANCE 3-Axis gantry robot, a pneumatic sliding front safety door, a Mitsubishi industrial HMI/PLC control architecture, and an automated component sliding tray.
The machine operates via a dual two-hand cycle start control system. Once a component is placed onto the extended tray and the two-hand push buttons are simultaneously depressed, the front door slides downward under pneumatic control, and the Fisnar robot dispenses sealant along pre-programmed toolpaths.
To prevent chemical curing and nozzle tip blockage during non-operational or idle states, the machine features an automated Sealant Parking & Purging mechanism. If the machine remains idle for greater than 1.0 minute (configurable), the robot head automatically returns to the dedicated parking plate and executes a purge cycle into the purging cup.

| Specification Parameter | As-Built / Pilot Value |
| --- | --- |
| Machine Manufacturer (OEM) | Rani Enterprises |
| Robot Gantry Dispenser | Fisnar F4303N ADVANCE 3-Axis Robot |
| Control Interface | Mitsubishi Industrial HMI (GLUE DISPENSER) |
| Pneumatic Supply Pressure | ~6 bar (SMC FRL Assembly) / 2-3 bar Dispenser Inlet |
| Electrical Power Rating | **[PLACEHOLDER — Confirm Voltage / Amps from Nameplate]** |
| Guarding & Interlocks | Omron GLS-M1 Non-Contact Switches (Left & Right Side Doors) |
| Cycle Start Control | Dual Two-Hand Safety Control Push Buttons |

3. Hazard Identification & Risk Assessment Summary

A high-level safety review was conducted in accordance with ISO 12100:2010 risk reduction methodology. Mechanical, electrical, pneumatic, and chemical hazards associated with sealant dispensing were identified and evaluated.
Primary hazard zones include: 1. Mechanical Pinch & Entrapment: Vertical movement of the pneumatic sliding front door and 3-axis motion of the Fisnar robot gantry. 2. Chemical & High Pressure: Sealant nozzle dispensing under pneumatic pressure (2–3 bar) and chemical contact during tip cleaning. 3. Electrical Energy: 240V/24VDC control panel enclosure housing the Mitsubishi PLC and Mean Well power supply.
Safeguarding measures implemented on the machine include dual two-hand cycle controls requiring sustained dual-hand contact during door closure, Omron GLS-M1 non-contact interlock switches on maintenance access doors, and prominent Emergency Stop controls. For full quantitative risk scoring, refer to the formal Machine Safety Assessment Report once executed by Rani Enterprises.

4. Lockout / Tagout (LOTO) Procedures

Energy isolation is mandatory before performing any maintenance, nozzle replacement, cleaning, or internal adjustments within guarded zones.

Energy Isolation Points (Table 4.2)

| LOTO ID | Energy Type | Isolation Method | Location | Verification |
| --- | --- | --- | --- | --- |
| LP-01 | Electrical | Main Salzer Rotary Isolator Switch (LK OFF) | Right side of control panel enclosure | Attempt HMI power ON; verify 0V on panel meter |
| LP-02 | Pneumatic | SMC FRL Air Shut-off Valve + Manual Pressure Relief | Rear pneumatic FRL unit (~6 bar gauge) | Check pressure gauge reads 0 bar; pull relief valve |

5. Emergency Stop Function

The machine is equipped with Category 0 Emergency Stop buttons designed to immediately remove electrical and pneumatic power to motion actuators upon actuation.

Emergency Stop Reset Flowchart (Sec 5.5)

| 1. Clear Hazard | Inspect machine cell and remove all personnel/obstructions. |
| --- | --- |
| 2. Reset Button | Twist and release the actuated Emergency Stop button. |
| 3. Press RESET | Press the blue RESET button on the operator panel. |
| 4. HMI Acknowledge | Clear 'Machine Emergency Pressed' alarm on Mitsubishi HMI. |

6. Operating Instructions

Operating procedures for normal production, component loading, and automatic sealant dispensing.

Normal Operating Sequence Flowchart (Sec 6.4)

| 1. Power ON | Turn Salzer main isolator ON and verify 24VDC PLC power. |
| --- | --- |
| 2. Air Supply | Open SMC FRL valve; confirm 6 bar main / 2-3 bar dispenser air. |
| 3. Load Component | Place raw component into the extended sliding tray fixture. |
| 4. Two-Hand Start | Simultaneously press and hold dual two-hand push buttons. |
| 5. Auto Door Close | Pneumatic front door slides down; dispensing cycle initiates. |
| 6. Dispensing | Fisnar robot executes sealant path; door opens upon completion. |
| 7. Unload Part | Remove finished component from tray. |

Sealant Anti-Drying / Parking & Purging Logic

Sealant Anti-Drying / Purging Protocol:   If no cycle is started within 1.0 minute (configurable on HMI Settings screen), the control system automatically triggers the Purge Routine. The Fisnar robot moves to the dedicated parking position, applies a brief purge shot into the purging cup, and rests over the parking plate to prevent nozzle tip curing.

7. Maintenance & Care

Preventive maintenance schedules to ensure machine reliability and operator safety.

| Frequency | Preventive Maintenance Task |
| --- | --- |
| Daily / Shift Start | Inspect pneumatic FRL water trap bowl; clean sealant nozzle tip and empty purging cup. |
| Weekly | Check Omron GLS-M1 side door safety switches for proper interlock trip function. |
| Monthly | Inspect Fisnar robot linear guides and apply light lubricant; check main Salzer isolator. |
| Six-Monthly | Inspect control panel terminals, Mean Well NDR-240 PSU wiring, and pneumatic tubing. |

8. HMI Alarms, Control Panel & Operating Modes

The Mitsubishi industrial HMI provides machine control, setting configuration, and fault diagnostics.

Key HMI Alarm & Fault Diagnostic Codes

| HMI Alarm Code | Probable Cause | Corrective Action |
| --- | --- | --- |
| Air Pressure Low [X12] | Pneumatic FRL supply pressure dropped below 4.0 bar. | Check main air line and SMC FRL pressure regulator. |
| Front Auto Door open Error [X6] | Front pneumatic sliding door opened during cycle or failed to close. | Check pneumatic cylinder sensor and door guides. |
| Machine Emergency Pressed [X0] | One or more Emergency Stop buttons activated. | Release E-stop, press RESET button, and clear alarm. |

9. Safety Features, Interlocks & Compliance Standards

Safety interlocks and applicable international standards governing the machine.

Applied Compliance Standards:

• ISO 12100:2010 — Safety of machinery — General principles for design — Risk assessment and risk reduction

• ISO 20607:2019 — Safety of machinery — Instruction handbook — General drafting principles

• IEC 82079-1:2019 — Preparation of information for use (instruction handbook)

• ISO 13849-1 — Safety-related parts of control systems

• ISO 13850 — Emergency stop function — Principles for design

• IS 4571:2008 / Factories Act 1948 — Indian Machine Safety Regulations

Appendix A — Maintenance Log Template

| Date | Maintenance Task | Technician Name | Action Taken | Signature |
| --- | --- | --- | --- | --- |

Appendix B — LOTO Log Template

| Date | LOTO Point | Authorized Person | Time Applied | Time Removed | Signature |
| --- | --- | --- | --- | --- | --- |