OPERATING & SAFETY MANUAL

Robotic Sealant Dispenser Machine

Rani Enterprises

| Manufacturer (OEM) | Rani Enterprises |
| --- | --- |
| End User | **[PLACEHOLDER — end-user facility name / site address]** |
| Machine Type | Robotic sealant / glue dispensing machine |
| Document Title | Operating & Safety Manual |
| Document No. | RANI-SEALANT-OSM-001 |
| Revision | V1.0 |
| Date | July 2026 |
| Prepared By | Certified Safety Professional |
| Status | Draft |

CONFIDENTIAL — Draft for review

# Revision History

Four-column revision table (Prepared By column not used).

| Revision | Date | Description | Reviewed By |
| --- | --- | --- | --- |
| V1.0 | July 2026 | Initial draft from site photos and intake notes | Certified Safety Professional |
| V2.0 | TBD | Post customer review / closed open actions | OEM / End User |

# Table of Contents

H1 entries only. After opening in Word: right-click → Update Field → Update entire table if a TOC field is inserted later. This edition uses a static TOC list.

1 Front Matter

2 Machine Overview

3 Hazard Identification

4 Lockout / Tagout (LOTO)

5 Emergency Stop

6 Operating Instructions

7 Maintenance & Care

8 HMI, Alarms & Controls

9 Safety Features, Schematics & Compliance

Appendix A Maintenance Log Template

Appendix B LOTO Log Template

# 1. Front Matter

## 1.1 Document Scope

This Operating & Safety Manual (OSM) provides information for the safe operation, energy isolation, and basic maintenance of the Robotic Sealant Dispenser Machine manufactured by Rani Enterprises.

The machine integrates a Fisnar F4303N ADVANCE robot/dispenser system with a Mitsubishi PLC/HMI cell controller, automatic front door, side-door interlocks, two-hand cycle start, and pneumatic services.

This document is prepared from site photographs and intake notes. A formal Machine Safety Assessment was not available at the time of drafting. Items marked PLACEHOLDER or Open Action must be closed before treating the manual as customer-released.

Preparation follows the principles of ISO 20607:2019, ISO 12100:2010, IEC 82079-1:2019, and IS 4571:2008. No performance-level (PLr) claims are made without an independent assessment.

## 1.2 Intended Audience

Production operators (instructed persons) who load components, start cycles, and unload parts

Maintenance and troubleshooting personnel (skilled persons)

Supervisors and safety representatives

Contractors working near the machine (awareness only)

Untrained persons shall not operate, teach, adjust, or maintain this machine.

## 1.3 General Safety Rules

Read this manual before operating or servicing the machine.

Only trained and authorised persons may operate or maintain the machine.

Never bypass two-hand controls, door interlocks, E-Stops, or guards.

Apply full Lockout/Tagout (Section 4) before any intervention inside the process zone, panel work, or pneumatic work.

Keep hands clear of the robot path, dispenser tip, and automatic front door during motion.

Do not reach into the cell while the robot is running or the front door is closing.

Use parking and purging functions as required to prevent tip dry-out and unplanned interventions.

Report damaged safety devices immediately and take the machine out of service until restored.

Wear PPE required by the end-user site and the sealant Safety Data Sheet (SDS).

## 1.4 Signal Word Hierarchy

| Signal Word | Meaning |
| --- | --- |
| WARNING | Hazardous situation that could result in death or serious injury if not avoided. |
| CAUTION | Hazardous situation that could result in minor or moderate injury if not avoided. |
| NOTICE | Practices not related to personal injury, or important operational information. |
| PLACEHOLDER | Data not available from source inputs; must be confirmed. |
| OPEN ACTION | Tracked gap — see standalone Open Actions register. |

NOTICE DANGER signal-word boxes are not used in this manual.

## 1.5 Prepared By

Prepared by: Certified Safety Professional

Role only — no individual name is used in customer-facing attribution fields of this manual.

# 2. Machine Overview

## 2.1 Machine Identification

| Field | Value |
| --- | --- |
| Machine name | Robotic Sealant Dispenser Machine |
| Manufacturer (OEM) | Rani Enterprises |
| End user | **[PLACEHOLDER — end-user facility name / site address]** |
| Machine type | Robotic sealant / glue dispensing machine |
| Integrated robot | Fisnar F4303N ADVANCE |
| Label observed | ROBOTIC SEALANT DISPENSER MACHINE |
| Model / Serial / Year | [PLACEHOLDER — confirm from nameplate — OA-P7-002] |
| Site address | **[PLACEHOLDER — installation site address]** |

PLACEHOLDER [PLACEHOLDER — Model No., Serial No., Asset No., Year of manufacture, power rating — OA-P7-002] Photo reference: Images/Nameplate.jpg (title label only; no electrical nameplate data).

## 2.2 Technical Specifications

| Parameter | Value |
| --- | --- |
| Electrical supply voltage | **[PLACEHOLDER — supply voltage, source: nameplate / site electrical, status: not on machine label photo]** |
| Phases / frequency | **[PLACEHOLDER — single / three phase]** / **[PLACEHOLDER — typically 50 Hz India; confirm]** |
| Machine power rating | **[PLACEHOLDER — machine power rating kW / A unknown]** |
| Control power | 24 VDC power supply observed (Mean Well NDR-240-24 in panel) |
| PLC | Mitsubishi compact PLC observed in panel (exact model string **[PLACEHOLDER — read PLC faceplate]**) |
| HMI | Mitsubishi Electric touchscreen HMI labelled GLUE DISPENSER |
| PSU observed | Mean Well NDR-240-24 (photo PLC panel open) |
| Main isolator | Lauritz Knudsen OFF-salzer rotary POWER ON/OFF (red handle, yellow pad) |
| Robot | Fisnar F4303N ADVANCE |
| Plant air (nominal) | 6 bar (intake README; FRL gauge photo shows operation near ~6 bar) |
| Dispenser inlet set range (HMI) | 2.00 – 3.00 bar (HMI Settings screen photo) |
| Hydraulic | Not applicable |
| Operating modes | AUTO, MANUAL |
| Models observed on HMI | JPTP, MGTP, APTDPT |
| Cycle time (specified) | **[PLACEHOLDER — customer cycle-time specification unknown]** |
| Cycle time (observed instance) | 12.1 s displayed on HMI Process screen (instance only) |
| Idle time to parking (HMI) | 3.0 minutes |
| Sealant capacity | **[PLACEHOLDER — sealant cartridge/syringe capacity; OA-P7-006]** |
| Axis / guide speed | **[PLACEHOLDER — guide/axis speed unknown]** |

## 2.3 Intended Use

Operator places component on fixture tray. Operator presses dual two-hand cycle-start buttons. Front automatic door closes (slides from top per intake). Fisnar robot applies sealant path for selected model/program. Cycle completes; door opens; operator unloads part. On idle, parking and purging sequence moves tip to parking/purging cups to prevent tip dry-out (HMI Idle Time To Go Parking = 3.0 min as photographed).

Intended users: trained production operators and skilled maintenance personnel.

Reasonably foreseeable misuse includes: reaching into the cell during robot motion or door closure; bypassing two-hand, door, or E-Stop devices; teaching or changing robot programs without authorisation; operating with side doors open or interlocks defeated; running without adequate air pressure; ignoring parking/purge requirements; and performing electrical/pneumatic work without LOTO.

## 2.4 Layout & Key Zones

Operator load/unload face with two-hand stations and automatic front door

Process zone: Fisnar robot, dispenser, fixture tray, parking and purging cups

Left/right side doors with non-contact safety switches

Upper front control face: HMI and indicators

Side control panel (Bhartia BCH) with isolator, FRL, PLC/PSU

Tower light on top structure

NOTICE Image placeholders (customer or later embed): Images/Full view Front.jpg; Full view side.jpg; Component on tray.jpg.

## 2.5 Key Components

Fisnar F4303N ADVANCE cartesian robot base and head

Fisnar blue sealant dispenser cartridge/syringe assembly

Component fixture tray / nest

Parking cup and purging cup for dispenser tip

Automatic front safety door (pneumatically assisted)

Left and right side access doors with Omron GLS-M1 non-contact switch

Dual two-hand cycle-start stations (left RESET + green; right green + E-Stop)

Mitsubishi HMI and PLC cell control

Mean Well 24 VDC power supply (NDR-240-24 observed)

Lauritz Knudsen rotary main isolator (POWER ON/OFF)

SMC FRL with pressure gauge / pressure-switch interface

Status tower light

# 3. Hazard Identification

This section is a narrative summary of hazards reasonably identified from machine construction, photographs, HMI status/alarms, and intake process description. It is not a substitute for a formal Machine Safety Assessment.

OPEN ACTION OA-P7-001: No formal Machine Safety Assessment was available for this machine. An Independent Safety Consultant should perform a Machine Safety Assessment before claiming compliance completeness or performance levels.

Principal hazards associated with this equipment include:

Mechanical / robot motion. The Fisnar robot and dispenser head move within the enclosure. Contact with the moving head, tip, or cable track can cause impact, puncture, or shearing injury. Labels on the robot structure warn operators to keep hands clear of moving parts.

Automatic front door. The front door closes from the top during the cycle (intake description). Crush or impact injury can occur if hands or body are in the doorway during closure. The cell does not include a light curtain at the load face (intake). Residual risk depends on door design, two-hand start discipline, and correct interlock function (OA-P7-005).

Side door access. Left and right side doors provide maintenance access. Omron non-contact switches and HMI side-door status are present. Opening a side door during motion, or defeating the sensor, can expose persons to robot motion.

Pneumatic energy. Plant air (about 6 bar at the FRL photo) drives door and process functions. Unexpected motion or residual pressure after shutdown can injure during maintenance if air is not isolated and dumped.

Electrical energy. The control panel contains mains distribution, a 24 VDC supply, PLC, and relays. Shock and arc hazards exist during live troubleshooting. Electrical work requires isolation at the main POWER ON/OFF switch and verification by a skilled person.

Chemical / sealant. Sealant materials may present skin, eye, or inhalation hazards and may solidify in the tip if idle too long. Product identity and SDS were not provided (OA-P7-006). Parking and purging reduce process failure and secondary unsafe interventions.

Control system faults / low air. HMI live alarms observed include Air Pressure Low, Robot Emergency Pressed [X12], Front Auto Door open Error [X6], and Machine Emergency Pressed [X0]. Operators must treat alarms as stop conditions and clear causes before restart.

Administrative residual risk. Until a Machine Safety Assessment is completed, residual risk ratings and required additional protective measures remain incomplete. This manual uses PLACEHOLDER and Open Actions rather than inventing scores or PLr values.

End of Section 3. Commission a Machine Safety Assessment and update this manual when the report is available.

NOTICE Open Actions are maintained as a standalone Excel file — not inside this manual. File: RaniEnterprises_RoboticSealantDispenser_OpenActions_V1.0.xlsx

# 4. Lockout / Tagout (LOTO) Procedures

## 4.1 Energy Isolation Points

Isolate all hazardous energy before entering the process zone for cleaning near the robot path, tip change, fixture work, panel access, or any task where unexpected motion or energisation could cause injury.

Energy sources: Electrical — supply via main rotary isolator (voltage PLACEHOLDER — OA-P7-002); Pneumatic — plant air via FRL / supply isolation (nominal ~6 bar); Hydraulic — not applicable.

| Point ID | Energy Type | Image Code | Isolation Method | Location |
| --- | --- | --- | --- | --- |
| LP-01 | Electrical | IEC60417-5036 | Main rotary POWER ON/OFF isolator — turn to OFF; apply lockout hasp/padlock if hasp fitted; verify zero energy | Control panel side — labelled POWER ON/OFF (Lauritz Knudsen OFF-salzer; Isolato switch.jpg) |
| LP-02 | Pneumatic | ISO7010-M004 | Isolate plant air supply to machine; dump residual pressure at FRL/downstream path; lock isolation device if lockable | SMC FRL / pressure-switch area on panel side (Pneumatic FRL.jpg) |

PLACEHOLDER [PLACEHOLDER — Confirm lockable air isolation valve and isolator hasp/padlock hardware — OA-P7-007]

## 4.2 LOTO Equipment Required

Personal padlocks (one key per authorised person)

Lockout hasps for multi-person lockout

Danger/lockout tags with name, date, reason

Voltage tester / proving unit (skilled electrical person)

[PLACEHOLDER — site-specific LOTO kit inventory]

## 4.3 Tag-Out Sequence

START

Notify affected personnel

Stop machine in safe state

Turn isolator to OFF

Isolate plant air supply

Dump residual air pressure

Apply locks and tags

Verify zero energy

Perform work

END

## 4.4 Tag-In Sequence

START

Confirm tools removed

Confirm guards doors closed

Remove personal locks tags

Restore air supply

Turn isolator to ON

Reset E-Stops if needed

Function-check safety devices

Return to production

END

# 5. Emergency Stop

## 5.1 E-Stop Devices

Two emergency-stop actuators are photographically confirmed. E-Stop is for emergency conditions. It is not a substitute for LOTO during maintenance.

| ID | Location | Type | Source |
| --- | --- | --- | --- |
| ES-01 | Right front two-hand station — labelled E STOP | Push-button mushroom E-Stop | right side push button and estop.jpg; HMI X0 |
| ES-02 | Fisnar F4303N ADVANCE controller face — EMERGENCY STOP | Push-button mushroom E-Stop (twist-reset style per photo) | component tray button.jpg; HMI X12 |

NOTICE ES-01: right two-hand station E STOP — HMI has shown Machine Emergency Pressed [X0]. ES-02: Fisnar controller EMERGENCY STOP — HMI has shown Robot Emergency Pressed [X12].

## 5.2 Activation Sequence

START

Detect emergency condition

Press nearest E-Stop hard

Machine motion stops

Keep clear of process zone

Alert supervisor if needed

Do not reset until safe

END

## 5.3 Reset Sequence

START

Identify why E-Stop pressed

Clear hazard and personnel

Close side doors fully

Twist-reset E-Stop device

Clear HMI emergency alarm

Press RESET if required

Confirm ready status

Restart only if safe

END

## 5.4 After E-Stop

Investigate cause (operator, robot fault, door fault, air pressure, etc.).

Check HMI ALARM list and status indicators (EMERGENCY, ROBOT EMERGENCY).

Confirm front automatic door and side-door statuses before resuming AUTO.

If the dispenser tip was mid-cycle, follow parking/purge guidance before long idle.

Repeated unexplained E-Stops → take out of service and call skilled maintenance.

# 6. Operating Instructions

## 6.1 Pre-Start Checks

Guards and side doors closed and undamaged; Omron switches seated

Front automatic door free of obstruction

Main isolator ON; HMI powered; tower light functioning

Plant air present; FRL gauge near expected operating pressure

No active emergency on HMI; E-Stops released

Correct model selected (e.g. JPTP / MGTP / APTDPT)

Sealant cartridge installed and within process life; tip clean

Work area clear; authorised operator only

## 6.2 Startup

START

Turn POWER isolator ON

Confirm air supply ready

Release all E-Stops

HMI boots to GLUE DISPENSER

Select AUTO or MANUAL

Select model program

Clear any alarms

Verify doors closed

END

## 6.3 Automatic Production Cycle

START

Select model on HMI

Place component on tray

Hands on both green starts

Press two-hand together

Front door closes

Robot dispenses sealant

Cycle ends door opens

Unload finished part

END

WARNING Keep hands outside the door path and robot envelope during the cycle. Do not reach in if the door fails to close or the robot is still moving.

## 6.4 Model Selection

From HMI MODEL SELECTION: running model and robot program number are displayed. Observed model names include JPTP, MGTP, and APTDPT. Select the model matching the component before starting production.

Unauthorised model/program changes can apply sealant incorrectly and may create unexpected robot paths. Only authorised persons may change models or teach programs (OA-P7-011).

## 6.5 Manual Mode

MANUAL mode is available on the HMI for setup and recovery tasks. Manual operations must be performed only by trained personnel with an understanding of robot motion and door behaviour.

CAUTION Manual mode does not remove the need for LOTO when hands enter the cell for tool or tip work.

## 6.6 Parking and Purging

The machine provides parking and purging cups for the dispenser tip. HMI Settings observed: Idle Time To Go Parking = 3.0 Min; Parking & Purging Sequence = ON.

Purpose: if the tip is left idle, sealant can dry and block dispensing. Use automatic parking/purging and the robot DISPENSER PURGE control as trained.

NOTICE Confirm process idle limit with sealant SDS and OEM (OA-P7-006). Image placeholders: Images/dispenser parking.jpg; dispenser purging.jpg; Dispenser.jpg.

## 6.7 Shutdown

START

Complete or abort cycle safely

Park or purge dispenser tip

Clear HMI to idle state

Turn isolator OFF if end of shift

Isolate air if required by site

Clean work area

Record issues in log

END

## 6.8 Forbidden Actions

Bypass two-hand, door sensors, or E-Stops

Operate with side doors open in production

Disable Front Automatic Safety Door or Side Door Safety Sensor settings without formal change control

Teach robot programs without authorisation

Clear tip blockage with unguarded hands while power/air available

Work in the panel without electrical LOTO

# 7. Maintenance & Care

## 7.1 General Rules

All maintenance inside the process zone or panel requires LOTO (Section 4). E-Stop alone is not isolation.

Only skilled persons may open the control panel, modify pneumatics, or change electrical components.

## 7.2 Recommended Checks

Daily (operator / end of shift):

Visual check of doors, two-hand buttons, E-Stops, tip condition

Confirm no damaged cables or air leaks

Empty purge waste as required; keep parking/purge cups clean

Confirm HMI free of standing alarms before leaving

Weekly / periodic (skilled):

FRL drain and filter condition (SMC unit)

Omron side-door switch alignment and mounting

Front door free travel and pneumatic fittings

Isolator and panel door condition

Robot mechanical play / cable track condition (per Fisnar OEM guidance)

PLACEHOLDER [PLACEHOLDER — OEM PM schedule and Fisnar service intervals — attach when available]

## 7.3 Sealant Tip and Dispenser Care

Follow sealant manufacturer instructions for storage and pot life.

Use parking/purge to reduce tip clogging.

Replace tips/cartridges only after energy isolation if hands enter the motion zone.

Dispose of waste sealant per site chemical rules and SDS.

## 7.4 After Maintenance

Restore all guards and close doors

Remove locks only after tools and persons are clear

Function-test E-Stops, two-hand start, door behaviour, and air-pressure alarms before production release

Record work in Appendix A log

# 8. HMI, Alarms & Controls

## 8.1 HMI Overview

Mitsubishi Electric HMI application title: GLUE DISPENSER.

Main menu functions observed: AUTO, MANUAL, MODEL SELECTION, SETTINGS, ALARM, ALARM HISTORY, DATA LOG, INPUT STATUS, OUTPUT STATUS.

Process screen shows mode and door/air/robot status lamps, model, robot program number, OK/NG/TOTAL counts, cycle time, and dispenser pressure.

NOTICE Network labels on menu photo: PLC 192.168.100.010, HMI 192.168.100.011. Confirm before network changes (OA-P7-009).

## 8.2 Settings Screen (as photographed)

| Parameter | Value |
| --- | --- |
| Dispenser Inlet Pressure Min/Max | 2.0 / 3.0 Bar |
| Pressure Check Delay | 1.0 Sec |
| Dispenser Pressure Check | ON |
| Side Door Safety Sensor | ON |
| Front Automatic Safety Door | ON |
| Idle Time To Go Parking | 3.0 Min |
| Parking & Purging Sequence | ON |

WARNING Do not turn safety-related settings OFF (side door sensor, front automatic safety door) for production convenience. Any change requires documented authorisation and risk review.

## 8.3 Alarms Observed

| Message / Code | Notes |
| --- | --- |
| Air Pressure Low | HMI Alarms.jpg live list |
| Robot Emergency Pressed [X12] | HMI Alarms.jpg live list |
| Front Auto Door open Error [X6] | HMI Alarms.jpg live list |
| Machine Emergency Pressed [X0] | HMI Process.jpg status banner |

PLACEHOLDER [PLACEHOLDER — Full HMI/PLC alarm catalogue — OA-P7-008]

## 8.4 Alarm Response

START

Stop and keep clear

Read HMI live alarm

Identify cause safely

Apply LOTO if entry needed

Correct root cause

Reset E-Stop if used

Clear alarm on HMI

Test then resume

END

## 8.5 Operator Hardware Controls

Cycle start requires simultaneous two-hand green buttons per intake description and labelled stations

Left station: RESET (yellow), TWO HAND CYCLE START (green)

Right station: TWO HAND CYCLE START (green), E STOP (red)

Robot face: DISPENSER PURGE, PROG NO., RUN/TEACH, START, EMERGENCY STOP

Panel: POWER ON/OFF isolator; SMC FRL

# 9. Safety Features, Schematics & Compliance

## 9.1 Safety Features Summary

| ID | Device | Detail (summary) |
| --- | --- | --- |
| SD-01 | Two-hand cycle start | Paired green TWO HAND CYCLE START buttons left and right of load zone (photos). Intended to keep hands clear during cycl… |
| SD-02 | Machine emergency stop (right station) | Red mushroom E STOP on right two-hand station (right side push button and estop.jpg). HMI references Machine Emergency P… |
| SD-03 | Robot emergency stop (Fisnar controller) | Red mushroom EMERGENCY STOP on Fisnar F4303N ADVANCE face. HMI references Robot Emergency Pressed [X12]. |
| SD-04 | Front automatic safety door | Vertical/sliding automatic front door (intake: slides from top). HMI Settings: Front Automatic Safety Door ON. Alarm: Fr… |
| SD-05 | Side door non-contact interlocks (L/R) | Omron GLS-M1 non-contact safety switch photographed on side door structure. HMI: Side Door Safety Sensor ON; status LH/R… |
| SD-06 | Power isolation switch | Rotary red/yellow POWER ON/OFF isolator (Lauritz Knudsen OFF-salzer). |
| SD-07 | Pneumatic FRL + pressure monitoring | SMC FRL with gauge; HMI AIR PRESSURE status and Air Pressure Low alarm; dispenser pressure check setting ON. |
| SD-08 | Perimeter guarding / side doors / polycarbonate panels | Aluminium extrusion frame with side doors and process enclosure (full view photos). Guard integrity must be maintained. |
| SD-09 | Tower light (status beacon) | Red/amber/green stack light on machine top (Full view side.jpg). Exact colour meanings **[PLACEHOLDER — confirm site map… |
| SD-10 | Moving-parts caution label (robot area) | CAUTION keep hands clear of moving parts label on robot structure (Dispenser.jpg / Component on tray.jpg). |

OPEN ACTION OA-P7-005: Validate front automatic door performance and residual risk of no light curtain at the load face. PLr / category of safety functions are not claimed without a Machine Safety Assessment (OA-P7-001).

## 9.2 Electrical Schematic (placeholder for customer insert)

PLACEHOLDER [PLACEHOLDER — Electrical schematic drawing to be inserted by OEM/customer — OA-P7-003] When available, insert: single-line supply, main isolator, PLC I/O power, E-Stop circuit overview, panel layout. Photo reference for as-built panel interior: Images/PLC panel open.jpg (not a substitute for a controlled schematic).

## 9.3 Pneumatic Schematic (placeholder for customer insert)

PLACEHOLDER [PLACEHOLDER — Pneumatic schematic drawing to be inserted by OEM/customer — OA-P7-004] When available, insert: supply, FRL, isolation valve, front door actuators, dispenser air path. Photo references: Images/Pneumatic FRL.jpg; Front door pneumatic view.jpg.

## 9.4 Applicable Standards (reference)

The following standards and regulations are referenced as design/instruction principles for this manual. Clause numbers are not fabricated.

ISO 12100:2010

ISO 20607:2019

IEC 82079-1:2019

IS 4571:2008

ISO 13849-1 (principles — no PLr claims without assessment)

ISO 13850 (E-Stop principles)

ANSI B11.19 (safeguarding principles)

Factories Act 1948 (India)

Explicitly not referenced in this manual: IEC 61508; IEC 61800-5-2.

## 9.5 Liability Disclaimer

This manual is prepared from available photographs and intake information for the Robotic Sealant Dispenser Machine. It does not replace a formal Machine Safety Assessment, OEM electrical/pneumatic design packages, or site-specific safe work procedures.

Rani Enterprises (OEM), the end user, and operators remain responsible for correct installation, training, maintenance, and safe use. The Certified Safety Professional preparing this draft is not liable for use of incomplete data, defeated safeguards, unauthorised modifications, or failure to close Open Actions before production release.

Where information was unavailable, PLACEHOLDER text and Open Actions are used. Closing those actions is a condition of treating this document as complete.

## 9.6 Document Sign-Off

Prepared by: Certified Safety Professional

Role: Independent Safety Consultant (instruction manual authoring)

Reviewed by: ___________________________  Date: ___________

Approved by (OEM): _____________________  Date: ___________

Approved by (End User): _________________  Date: ___________

Sign-off appears once in this manual (this subsection only).

# Appendix A — Maintenance Log Template

Use this log for routine and corrective maintenance. Attach additional pages as needed.

| Date | Task performed | Parts replaced | Technician | LOTO (Y/N) | Next due | Remarks |
| --- | --- | --- | --- | --- | --- | --- |

# Appendix B — LOTO Log Template

Use this log whenever Lockout/Tagout is applied.

| Date | Time on | Time off | Zone | LP-01 | LP-02 | Worker | Lock ID | Reason | Zero energy | Supervisor |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |