[LOGO PLACEHOLDER]

OPERATING & SAFETY MANUAL

Low Energy Test Bench (Clutch Endurance Test)

Machine Reference: [PLACEHOLDER — internal asset tag, pending customer confirmation]

Manufacturer: [PLACEHOLDER — manufacturer not identified in source Machine Safety Assessment Report; machine described as a retrofit unit relocated from another Valeo facility]

End User: Valeo Friction Materials India Private Limited

Installation: R&D Cell, No. 16A, Sengundram Industrial Area, Melrosapuram (via Singaperumal Koil), Chengalpattu / Kancheepuram District, Chennai, Tamil Nadu — 603204

| Field | Value |
| --- | --- |
| Document Title | Operating & Safety Manual — Low Energy Test Bench |
| Document No. | [PLACEHOLDER — to be assigned] |
| Revision | V1.0 |
| Date | [PLACEHOLDER — issue month/year] |
| Prepared By | Certified Safety Professional |
| Status | Draft |

CONFIDENTIAL

# REVISION HISTORY

| Revision | Date | Description | Reviewed By |
| --- | --- | --- | --- |
| V1.0 | [PLACEHOLDER — issue date] | Initial issue | [PLACEHOLDER — Valeo Friction Materials India] |

# TABLE OF CONTENTS

1. Front Matter	5

2. Machine Overview	6

3. Hazard Identification	8

4. Lockout/Tagout (LOTO) Procedures	9

5. Emergency Stop	11

6. Operating Instructions	12

7. Maintenance & Care	15

12. Safety Features & Interlocks	16

13. Compliance & Certifications	18

Appendix A — Maintenance Log Template	20

Appendix B — LOTO Log Template	21

Right-click → Update Field → Update entire table.

Note on section numbering: This is a Tier C manual. Sections 8 (Electrical Schematic), 9 (Pneumatic Schematic), 10 (Hydraulic Schematic), and 11 (HMI Alarms & Fault Codes) are omitted per Tier C scope — no schematic drawings or HMI alarm/fault code data were provided in the source Machine Safety Assessment Report. Section numbers 2–7 and 12–13 are retained as-is for consistency with the master template; do not renumber.

# 1. FRONT MATTER

## 1.1 Scope of this Manual

This manual covers the safe operation, lockout/tagout, emergency stop procedures, routine operation, and maintenance of the Low Energy Test Bench (clutch endurance test) installed at the R&D Cell, Valeo Friction Materials India Private Limited, Chennai. It has been prepared based on the Machine Safety Assessment Report (Report No. 1, Revision 0.3) covering mechanical and electrical hazards, and on data supplied by the end user.

## 1.2 Intended Audience

This manual is intended for machine operators, maintenance personnel, calibration technicians, and housekeeping staff who interact with this machine, as defined in Section 2.5 (Tasks & Affected Persons).

## 1.3 How to Use this Manual

Read this manual in full before operating, maintaining, or servicing the machine. Sections 4 and 5 (LOTO and Emergency Stop) must be understood and followed exactly before any maintenance or fault-finding activity. Where information could not be verified against source documentation, it is marked as an Open Action or Placeholder — refer to the standalone Open Actions Register for these items.

## 1.4 Definitions & Abbreviations

| Term | Definition |
| --- | --- |
| LOTO | Lockout/Tagout |
| E-Stop | Emergency Stop |
| PPE | Personal Protective Equipment |
| DaN | Decanewton (unit of force, clutch displacement) |
| RPM | Revolutions Per Minute |
| PLr | Performance Level required (per ISO 13849-1) |
| HMI | Human-Machine Interface |

## 1.5 Signal Words

| Signal Word | Meaning |
| --- | --- |
| WARNING | Indicates a hazardous situation which, if not avoided, could result in serious injury |
| CAUTION | Indicates a hazardous situation which, if not avoided, may result in minor or moderate injury |
| NOTICE | Indicates information considered important but not hazard-related |

## 1.6 Prepared By

Certified Safety Professional

# 2. MACHINE OVERVIEW

## 2.1 Machine Identification

| Field | Value |
| --- | --- |
| Machine Name | Low Energy Test Bench (Clutch Endurance Test) |
| Machine Type | Testing machine |
| Manufacturer | [PLACEHOLDER — not identified; retrofit unit, no nameplate present] |
| Manufacturing Date | 8 August 2025 |
| Re-manufacturing Date | 8 August 2025 |
| Model No. | Not Available |
| Serial No. | Not Available |
| Asset No. | Not Available |
| Installation Site | R&D Cell, Valeo Friction Materials India Pvt. Ltd., Sengundram Industrial Area, Melrosapuram, Chennai, Tamil Nadu 603204 |

⚠️ OPEN ACTION OA-P8-001 Manufacturer name, model number, and serial number are not available. Machine is a retrofit unit relocated from another Valeo facility with no nameplate. Customer to confirm or source manufacturer records.

## 2.2 Intended Use

Automated endurance testing of automotive clutch assemblies (friction disc, cover plate, flywheel) by simulating repeated engage/disengage cycles to evaluate torque, wear, displacement, and thermal performance. Operated by authorised personnel only.

## 2.3 Reasonably Foreseeable Misuse

•  Operating without safety covers in place

•  Inserting hands inside the bench during a test run

•  Unauthorised persons operating the machine

•  Incorrect air pressure setting (outside 4–6 bar)

•  Running a test without verifying clutch displacement (450–500 DaN)

•  Skipping PPE requirements

## 2.4 Machine Parameters

| Parameter | Value |
| --- | --- |
| Processed workpieces | Clutch variants — impregnated yarn made of glass fibre, infused with metal |
| Capacity | 3–4 tests per month, continuous intervals |
| Operating modes | Manual, Automatic |
| Electrical supply | 415 V AC, 3-phase |
| Pneumatic supply | 6 bar (operating range 4–6 bar) |
| Hydraulic supply | Not Available |
| Machine footprint | Approx. 3 m (L) × 2.5 m (W) |
| Operating clearance | Add 1 m on all sides |
| Ideal total space | 5 m (L) × 4.5 m (W) ≈ 22.5 m² |
| Ambient temperature | 20°C to 35°C |
| Humidity | 40–70% RH, non-condensing |
| Time limits | 200 days/year, up to 24 hours/day, 1 shift/day |

## 2.5 Tasks & Affected Persons

| Person | Skill Level | Description |
| --- | --- | --- |
| Operator 1 | Instructed | Trained on test bench with work instructions; bi-yearly refresher |
| Operator 2 | Instructed | Expert-trained for critical HMI parameter modification |
| Maintenance Person | Skilled | Trained in electrical maintenance; bi-yearly refresher |
| Housekeeping Staff | Unskilled | Floating manpower, non-technical |
| Third-Party Vendor (Calibration) | Skilled | NABL-accredited, calibrates torque/pressure/temperature instruments |
| Visitors | Unskilled | Not trained to access machine parts or doors |

Task groups performed at the machine: Air filter cleaning; Clutch assembly load/unload; Test program monitoring; Calibration of temperature/field sensors; Maintenance/abnormality reaction; Housekeeping; Test program creation & launch (startup test); Presence at the machine (non-task personnel in the hazard area).

## 2.6 Manufacturing Process Description

The operator assembles the clutch (flywheel, friction disc, cover plate) onto the test bed, sets displacement (450–500 DaN), and starts the machine via the MORPHEE software interface. The bench runs automated engage/disengage cycles, recording torque, temperature, and RPM. At 6,000 cycles, the friction disc is inspected for wear and reassembled to continue. Post-test, the clutch is disassembled, parts are measured, and scrapped components are handed to EHS.

[IMAGE PLACEHOLDER — Overall machine view, front. Source: customer site photo, pending upload.]

# 3. HAZARD IDENTIFICATION

The machine presents mechanical, electrical, thermal, noise, and dust-related hazards, identified and evaluated in accordance with ISO 12100.

Mechanical hazards arise primarily from rotating parts (spindle, flywheel, friction disc) and the pneumatic cylinder used during clutch loading. The main drive does not stop instantaneously on command — residual rotation continues for several seconds after the stop signal, creating a crushing hazard during the clutch assembly load/unload process and during maintenance access to the drive shaft and coupling, where drawing-in or entanglement is possible.

Electrical hazards are concentrated at the control panel, where basic protection exists for the live bus bar terminals, but full IP2X-rated enclosure protection is not currently confirmed. This was identified as a high-severity risk in the source assessment, primarily affecting maintenance personnel during fault-finding activities. Power fluctuation controls are also noted as absent in the current system.

Thermal hazards result from internal friction-induced heat, which can reach approximately 500°C within the test chamber during operation. Guard-locking interlocks prevent door access until the chamber cools to a tolerable range; override of this function is restricted to authorised personnel using manual mode.

Noise hazards occur at high test speeds (approximately 3,000 RPM) and are rated as low residual risk given the desk-partition design that separates the operator control position from the machine perimeter.

Material/substance hazards relate to fine dust generated during testing. A dust collection/exhaust system is present, though the source assessment notes the fixed pipeline may require modification and regular inspection.

Across all identified hazard categories, risk reduction is achieved primarily through fixed and movable interlocked guards, guard-locking devices, hold-to-run controls, emergency stop provisions, lockout/tagout procedures, and mandatory PPE. The source assessment notes that the primary fixed guard (G001) uses a retaining lever that could potentially be removed, which is carried forward here as an identified residual risk rather than treated as fully mitigated.

Full hazard identification and risk assessment is documented in the Machine Safety Assessment Report [Report No. 1, Revision 0.3, dated 3 April 2026].

# 4. LOCKOUT/TAGOUT (LOTO) PROCEDURES

## 4.1 Purpose

This section defines the energy isolation points and procedures required before any maintenance, cleaning, or fault-finding task on the machine, in accordance with ISO 14118 and IEC 60204-1.

## 4.2 Energy Isolation Points

| Point ID | Energy Type | Image | Isolation Method | Location |
| --- | --- | --- | --- | --- |
| LP-01 | Electrical | [ISO E001 icon] | On-board LOTO / Emergency off (per IEC 60204-1) | Control panel |
| LP-02 | Pneumatic | [ISO pneumatic icon] | Pneumatic shut-off and exhaust valve (per ISO 4414) | [PLACEHOLDER — exact panel location pending site confirmation] |

⚠️ OPEN ACTION OA-P8-002 Exact physical location of pneumatic shut-off/exhaust valve not specified in source data. Customer to confirm for LOTO point diagram.

## 4.3 General LOTO Rules

•  Only trained, authorised personnel may perform lockout/tagout.

•  All energy sources (electrical, pneumatic) must be isolated before entering the hazard zone for maintenance.

•  Tamper-resistant fasteners are fitted at guard access points — special tools are required and must not be left accessible to unauthorised personnel.

•  A written Lockout/Tagout Policy, paired with the machine-specific procedure, must be followed (per ISO 14118 supplemental measures).

## 4.4 Tag-Out Procedure (Flowchart)

1. Start

2. Notify affected personnel of planned isolation

3. Complete test cycle or pause at safe point

4. Switch off main electrical supply at on-board LOTO device

5. Close pneumatic shut-off and exhaust valve

6. Confirm zero energy state (check indicator/gauge)

7. Apply personal lock and tag

8. End — machine safe for maintenance

## 4.5 Tag-In Procedure (Flowchart)

1. Start

2. Complete maintenance task

3. Remove tools and confirm work area clear

4. Remove personal lock and tag

5. Restore pneumatic supply

6. Restore electrical supply at on-board device

7. Confirm guards closed and interlocks engaged

8. End — machine ready for restart

# 5. EMERGENCY STOP

## 5.1 Purpose

This section describes the emergency stop function in accordance with ISO 13850.

## 5.2 E-Stop Locations

E-Stop push buttons are fitted at the operator control desk and at the machine perimeter (identified as devices T004 and ESTOP in the source assessment). An additional on-board emergency-off device (T008) is provided for the electrical control panel.

## 5.3 E-Stop Function

Activating any E-Stop push button immediately removes power to hazardous motion. The main drive requires several seconds to fully stop due to residual rotation — operators must not assume immediate stoppage.

## 5.4 E-Stop Activation Procedure (Flowchart)

1. Start

2. Identify hazardous situation

3. Press nearest E-Stop push button

4. Confirm machine motion has stopped

5. Notify supervisor/maintenance

6. End — machine in stopped state

## 5.5 E-Stop Reset Procedure (Flowchart)

1. Start

2. Confirm hazard cleared and area safe

3. Confirm all guards closed and interlocks engaged

4. Rotate/release E-Stop button

5. Press reset button (blue, indicator-lit)

6. End — machine ready for restart

# 6. OPERATING INSTRUCTIONS

## 6.1 Operating Modes

Manual and Automatic, selected via the MORPHEE software interface at the operator desk.

## 6.2 Required PPE

General PPE as specified for noise and dust exposure near the machine area — refer to site PPE policy.

[PLACEHOLDER — specific PPE item list (e.g., hearing protection class, glove type) not itemised in source data; customer to confirm.]

## 6.3 Pre-Shift Checks

•  Visually inspect guards (G001–G003) for damage or missing fasteners

•  Confirm E-Stop buttons are unobstructed and functional

•  Confirm dust collection/exhaust system is operating

•  Confirm air pressure gauge reads within 4–6 bar

## 6.4 Pre-Start Checks (Flowchart)

1. Start

2. Inspect guards and interlocks

3. Confirm E-Stop accessible and reset

4. Confirm air pressure 4–6 bar

5. Confirm work area clear of unauthorised persons

6. Confirm PPE worn

7. End — ready for startup

## 6.5 Startup Sequence (Flowchart)

1. Start

2. Power on control panel

3. Launch MORPHEE software

4. Select operating mode (Manual/Automatic)

5. Load clutch assembly per Section 2.6

6. Set displacement parameter (450–500 DaN)

7. Close and confirm all guard doors

8. End — begin test cycle

## 6.6 Test Program Creation & Startup Test

Setting, teaching/programming, and process changeover occur either every 24 hours (for frequent tests) or every 15 days (for less frequent tests), performed by Operator 2. Total setup time exceeds one hour. During this activity, Operator 2 may need to open a door in manual mode to activate the stroke lever holding test parts — the source assessment identifies this as a specific crush-injury risk point requiring strict adherence to hold-to-run control procedures.

## 6.7 Normal Operating Cycle (Flowchart)

1. Start

2. Automated engage/disengage cycles begin

3. System records torque, temperature, RPM

4. At 6,000 cycles: pause for friction disc inspection

5. Reassemble and resume if disc acceptable

6. Continue cycles to test completion

7. Disassemble clutch and measure parts

8. Hand scrapped components to EHS

9. End — test complete

## 6.8 Shutdown Sequence (Flowchart)

1. Start

2. Complete or pause current test cycle

3. Stop via HMI/software control

4. Confirm rotating parts fully stopped

5. Isolate air supply if end of shift

6. End — machine in safe standby state

## 6.9 Emergency Shutdown (Flowchart)

1. Start

2. Press E-Stop immediately

3. Confirm all motion stopped

4. Do not attempt to open guards until confirmed safe

5. Notify supervisor

6. End — await maintenance/reset per Section 5.5

# 7. MAINTENANCE & CARE

## 7.1 Maintenance Philosophy

Maintenance activities are performed by trained, skilled maintenance personnel. Major troubleshooting is typically performed by a third party at approximately six-month intervals, per the source assessment.

## 7.2 Maintenance Schedule

| Frequency | Task |
| --- | --- |
| Weekly | Visual inspection of guards, interlocks, and E-Stop devices |
| Monthly | Inspect and clean/replace air filter (per task group in source data) |
| Six-Monthly | Third-party fault-finding/troubleshooting as required; calibration of torque, pressure, and temperature instruments (NABL-accredited vendor) |

[PLACEHOLDER — Detailed OEM-recommended maintenance intervals not available; schedule above reflects only the activities described in the source Machine Safety Assessment Report. Customer to confirm against any available OEM documentation.]

## 7.3 Air Filter Replacement Procedure

The trained operator stops the machine, isolates it from power/air supply per Section 4, and opens the filter housing access panel. The used filter is removed and disposed of per waste handling guidelines; a new filter is inserted in the correct orientation. The housing is secured, and power/air is restored. Final checks confirm proper sealing and airflow before resuming operation.

## 7.4 Housekeeping

Housekeeping staff, under supervision of Operator 1, clean residual dust from the machine perimeter and surface areas. This task carries residual thermal and mechanical risk (proximity to rotating parts and hot surfaces) and must only be performed with guards in place and PPE worn.

# 12. SAFETY FEATURES & INTERLOCKS

## 12.1 Overview

Risk reduction on this machine follows the three-step hierarchy of ISO 12100: inherent safe design, engineering controls (guards and devices), and administrative measures.

## 12.2 Safety Function Hierarchy (Flowchart)

1. Start — hazard identified

2. Input: guard/interlock sensor state

3. Logic: safety-related control system (guard locking, E-Stop circuit)

4. Output: stop command to drive/actuators

5. End — hazardous motion prevented

Key points:

•  Three movable/sliding guard doors are fitted with mechanical stoppers and interlocks (guard locking function), designed per EN ISO 13855.

•  The fixed guard over the drive area (G001) uses a removable lever — flagged in the source assessment as a residual risk if tampered with.

•  An acrylic fixed guard is specified for the live bus bar terminal area (dimensions not finalised in source data).

## 12.3 Interlocks & Guard Locking

Guard locking (per ISO 14119) prevents door opening until hazardous motion has stopped and, for the thermal chamber, until internal temperature has reduced to a tolerable range. Override to manual mode is restricted to trained personnel via password-protected access.

## 12.4 Energy Isolation Devices

Refer to Section 4.2 for the full list of LOTO points. Additional devices include hold-to-run pushbuttons (restricted to trained personnel, protected from accidental external access) and a reset button with indicator light.

## 12.5 Daily Safety Verification Checklist

•  Confirm all three guard doors close and latch fully

•  Confirm E-Stop buttons trigger immediate stop on test press

•  Confirm guard-locking interlock prevents door opening during a running cycle

•  Confirm dust collection exhaust is drawing air (visual/audible check)

•  Confirm tamper-resistant fasteners are intact on G001 and G004 guards

•  Confirm no visible damage to acrylic bus bar guard

•  Confirm reset button indicator light functions correctly

•  Confirm pneumatic pressure gauge reads within 4–6 bar range

# 13. COMPLIANCE & CERTIFICATIONS

## 13.1 Applicable Standards

This manual has been prepared with reference to:

•  ISO 12100:2010 — Safety of machinery, General principles for design

•  ISO 20607:2019 — Safety of machinery, Instruction handbook

•  IEC 82079-1:2019 — Preparation of information for use

•  IS 4571:2008 — Indian machinery safety standard

•  ISO 13849-1 — Safety-related parts of control systems

•  ISO 13850 — Emergency stop function

•  ISO 14119 — Interlocking devices associated with guards

•  ISO 14120 — Guards, general requirements

•  ISO 13857 — Safety distances

•  ISO 4414 — Pneumatic fluid power systems

•  Factories Act, 1948 (India)

## 13.2 Basis of Assessment

This manual is based on findings of an Independent Safety Consultant's Machine Safety Assessment Report (Report No. 1, Revision 0.3, dated 3 April 2026), covering mechanical and electrical hazards only. Other hazard types (noise, dust/substance) were noted where observed but were not the primary subject of the referenced assessment.

## 13.3 Prepared By

Certified Safety Professional — Manual Author

Designation: Certified Safety Professional

Signature: _________________________

Date: _________________________

## 13.4 Liability Disclaimer

LIABILITY DISCLAIMER

This Operating & Safety Manual has been prepared by a Certified Safety Professional in accordance with ISO 12100:2010, ISO 20607:2019, IEC 82079-1:2019, and IS 4571:2008, based on the Machine Safety Assessment and technical documentation provided.

Where original technical data was unavailable, professional judgment has been applied. All such areas are documented as Open Actions.

The customer is responsible for final review and verification before the machine is placed into service. Items marked as Open Actions must be verified and signed off by the customer's technical or safety representative.

Neither the consulting practice nor the author assumes liability for injuries or damages resulting from: non-compliance with this manual; failure to maintain per the schedule; or operation by untrained personnel.

## 13.5 Sign-Off Blocks

Manufacturer Authorization

Name: _________________________     Title: _________________________     Date/Signature: _________________________

End User Authorization

Name: _________________________     Title: _________________________     Date/Signature: _________________________

Certified Safety Professional — Manual Author

Title: Certified Safety Professional     Date/Signature: _________________________

# APPENDIX A — MAINTENANCE LOG TEMPLATE

| Date | Task Performed | Performed By | Findings | Next Due |
| --- | --- | --- | --- | --- |

# APPENDIX B — LOTO LOG TEMPLATE

| Date | Isolation Point(s) | Locked By | Time Locked | Time Restored | Verified By |
| --- | --- | --- | --- | --- | --- |