OPERATING & SAFETY MANUAL

BG Snap Assembly Machine

Indo-MIM Limited  |  Tata Electronics, Hosur

| Document Title | Operating & Safety Manual — BG Snap Assembly Machine |
| --- | --- |
| Document Number | IndoMIM-BGSnap-OSM-001 |
| Revision | V2.0 |
| Date | May 2026 |
| Prepared By | Certified Safety Professional |
| Status | DRAFT — Pending site data (schematics, IPC alarms, site photos) |

CONFIDENTIAL

TABLE OF CONTENTS

Right-click this area in Word and select Update Field (or press Ctrl+A then F9) to generate the Table of Contents.

No table of contents entries found.

1.  FRONT MATTER

1.1  Document Scope

This manual covers the BG Snap Assembly Machine (Project Reference V67) installed at Tata Electronics, MG Road, Hosur, Tamil Nadu 631604, designed and supplied by Indo-MIM Limited. It addresses machine identification, hazard identification, LOTO procedures, emergency stop, operating instructions, maintenance, schematics, IPC alarms, safety features, and compliance. It does not cover design, manufacture, installation, or commissioning.

1.2  Intended Audience

| Audience | Sections to Read |
| --- | --- |
| All personnel near machine | Sections 1, 2, 3, Appendix A |
| Machine operators | Sections 1–6, Appendix A |
| Setup technicians | All sections |
| Maintenance technicians | All sections — emphasis on Sections 4, 7, 12 |
| EHS / Safety personnel | Sections 3, 4, 5, 12, 13, Appendix A |
| Management / Auditors | Sections 1, 3, 13, Appendix A |

1.3  General Safety Rules

Untrained personnel must not operate this machine. No exceptions.

No maintenance inside the guarding envelope without completing full LOTO (Section 4).

No safety device, interlock, guard, or label may be removed or bypassed without written authorization from Indo-MIM Limited and Tata Electronics EHS.

All incidents, near misses, and faults must be reported to the shift supervisor and recorded in the maintenance logbook immediately.

The machine must be operated only within the intended use defined in Section 2.3.

PPE must be worn at all times during maintenance as specified in Appendix A.

This manual must remain at the machine and be available to all operators and technicians during their shift.

1.4  Signal Word Hierarchy

| Signal Word | Meaning |
| --- | --- |
| WARNING | Imminently hazardous — will result in death or serious injury if not avoided. |
| WARNING | Potentially hazardous — could result in death or serious injury. |
| CAUTION | Potentially hazardous — may result in minor or moderate injury. |
| NOTICE | May result in property or equipment damage. No personal injury risk. |

1.5  Revision History

| Rev | Date | Description | Prepared By |
| --- | --- | --- | --- |
| V1.0 | April 2026 | Initial issue — DFM V1.4 & RA V1.1 basis. Placeholders for site items. All OAs open. | Certified Safety Professional |
| V2.0 | May 2026 | Tier B reissue — condensed to 40 pages. Placeholders retained for schematics, IPC alarms, site photos. OAs remain open pending site visit. | Certified Safety Professional |
| V3.0 | May 2026 | Site visit updates: single phase supply correction, IPC nomenclature (replaces HMI), E-stop count updated to 11, LOTO corrections, compulsory energy isolation note added, author field corrected to role only. | Indo-MIM / Tata Electronics |

1.6  ISO 7010 Safety Signs Used in This Manual

| Code | Description | Used In |
| --- | --- | --- |
| W001 | General warning | Secs 1, 3, 6 |
| W003 | Electricity hazard | Secs 3, 4, 8 |
| W017 | Crushing hazard | Secs 3, 5, 6 |
| W019 | Automatic start-up | Secs 3, 6 |
| W028 | Entanglement hazard | Secs 3, 6 |
| P002 | Do not touch | Secs 3, 4 |
| P010 | Do not switch on | Secs 4, 5 |
| M008 | Wear protective gloves | Secs 4, 7, App A |
| M009 | Wear safety footwear | Secs 4, 7, App A |

Refer to Section 13 for the full Liability Disclaimer.

This manual was prepared by Certified Safety Professional, in accordance with ISO 12100:2010, ISO 20607:2019, IEC 82079-1:2019, and IS 4571:2008. Where data was unavailable, gaps are formally recorded as Open Actions — none have been ignored or assumed away. Indo-MIM Limited and Tata Electronics are jointly responsible for final review and verification. All Open Actions must be closed before the machine enters production service.

2.  MACHINE OVERVIEW

2.1  Machine Identification

| Field | Details |
| --- | --- |
| Machine Name | BG Snap Assembly Machine |
| Project Reference | V67 — BG Snap Assembly Station |
| Machine Type | Automated Snap & Screw Assembly Line |
| Manufacturer | Indo-MIM Limited |
| End User | Tata Electronics |
| Installation Location | Tata Electronics, MG Road, Hosur, Tamil Nadu — 631604 |
| DFM Reference | BG Snap Assembly DFM V1.4, dated 18-March-2026 |
| Risk Assessment Ref. | Machine Safety Assessment Report V1.1, dated 23-March-2026, by Independent Safety Consultant |
| Machine Weight | OPEN ACTION OA-P5-004 — Pending Indo-MIM confirmation |

2.2  Technical Specifications

| Parameter | Specification |
| --- | --- |
| Main Supply | 230V AC AC, 3-phase, 50 Hz |
| Station Supply | 220V AC per station |
| Total Connected Power | 54 kW (full line) |
| Maximum Current | [PLACEHOLDER — confirm current rating] |
| Compressed Air | 0.5 – 0.7 MPa (clean, dry, oil-free) |
| Line Dimensions (L×W×H) | 8,400 mm × 2,100 mm × 2,200 mm |
| Operator Headcount | 1 (tray loading and unloading only) |
| Cycle Time | 5 seconds per piece |
| UPH | 629 |
| OEE | 87.42% (Avail: 93.04% / Perf: 97.03% / 1PY: 96.84%) |
| Design Life | 10 years (with scheduled maintenance) |
| Operating Environment | Indoor, controlled temperature & humidity, clean-room-like |

2.3  Intended Use

This machine assembles three snap components (part numbers 613-31681, 613-38625, 613-38626) onto each HSG unit, securing each snap with two screws — six screws per unit, every five seconds. Any use outside this scope is foreseeable misuse and is strictly prohibited.

2.4  Station Layout & Line Configuration

| Station | Name | Function | Key Equipment |
| --- | --- | --- | --- |
| S1 | Loading Machine | Picks HSG from input tray, scans barcode, transfers into Maglev fixture | KUKA KR13 R850, barcode scanner, Cognex |
| S2 | Snap Assembly & Screwing | Assembles 1st snap; fastens 2 screws; CCD vision recheck | HIWIN axes, Atlas QMC-21, Deprag feeder, YIG reel feeder |
| S3 | Snap Assembly & Screwing | Assembles 2nd snap; fastens 2 screws; CCD vision recheck | Same as S2 |
| S4 | Flipping Station | Flips HSG 180° to expose reverse face for 3rd snap | Rotating cylinder, HIWIN Z-axis, pneumatic unlock |
| S5 | Snap Assembly & Screwing | Assembles 3rd snap on reverse face; fastens 2 screws; CCD vision recheck | Same as S2/S3 |
| S6 | Unloading Machine | Picks completed HSG; routes OK to output tray; NG to buffer/NG tray | KUKA KR13 R850, buffer fixture, NG tray |

Fixture transport: Product fixtures circulate on the Beckhoff Maglev conveyor — upper layer S1→S6, lower layer return. RFID readers (Pepperl+Fuchs IQT1-18GM-IO-V1) at each station link the HSG barcode to fixture ID for full traceability.

2.5  Key Components

| Component | Brand & Model | Station(s) |
| --- | --- | --- |
| 4-axis SCARA Robot | KUKA KR13 R850 Z200-2 CS | S1, S6 |
| Smart Screwdriver | Atlas Copco QMC-21-10-HM4 | S2, S3, S5 |
| Screwdriver Controller | Atlas Copco MTF6000 | S2, S3, S5 |
| Screw Feeder | Deprag Mini Screw Feeder | S2, S3, S5 |
| Tape Reel Feeder | YIG Tape Reel Feeder | S2, S3, S5 |
| Maglev Conveyor | Beckhoff / Baeyal-Golytec | Full line |
| Servo Axes | HIWIN KK / Toyo GTH series | All stations |
| Safety PLC | FLX3 series | Full line |
| Safety Door Latch | Keyence GS-51P5 (PLe / Cat 4 / SIL3) | S1–S6 |
| Safety Light Curtain | Keyence GL-VM39FP (PLe / Cat 4 / SIL3) | S1, S6 |
| IPC & Motion Card | Advantech IPC 7132 + PCIE-1203 | All stations |
| RFID Reader | Pepperl+Fuchs IQT1-18GM-IO-V1 | All stations |
| Barcode Scanner | Cognex | S1 |
| 3D Snapshot Sensor | LMI Gocator 3506 | S2, S3, S5 |

2.6  Maintenance Access Zones

| Zone | Clearance | Purpose |
| --- | --- | --- |
| Front — S1 and S6 ends | 1,000 mm | HSG tray loading and unloading |
| Rear of line | 1,000 mm | Snap/screw loading, general maintenance |
| Between S2/S3 and S5 | 500 mm minimum | Fixture access and channel maintenance |
| Top of machine | 1,000 mm above 2,200 mm | Overhead service and cable management |

PLACEHOLDER — Machine Layout Diagram (Figure 2-1) Full line top-view layout (S1–S6, Maglev conveyor path, access zones). Source: DFM V1.4. 300 DPI, max 160 mm wide.

3.  HAZARD IDENTIFICATION & RISK ASSESSMENT

3.1  Methodology

The hazard assessment was conducted at design stage by an Independent Safety Consultant using ISO 12100:2010 methodology, covering all lifecycle phases. Each hazard was scored on Severity (S), Exposure (E), and Avoidance (A) to produce a risk level. Risk reduction was applied until all residual risks reached Low. The formal Machine Safety Assessment Report (V1.1, 23-March-2026) is held by Indo-MIM Limited and Tata Electronics.

3.2  Hazard Summary

| Category | Hazards Identified | Initial Risk | Residual Risk |
| --- | --- | --- | --- |
| Mechanical | 10 | Medium | Low (all) |
| Electrical | 4 | High (3) / Medium (1) | Low (all) |
| Ergonomic | 2 | Low | Low (all) |
| Control System | 4 | High (3) / Low (1) | Low (all) |
| Chemical | 0 | Not applicable | Not applicable |
| Total | 20 | — | All Low |

3.3  Hazard Narratives

3.3.1  Mechanical Hazards

Shearing from multi-axis servo gantries during maintenance. Entanglement and pinch at rotating fixtures, winder assemblies, and KUKA robot EOAT. Impact and trapping from KUKA SCARA arms and shuttle transfers. Gravity-assisted crushing at vertical lifting actuators. Controls: fixed perimeter guarding, interlocked access doors, STO on all servo drives.

3.3.2  Electrical Hazards

230V, Single Phase, [PLACEHOLDER — confirm current rating] in electrical enclosures and servo drive cabinets. Hazard types: contact with live parts, shock from insulation failure, residual stored energy, and overheating. Controls: enclosed panel design, lockable main isolator LP-01, protective earthing, LOTO (Section 4).

3.3.3  Ergonomic Hazards

Manual tray loading and unloading at S1 and S6. Awkward posture and physical strain from tray weight and frequency. Assessed Low risk. Managed through operator training and correct work posture guidance (Section 6).

3.3.4  Control System Hazards

Unexpected start-up, hazardous motion with doors open, incorrect reset logic, and safety function bypass. All require PLd/PLc control functions per ISO 13849-1:2015, implemented through the Safety PLC (Section 12).

3.4  Risk Reduction Hierarchy (ISO 12100)

| Step | Measure Applied |
| --- | --- |
| 1 — Inherently Safe Design | Enclosed station housings, Maglev conveyor routing, and single-point operator interaction at defined loading/unloading zones minimize access to hazardous zones. |
| 2 — Safeguarding | Keyence GS-51P5 safety door latches (PLe/Cat4/SIL3), Keyence GL-VM39FP light curtains (PLe/Cat4/SIL3), Safety PLC, STO on all servo drives, dual contactors, robot safety per ISO 10218-1. |
| 3 — Information for Use | This manual, safety warning labels, operator training, PPE requirements, and maintenance procedures address residual risks not eliminated by Steps 1 and 2. |

3.5  Applicable Standards

| Standard | Title | Relevance |
| --- | --- | --- |
| ISO 12100:2010 | Safety of Machinery — Risk Assessment and Reduction | All sections |
| ISO 13849-1:2015 | Safety-Related Parts of Control Systems | Safety PLC, STO, interlocks, E-stop |
| ISO 13850:2015 | Emergency Stop Function | E-stop design — Section 5 |
| ISO 14119:2013 | Interlocking Devices for Guards | Safety door latches — Section 12 |
| ISO 14120:2015 | Guards — General Requirements | Fixed and interlocked guard design |
| ISO 14118:2017 | Prevention of Unexpected Start-Up | Restart prevention — Section 12 |
| ISO 13855:2024 | Positioning of Safeguards | Light curtain positioning |
| ISO 10218-1:2011 | Safety Requirements for Industrial Robots | KUKA robot integration |
| ISO 20607:2019 | Instruction Handbook | Manual structure and content |
| IEC 82079-1:2019 | Preparation of Instructions for Use | Signal word hierarchy |
| IEC 60204-1:2016 | Electrical Equipment of Machines | Panel design — Section 8 |
| IEC 61496-1:2020 | Electro-Sensitive Protective Equipment | Light curtain spec |
| IS 4571:2008 | Safety of Machinery (Indian Standard) | Indian regulatory compliance |
| IS 12100:2012 | Risk Assessment — Indian Adoption | Indian standards framework |
| Factories Act 1948 | Indian Industrial Labour Law | Statutory compliance — Section 21 |

3.6  Personnel Categories

| Category | Permitted Tasks | Training Required |
| --- | --- | --- |
| Operator | Tray loading/unloading, IPC monitoring, alarm acknowledgement, normal cycle start/stop | Machine-specific operator training per this manual |
| Setup Technician | Recipe selection, parameter adjustment via IPC, fixture changeover | Operator training + setup/changeover training |
| Maintenance Technician | Scheduled and corrective maintenance, component replacement, LOTO execution | Operator + maintenance training + LOTO certification |
| Safety / Authorized Person | Safety PLC parameters, safety function testing, RA review | All above + safety system authorization |
| Manufacturer's Service Engineer | Robot programming, servo parameters, safety PLC config | Manufacturer authorization only |

WARNING [ISO 7010 W001] Residual Risk: Safeguards reduce risk — they do not eliminate it. Residual risks remain manageable only if LOTO is followed, PPE is worn, and procedures are respected. Read Appendix A before your first shift.

4.  LOCKOUT / TAGOUT (LOTO) PROCEDURES

Complete LOTO before entering the machine guarding envelope for any maintenance, repair, cleaning, inspection, or fault-clearing activity. No exceptions for quick jobs or familiar tasks.

4.1  Energy Isolation Points

| LOTO ID | Energy Type | Description | Isolation Device | Location |
| --- | --- | --- | --- | --- |
| LP-01 | Electrical | 230V, Single Phase, [PLACEHOLDER — confirm current rating] main supply | Lockable rotary main isolator | Main electrical panel — right side of S1 |
| LP-02 | Pneumatic | 0.5–0.7 MPa compressed air | Solenoid valve + lockout hasp | Air inlet — rear of machine base, S1 end |
| LP-03 | Electrical (Servo) | STO — all HIWIN and Toyo axes | STO command via Safety PLC | Integrated in Safety PLC |
| LP-04 | Electrical (Robot) | KUKA robot controller power | Robot controller main switch + robot controller enable | KR CS Box-2 inside S1 and S6 enclosures |
| LP-05 | Electrical (Maglev) | Maglev linear motor supply | Maglev conveyor PLC isolator | Maglev control panel — centre of line |

OPEN ACTION [OA-P5-001] All LOTO points identified from DFM V1.4 and RA. Physical location, labelling, and hasp availability must be verified during site visit at Tata Electronics, Hosur before this section is finalized. Responsibility: Certified Safety Professional — site visit ¦ Target: Dry run ¦ Status: Open

4.2  LOTO Equipment Required

Personal padlock — one per technician, keyed differently from all others

Lockout hasp — multi-lock capable, for LP-01 and LP-02

Danger tag — pre-printed with technician name, date, time, reason

Calibrated voltage tester — for LP-01 isolation verification

Pneumatic pressure gauge — to verify LP-02 bleed-down to zero

LOTO log sheet — to record all isolation activities

4.3  De-energization Sequence (Tag-Out)

Pre-condition: Machine in stopped state, all cycles complete. Follow steps in exact order.

Step 1 — Notify: Inform all personnel in the vicinity. Confirm no person is inside the guarding envelope.

Step 2 — Stop the Machine: Press IPC STOP. Confirm all axes and robots have stopped and active alarms acknowledged.

Step 3 — Select Maintenance Mode: At IPC, select Maintenance Mode. Disables automatic cycle initiation from all stations. NOTE: Compulsory energy isolation (full LOTO per Section 4) is required before any personnel enter guarded zones, even in Maintenance Mode.

Step 4 — Isolate LP-01 (Main Electrical): Turn main rotary isolator to OFF. Apply personal padlock through hasp. Attach Danger tag.

Step 5 — Verify LP-01 Isolation: Using calibrated voltage tester, verify voltage absent at main panel incoming terminals and servo drive inputs. Do not proceed if voltage detected.

Step 6 — Isolate LP-02 (Compressed Air): Close LP-02 ball valve. Apply padlock and tag. Wait for downstream pressure to bleed. Confirm gauge reads zero before proceeding.

Step 7 — Verify LP-02 Isolation: Confirm pressure gauge reads 0 MPa. Attempt to actuate a pneumatic cylinder — no movement must occur.

Step 8 — Isolate LP-04 (KUKA Robots): At each robot controller (S1 and S6), turn main switch to OFF. Dock robot controller. Apply personal padlock to controller cabinet door hasp.

Step 9 — Isolate LP-05 (Maglev): Turn Maglev conveyor PLC isolator to OFF. Confirm movers are stationary and status indicator shows de-energized.

Step 10 — Confirm LP-03 STO Status: Verify Safety PLC confirms STO active on all servo drives. All servo drive enable indicator lights must be off.

Step 11 — Allow Stored Energy to Dissipate: Wait 60 seconds after LP-01 isolation. Servo drive DC bus capacitors hold charge sufficient to cause a serious shock — this wait is mandatory.

Step 12 — Record in LOTO Log: Record technician name, date, time, all points locked out, reason, and signature.

WARNING [ISO 7010 P010] Your padlock. Your lock. Your life. No one else's padlock protects you. If you are inside the machine, your lock must be on every isolation point — regardless of who else has locked out.

4.4  Re-energization Sequence (Tag-In)

Pre-condition: All maintenance complete. All tools and materials removed. All guards and covers reinstalled and fastened.

Step 1 — Inspect and Clear: Walk full perimeter. Verify no tools or parts inside machine. Confirm all guards in place, access doors closed, no person inside.

Step 2 — Notify All Personnel: Inform all personnel machine is being re-energized. Ensure all are clear.

Step 3 — Restore LP-05 (Maglev): Turn Maglev PLC isolator to ON. Remove padlock and Danger tag from LP-05.

Step 4 — Restore LP-04 (KUKA Robots): Turn controller main switch to ON. Remove padlock. Allow boot sequence to complete.

Step 5 — Restore LP-02 (Compressed Air): Remove padlock and tag. Slowly open ball valve. Verify pressure builds to 0.5–0.7 MPa.

Step 6 — Restore LP-01 (Main Electrical): Remove padlock and tag. Turn main rotary isolator to ON. Verify panel indicator lights confirm power restored.

Step 7 — Confirm Safety System Status: Confirm Safety PLC reports all functions healthy. Test each access door interlock — machine must not allow cycle start with any door open.

Step 8 — Reset and Return to Normal Mode: Exit Maintenance Mode, select Normal Operating Mode. Perform a dry run cycle without product.

Step 9 — Record in LOTO Log: Record re-energization: technician name, time, confirmation all points restored, signature.

4.5  Partial Isolation

Partial isolation is only permitted with written authorization from the Indo-MIM maintenance supervisor and Tata Electronics safety representative, with a station-specific LOTO procedure documented and approved, a LOTO-certified technician performing the work, and a second authorized person present as safety observer at all times.

WARNING [ISO 7010 W001] Partial isolation increases residual risk significantly. Never performed by a single person working alone. Never used as a routine shortcut to avoid full LOTO.

OPEN ACTION [OA-P5-006] Partial isolation procedure conditions and authorization protocol to be finalized by Indo-MIM maintenance supervisor and Tata Electronics safety representative. Responsibility: Indo-MIM + Tata Electronics ¦ Target: Before V2.0 final release ¦ Status: Open

5.  EMERGENCY STOP FUNCTION

The E-stop is a last resort — not a pause button, not a production stop, and not a substitute for LOTO. Use it when something unexpected is happening and you need everything to stop immediately. For planned maintenance, use LOTO.

Designed in accordance with ISO 13850:2015 — Emergency Stop Function — Principles for Design.

5.1  E-Stop Device Locations

| E-Stop ID | Location | Station | Access |
| --- | --- | --- | --- |
| ES-01 | IPC control panel — left side | S1 Loading Machine | Front of machine |
| ES-02 | IPC control panel — left side | S2 Snap Assy & Screwing | Front of machine |
| ES-03 | IPC control panel — left side | S3 Snap Assy & Screwing | Front of machine |
| ES-04 | Flipping station control panel | S4 Flipping Module | Front of machine |
| ES-05 | IPC control panel — left side | S5 Snap Assy & Screwing | Front of machine |
| ES-06 | IPC control panel — left side | S6 Unloading Machine | Front of machine |
| ES-07 | [PLACEHOLDER — location TBC: site visit confirmation required] | [PLACEHOLDER] | [PLACEHOLDER] |
| ES-08 | [PLACEHOLDER — location TBC: site visit confirmation required] | [PLACEHOLDER] | [PLACEHOLDER] |
| ES-09 | [PLACEHOLDER — location TBC: site visit confirmation required] | [PLACEHOLDER] | [PLACEHOLDER] |
| ES-10 | [PLACEHOLDER — location TBC: site visit confirmation required] | [PLACEHOLDER] | [PLACEHOLDER] |
| ES-11 | [PLACEHOLDER — location TBC: site visit confirmation required] | [PLACEHOLDER] | [PLACEHOLDER] |

OPEN ACTION [OA-P5-007] Physical E-stop locations, mounting heights, and panel positions to be verified during site visit at Tata Electronics. Locations above are from DFM V1.4. Responsibility: Certified Safety Professional — site visit ¦ Target: Dry run ¦ Status: Open

5.2  Stop Category

Category 1 Stop (IEC 60204-1:2016) — controlled deceleration to standstill, then power removal. Selected because high-speed servo axes and KUKA robots require controlled deceleration to prevent mechanical damage and secondary hazards.

5.3  E-Stop Activation Sequence

(1) Signal transmitted to Safety PLC via dual-channel hardwired circuit. (2) All HIWIN and Toyo axes decelerate to standstill under controlled braking. KUKA robots execute Category 1 stop per ISO 10218-1. (3) STO applied to all servo drives once standstill reached. Maglev conveyor de-energized. (4) Pneumatic system moves to fail-safe state. (5) All five IPC stations display E-stop active alarm with station identifier.

5.4  E-Stop Reset Procedure

The E-stop function is latching — cannot be reset remotely or automatically. Follow all steps.

WARNING [ISO 7010 W019] The machine will move on restart. Before pressing RESET, walk the machine and identify why the E-stop activated. If you cannot identify the cause, do not reset — call the maintenance technician.

Step 1 — Identify Cause: Walk full machine perimeter. Identify reason. Resolve any hazardous condition. Confirm all personnel clear.

Step 2 — Release E-Stop Pushbutton: At the activated station, rotate mushroom-head pushbutton clockwise until released. Cannot be released from IPC.

Step 3 — Acknowledge Alarm at IPC: At the station IPC, acknowledge the E-stop alarm. Confirm no other active alarms before proceeding.

Step 4 — Confirm Safety System Status: Confirm Safety PLC reports all safety functions healthy. All door interlocks engaged. Light curtains unobstructed.

Step 5 — Restart Authorization: Only authorized operator or technician may authorize restart. Must be initiated from IPC.

Step 6 — Initiate Controlled Restart: Press RESET then CYCLE START. Machine performs controlled homing before resuming operation. Monitor first cycle carefully.

Step 7 — Record E-Stop Event: Record in maintenance log: station, time, cause, corrective action, name of person who reset.

5.5  What the E-Stop Does NOT Do

Does not isolate the 230V AC main electrical supply — the main panel remains energized.

Does not immediately vent all pneumatic pressure — residual pressure may remain.

Does not substitute for LOTO — never use E-stop as a lockout device for maintenance.

Does not prevent re-energization by another person — apply your personal LOTO padlock when entering.

5.6  Periodic E-Stop Testing

| Test | Frequency | Acceptance Criteria |
| --- | --- | --- |
| Daily (once per shift) | Operator activates each accessible E-stop button. Machine must enter safe state. Reset and confirm normal operation resumes before production starts. | Operator |
| Functional test — all E-stop buttons | Weekly | All axes stop, STO applied, IPC alarm displayed, no auto-restart |
| Dual-channel integrity test | Monthly | Safety PLC fault alarm within 500 ms on channel disconnect |
| Response time verification | Six-monthly | Standstill achieved within manufacturer-specified deceleration time |
| Full safety function audit | Annually | All safety functions verified to PLd/Category 3 per ISO 13849-1 |

6.  OPERATING INSTRUCTIONS

During automatic operation your role is: load trays at S1, collect trays at S6, monitor the IPC. Do not interfere with the automatic cycle. If something looks wrong, press E-stop and call the technician.

6.1  Operating Modes

| Mode | Description | Who Can Select |
| --- | --- | --- |
| Automatic | Full automatic assembly cycle. Runs continuously. | Trained Operator |
| Manual | Individual station/axis movements one step at a time. Setup, changeover, troubleshooting only. | Setup Technician or above |
| Maintenance | All automatic functions disabled. Used with LOTO during maintenance. | Maintenance Technician or above |

6.2  Pre-Start Checklist

| # | Check Item | Required Condition |
| --- | --- | --- |
| 1 | All access doors and safety guards | Fully closed and latched |
| 2 | All E-stop pushbuttons ES-01 to ES-06 | Released — buttons in extended position |
| 3 | Safety light curtains at S1 and S6 | Unobstructed — green indicator active |
| 4 | Compressed air pressure gauge at inlet | 0.5 to 0.7 MPa |
| 5 | Main electrical isolator LP-01 | ON position |
| 6 | All IPC displays | Powered on — no active alarms |
| 7 | KUKA robot controllers S1 and S6 | Powered on — no fault indicators |
| 8 | Maglev conveyor | Powered on — status normal on IPC |
| 9 | Input tray area at S1 | Clear of obstructions — tray loaded with HSG units |
| 10 | Output tray area at S6 | Empty tray in position |
| 11 | Screw feeders S2, S3, S5 | Loaded — feeder level indicator showing adequate supply |
| 12 | Tape reel feeders S2, S3, S5 | Snap tape reels loaded and threaded correctly |
| 13 | LOTO log | No open LOTO entries — machine released from maintenance |
| 14 | Work area around machine | Clear of personnel, tools, and obstructions |

6.3  Machine Startup Procedure

Pre-condition: All pre-start checks in Section 6.2 completed.

Step 1 — Power On: Confirm LP-01 in ON position. Confirm all IPC displays showing home screen.

Step 2 — Clear All Alarms: At each IPC (S1–S6), acknowledge and clear residual alarms. Do not proceed if any alarm cannot be cleared.

Step 3 — Initialize Safety System: At S1 IPC, confirm Safety PLC reports all functions healthy. Confirm all door interlocks engaged and light curtains active.

Step 4 — Home All Axes: Select Homing at S1 IPC. Initiate automatic homing sequence. Do not interrupt once started.

Step 5 — Initialize KUKA Robots: Confirm both robots (S1 and S6) show ready status. Contact maintenance if either shows a fault.

Step 6 — Initialize Maglev Conveyor: Confirm Maglev initialized and all fixture movers detected on IPC.

Step 7 — Select Recipe: Navigate to Recipe screen. Select correct product recipe. Confirm name and version match production order.

Step 8 — Select Automatic Mode: Select Automatic Mode at S1 IPC. Confirm mode indicator changes on all station IPCs.

Step 9 — Load Input Tray: Place loaded input tray at S1. Seat in guide rails. Push until clamping jaws engage audibly. Confirm tray detected on IPC.

Step 10 — Start Cycle: Press CYCLE START at S1 IPC. Monitor first cycle to confirm all stations operating normally.

6.4  Normal Operating Cycle

| Station | Sequence |
| --- | --- |
| S1 — Loading | KUKA robot picks HSG → barcode scan → secondary positioning fixture → Maglev fixture mover. Barcode fail → NG conveyor. |
| S2 — 1st Snap & Screw | Tape reel advances snap. Voice coil picks snap. CCD pre-inspection. Snap placed at 1st position. Atlas screwdriver drives 2 screws to torque. CCD recheck. Torque or seating NG → screws tossed, HSG flagged. |
| S3 — 2nd Snap & Screw | Same sequence as S2 for the 2nd snap position. |
| S4 — Flip | Flipping module suctions HSG, rotates 180°, replaces in fixture with reverse face exposed. |
| S5 — 3rd Snap & Screw | Same sequence as S2/S3 for 3rd snap on reverse face. |
| S6 — Unloading | KUKA robot picks completed HSG. OK → output tray. NG at any station → buffer fixture or NG tray. |

6.5  Process Flowchart

PLACEHOLDER — Process Flowchart — Normal Operating Cycle (Figure 6-1) S1→S2→S3→S4(flip)→S5→S6 with OK/NG decision paths at barcode scan, torque/seating check, and final all-stations check. Colours: #1F3864 process boxes, dashed NG paths. Legend lower-left. Full page width, 300 DPI.

6.6  Normal Shutdown

Step 1 — : Let the current cycle finish. Mid-cycle stops leave partially assembled HSG units requiring manual clearance.

Step 2 — : At S1 IPC, press CYCLE STOP. Machine completes current cycle and comes to a controlled stop.

Step 3 — : Remove input trays from S1, output trays from S6. Remove and segregate NG parts from buffer tray.

Step 4 — : At IPC, select Controlled Shutdown. Machine homes all axes, de-energizes Maglev, powers down robot controllers.

Step 5 — : Once IPC confirms shutdown complete, turn LP-01 to OFF if required by site procedure.

Step 6 — : Record any alarms, faults, or observations in the machine logbook.

6.7  Emergency Shutdown

Use only when an immediate stop is required due to an unsafe condition.

Step 1 — : Press the nearest E-stop (ES-01 through ES-06). Machine executes Category 1 controlled stop.

Step 2 — : Do not attempt to restart until the cause has been identified and resolved.

Step 3 — : Follow E-stop reset procedure (Section 5.4) before returning to service.

Step 4 — : Record the event in the logbook. Report to shift supervisor immediately.

7.  MAINTENANCE & CARE

All maintenance must be performed by trained and authorized technicians only. Complete full LOTO (Section 4) before entering the guarding envelope for any task.

WARNING [ISO 7010 P010] Every task that requires entering the machine requires LOTO first. Stated once here — applies everywhere. No exceptions for quick jobs or familiar tasks.

7.1  Weekly Maintenance

| # | Task | Station | LOTO |
| --- | --- | --- | --- |
| W-01 | Test all E-stop pushbuttons individually — verify controlled stop and IPC alarm | All | No |
| W-02 | Inspect all fixed guards and enclosure panels — verify fasteners are tight | All | No |
| W-03 | Clean CCD camera lenses (upper and lower) — lint-free cloth and approved optical solution only | S2,S3,S5 | ⚠️ LOTO |
| W-04 | Clean LMI Gocator 3D sensor windows — lint-free cloth only, no solvents | S2,S3,S5 | ⚠️ LOTO |
| W-05 | Inspect HIWIN/Toyo linear axis rails — remove debris, check for contamination | All | ⚠️ LOTO |
| W-06 | Inspect KUKA robot EOAT grippers and nozzles — check for wear, damage, or contamination | S1,S6 | ⚠️ LOTO |
| W-07 | Inspect snap tape reel feeder mechanism — clean feed path, check for tape debris | S2,S3,S5 | ⚠️ LOTO |
| W-08 | Inspect Deprag screw feeder — clean feed track, check for jammed or deformed screws | S2,S3,S5 | ⚠️ LOTO |
| W-09 | Inspect Atlas screwdriver nozzles — check for wear and screw seating integrity | S2,S3,S5 | ⚠️ LOTO |
| W-10 | Inspect Maglev mover carriages — check for debris on rail surface | Full line | ⚠️ LOTO |
| W-11 | Inspect fixture bodies — check push blocks, datum blocks, clamping surfaces | All | ⚠️ LOTO |
| W-12 | Check pneumatic tubing and fittings for leaks or damage | All | No |
| W-13 | Verify RFID reader status at all six stations — confirm correct fixture ID reading | All | No |

7.2  Monthly Maintenance

| # | Task | Station |
| --- | --- | --- |
| M-01 | Lubricate HIWIN linear guide rail blocks — apply recommended grease via grease nipple. See OA-P5-008. | All |
| M-02 | Lubricate Toyo servo axis ball screw — apply recommended grease. See OA-P5-008. | All |
| M-03 | Inspect and clean Pepperl+Fuchs RFID reader faces — check mounting and cable condition | All |
| M-04 | Inspect Keyence safety door latch mechanisms — check latch pin condition and actuator alignment | All |
| M-05 | Test dual-channel E-stop circuit integrity at Safety PLC input terminals — verify fault alarm within 500 ms | All |
| M-06 | Check servo drive cooling fan operation — confirm airflow, listen for abnormal noise | All |
| M-07 | Inspect pneumatic cylinder seals at tray locking and flipping modules — check for air weeping | S1,S4,S6 |
| M-08 | Check Maglev mover wheel condition — inspect for flat spots or uneven wear | Full line |
| M-09 | Verify Atlas screwdriver torque calibration — run calibration check per Atlas MTF6000. Confirm within ±5% of set value. | S2,S3,S5 |
| M-10 | Inspect IPC/motion card cooling — clean dust filters on Advantech IPC cabinets | All |
| M-11 | Back up IPC recipe files and PLC program to approved storage location | All |
| M-12 | Inspect voice coil actuator stroke — verify correct snap pick-up and placement position | S2,S3,S5 |

7.3  Six-Monthly Maintenance

| # | Task | Station | Specialist? |
| --- | --- | --- | --- |
| SM-01 | Full safety function audit — verify all PLe/Cat4 safety functions per ISO 13849-1 verification checklist | All | Authorized safety technician |
| SM-02 | KUKA robot full calibration check — verify TCP, axis positions, and working envelope | S1,S6 | KUKA authorized engineer |
| SM-03 | Vision system full calibration — all CCD cameras using calibration plate | S2,S3,S5 | Vision-trained technician |
| SM-04 | LMI Gocator 3D sensor full calibration — using reference target | S2,S3,S5 | Vision-trained technician |
| SM-05 | Replace Atlas screwdriver nozzles — scheduled replacement regardless of visual condition | S2,S3,S5 | Maintenance technician |
| SM-06 | Replace fixture push blocks (PU material) — scheduled replacement due to wear | All | Maintenance technician |
| SM-07 | Full electrical inspection — cable terminations, earth continuity, insulation resistance | All | Qualified electrician |
| SM-08 | Maglev conveyor full system check — mover positioning accuracy and rail alignment | Full line | Beckhoff authorized technician |
| SM-09 | E-stop response time verification — measure time from activation to full axis standstill | All | Authorized safety technician |

7.4  Consumables & Spare Parts

| Item | Station | Replacement Trigger | Min. Stock |
| --- | --- | --- | --- |
| Snap components (613-31681, 613-38625, 613-38626) | S2,S3,S5 | Tape reel empty | Per production plan |
| Screws (M-type per assembly spec) | S2,S3,S5 | Feeder at minimum level | 2-shift supply minimum |
| Atlas screwdriver nozzle tips (QMC-21-10-HM4) | S2,S3,S5 | Worn / 6-monthly schedule | 4 sets minimum |
| Fixture push blocks (PU material) | All | Worn or deformed / 6-monthly | 2 sets per station |
| Fixture datum blocks (PPS) | All | Cracked or out of tolerance | 1 set per station |
| Tape reel feeder drive belt (YIG compatible) | S2,S3,S5 | Worn or slipping | 2 per feeder type |
| Safety door latch (Keyence GS-51P5 complete) | All | Latch pin worn / interlock fault | 2 units minimum |

OPEN ACTION [OA-P5-008] Lubrication schedule — lubricant types, approved brands, application quantities, and service intervals for all HIWIN axes, Toyo axes, KUKA robot joints, and pneumatic cylinders — pending Indo-MIM engineering confirmation. Responsibility: Indo-MIM Engineering ¦ Target: Before manual finalization ¦ Status: Open

OPEN ACTION [OA-P5-009] Troubleshooting guide — common fault conditions, probable causes, and corrective actions — pending Indo-MIM engineering and Tata Electronics maintenance input following site visit. Responsibility: Indo-MIM Engineering + Tata Electronics Maintenance ¦ Target: Before manual finalization ¦ Status: Open

8.  ELECTRICAL SCHEMATIC

WARNING [ISO 7010 W003] The main electrical panel operates at 230V, Single Phase, [PLACEHOLDER — confirm current rating]. Access restricted to authorized electricians only. Complete full LOTO including LP-01 isolation and voltage verification before opening any electrical enclosure.

8.1  Key Electrical Parameters

| Parameter | Specification |
| --- | --- |
| Main Supply | 230V AC AC, 3-phase, 50 Hz |
| Maximum Current | [PLACEHOLDER — confirm current rating] |
| Total Connected Power | 54 kW (full line) |
| Station Supply | 220V AC per station |
| Control Voltage | 24V DC (Safety PLC and I/O) |
| Robot Controller | KUKA KR CS Box-2 |
| Safety PLC | FLX3 series |
| Motion Control | Advantech IPC 7132 + PCIE-1203 motion card |
| Communication | EtherCAT (motion), TCP/IP (MES/TRACE), OPC-UA |
| Main Isolator | Lockable rotary type, [PLACEHOLDER — confirm current rating], main panel |
| Protection Standard | IEC 60204-1:2016 |

8.2  Electrical Schematic Drawings

PLACEHOLDER — Full Electrical Schematic Drawing Set (Figure 8-1) Drawing set: (1) Main power distribution single-line (230V AC/3-phase/[PLACEHOLDER — confirm current rating]); (2) Control circuit — Safety PLC I/O wiring; (3) E-stop dual-channel circuit; (4) Servo drive and STO wiring; (5) KUKA robot controller interface; (6) Maglev conveyor PLC wiring; (7) Cable schedule. Source: Indo-MIM electrical design team. A3 at 300 DPI.

OPEN ACTION [OA-P5-002] Electrical schematic drawings for the BG Snap Assembly Machine have not been provided at the time of preparation of this manual. Responsibility: Indo-MIM Engineering ¦ Target: Before manual finalization ¦ Status: Open

9.  PNEUMATIC SCHEMATIC

WARNING [ISO 7010 W017] Residual pneumatic pressure remains in downstream lines after LP-02 isolation until fully bled down. Always verify pressure gauge reads zero before reaching near any pneumatic actuator.

9.1  Key Pneumatic Parameters

| Parameter | Specification |
| --- | --- |
| Supply Pressure | 0.5 to 0.7 MPa |
| Air Quality | Clean, dry, oil-free compressed air |
| Main Isolation | Solenoid valve — de-energise via IPC/control panel; manually verify vent before entry (LP-02) |
| Actuator Types | SMC and FESTO cylinders — tray locking, flipping, rotating |
| Solenoid Valves | 24V DC solenoid actuation via Safety PLC I/O |
| Fail-Safe Design | System dumps to safe state on E-stop activation |
| Design Standard | ISO 4414 — Pneumatic Fluid Power: Safety Requirements |

9.2  Pneumatic Schematic Drawings

PLACEHOLDER — Full Pneumatic Circuit Diagram (Figure 9-1) Diagram: (1) Main air supply to machine manifold; (2) FRL unit at inlet; (3) Solenoid valve bank all stations; (4) Individual actuator circuits; (5) ISO 1219-1 symbol legend; (6) Pressure settings table. Source: Indo-MIM pneumatic design team. A3 at 300 DPI.

OPEN ACTION [OA-P5-010] Pneumatic schematic drawings for the BG Snap Assembly Machine have not been provided at the time of preparation of this manual. Responsibility: Indo-MIM Engineering ¦ Target: Before manual finalization ¦ Status: Open

10.  HYDRAULIC SCHEMATIC

NOT APPLICABLE — The BG Snap Assembly Machine does not use any hydraulic systems. All motion is achieved through servo-driven electric axes, KUKA robots, and pneumatic actuators. This section is retained for completeness of the standard documentation framework.

11.  IPC ALARMS & FAULT CODES

The machine is equipped with five IPC stations (one per major section). All alarms are displayed on the relevant station IPC and simultaneously logged in the SQL-based production database.

11.1  Alarm Priority Levels

| Level | Colour | Description | Machine Response |
| --- | --- | --- | --- |
| 1 — Critical | Red | Safety-related or machine-stopping fault. Immediate attention required. | Machine stops automatically. Cycle cannot restart until alarm resolved. |
| 2 — Warning | Amber | Non-critical condition requiring attention within current shift. | Machine continues. Operator must acknowledge and monitor. |
| 3 — Information | Blue | Operational notification — no immediate action required. | Machine continues. Logged automatically. |

11.2  IPC Alarm Code List

PLACEHOLDER — Full IPC Alarm Code Table (Figure 11-1) Table columns: Alarm Code ¦ Description ¦ Station ¦ Priority (1/2/3) ¦ Probable Cause ¦ Operator Action ¦ Reset Procedure. Source: Indo-MIM software engineering — to be extracted from C# alarm module and SQL alarm table.

OPEN ACTION [OA-P5-003] Complete IPC alarm code list has not been provided at the time of preparation of this manual. Responsibility: Indo-MIM Engineering ¦ Target: Before manual finalization ¦ Status: Open

11.3  General Alarm Response Procedure

Step 1 — Read the Alarm: Note alarm code, description, and station on IPC. Do not acknowledge before understanding the alarm.

Step 2 — Assess Priority: Identify priority level from colour. For Level 1 Critical, stop all manual interaction and contact maintenance.

Step 3 — Do Not Override: Never use Manual Mode or maintenance bypass to override an active alarm without identifying the cause.

Step 4 — Contact Maintenance: For any alarm outside trained knowledge, contact maintenance immediately.

Step 5 — Record the Alarm: Record alarm code, time, and observations in the machine logbook.

12.  SAFETY FEATURES & INTERLOCKS

This section documents what is protecting you, how it works, and what happens when it activates. Operators must not modify, bypass, or interfere with any safety device or interlock under any circumstances.

12.1  Safety Architecture

| Level | Measure |
| --- | --- |
| 1 — Inherently Safe Design | Enclosed station housings, Maglev routing, and single operator interaction point at defined loading/unloading zones minimize access to hazardous zones during normal operation. |
| 2 — Fixed Guarding | All stations enclosed within fixed metal guarding panels fastened with tools. Must be reinstalled and fully fastened after any maintenance before return to service. |
| 3 — Interlocked Safeguarding | All access doors interlocked with Safety PLC. Opening any door during operation triggers safety-rated stop. Light curtains at S1 and S6 provide area sensing at operator interaction points. |
| 4 — Complementary Measures | E-stop pushbuttons at all six stations. Warning labels, PPE, operator training, and this manual address residual risks not eliminated by Levels 1–3. |

12.2  Safety Control System

| Parameter | Details |
| --- | --- |
| Safety PLC | FLX3 series — dual-channel, self-monitoring |
| Standard | ISO 13849-1:2015 |
| Independence | Operates independently from machine software (C#). Machine app fault does not affect Safety PLC. |
| Safety Outputs | Dual contactor control — servo drives and Maglev conveyor |
| STO Function | Safe Torque Off on all HIWIN and Toyo servo drives |
| Robot Interface | KUKA KR CS Box-2 safety I/O — ISO 10218-1 compliant |
| E-Stop Circuit | Dual-channel hardwired, monitored by Safety PLC |
| Door Circuit | Dual-channel per door, monitored by Safety PLC |
| Light Curtain | Direct Safety PLC input — OSSD dual-channel |
| Diagnostic Coverage | High DC per IEC 62061 |

12.3  Safety Device Inventory

OPEN ACTION [OA-P5-013] Safety device inventory is from DFM V1.4 and RA. Physical installation, cable routing, mounting positions, and label verification must be confirmed during dry run at Tata Electronics. Responsibility: Certified Safety Professional — site visit ¦ Target: Dry run ¦ Status: Open

| ID | Type | Brand & Model | Station | PL/Cat | Function |
| --- | --- | --- | --- | --- | --- |
| SD-01 | Safety door latch | Keyence GS-51P5 | S1 | PLe / Cat 4 | Interlocks S1 access door |
| SD-02 | Safety door latch | Keyence GS-51P5 | S2 | PLe / Cat 4 | Interlocks S2 access door |
| SD-03 | Safety door latch | Keyence GS-51P5 | S3 | PLe / Cat 4 | Interlocks S3 access door |
| SD-04 | Safety door latch | Keyence GS-51P5 | S4 | PLe / Cat 4 | Interlocks S4 access door |
| SD-05 | Safety door latch | Keyence GS-51P5 | S5 | PLe / Cat 4 | Interlocks S5 access door |
| SD-06 | Safety door latch | Keyence GS-51P5 | S6 | PLe / Cat 4 | Interlocks S6 access door |
| SD-07 | Safety light curtain | Keyence GL-VM39FP | S1 | PLe / Cat 4 | Area sensing at tray loading zone |
| SD-08 | Safety light curtain | Keyence GL-VM39FP | S6 | PLe / Cat 4 | Area sensing at tray unloading zone |
| SD-09 | E-stop pushbutton | Per IEC 60947-5-5 | S1 | PLd / Cat 3 | E-stop ES-01 |
| SD-10 | E-stop pushbutton | Per IEC 60947-5-5 | S2 | PLd / Cat 3 | E-stop ES-02 |
| SD-11 | E-stop pushbutton | Per IEC 60947-5-5 | S3 | PLd / Cat 3 | E-stop ES-03 |
| SD-12 | E-stop pushbutton | Per IEC 60947-5-5 | S4 | PLd / Cat 3 | E-stop ES-04 |
| SD-13 | E-stop pushbutton | Per IEC 60947-5-5 | S5 | PLd / Cat 3 | E-stop ES-05 |
| SD-14 | E-stop pushbutton | Per IEC 60947-5-5 | S6 | PLd / Cat 3 | E-stop ES-06 |
| SD-15 | Servo STO | Integrated in drives | All | PLd / Cat 3 | Safe Torque Off — all HIWIN and Toyo axes |
| SD-16 | Robot safety I/O | KUKA KR CS Box-2 | S1,S6 | PLd / Cat 3 | Robot safety stop per ISO 10218-1 |
| SD-17 | Dual contactor | Integrated in panel | All | PLe / Cat 4 | Power removal on safety demand |

12.4  Safety Interlock Logic

| Safety Condition | Safety PLC Response | Machine Response | Restart Method |
| --- | --- | --- | --- |
| E-stop activated (any station) | STO all drives | Category 1 stop — all axes | Manual reset + IPC acknowledge |
| Safety door opened (any station) | STO affected station | Category 1 stop — affected station | Door closed + IPC acknowledge + manual restart |
| Light curtain beam broken (S1/S6) | STO to S1 or S6 | Category 1 stop — affected station | Beam cleared + IPC acknowledge + manual restart |
| Safety PLC fault | STO all drives | Full machine stop | Fault resolved + authorized reset |
| Power failure | STO by default | All drives de-energized | Full startup sequence |
| Robot safety I/O fault | Robot safety stop | Robot stops — other stations may continue | Robot fault resolved + IPC acknowledge |

12.5  Safety Function Testing Requirements

| Safety Function | Frequency | Acceptance Criteria |
| --- | --- | --- |
| Safety door interlock — all doors | Weekly | Machine stops on door open; alarm displayed; no auto-restart |
| Safety light curtain — S1 and S6 | Weekly | Machine stops on beam break; curtain alarm; green indicator returns on clear |
| E-stop function — all buttons | Weekly | All axes stop; STO confirmed; IPC alarm displayed |
| Dual-channel door interlock integrity | Monthly | Safety PLC fault alarm within 500 ms on channel disconnect |
| Dual-channel light curtain integrity | Monthly | Safety PLC fault alarm within 500 ms on OSSD disconnect |
| STO on all servo drives | Monthly | Motor produces no torque with STO active |
| Robot safety stop | Monthly | Robot stops within defined deceleration time; no fault on restart |
| Full safety function audit | Six-monthly | All safety functions verified to stated PL and Category |

12.6  Prohibited Actions

Bypassing, defeating, or disabling any safety door interlock, light curtain, E-stop, or STO for any reason

Entering any guarded zone without completing full LOTO (Section 4)

Modifying Safety PLC parameters or safety circuit wiring without written authorization from Indo-MIM engineering

Removing or covering safety warning labels on the machine

Operating the machine with any fixed guard removed or access door open

Using tape, cable ties, or any device to hold a safety door latch closed while the door is open

Allowing untrained or unauthorized personnel to operate the machine in any mode

WARNING [ISO 7010 W028] The KUKA KR13 moves at up to 2700°/s with a 13 kg payload. There is no safe distance inside the working envelope while the robot is powered. Complete LP-04 isolation — both S1 and S6 controllers — before entering. Every time.

13.  COMPLIANCE & CERTIFICATIONS

13.1  International Safety Standards

| Standard | Title | Relevance |
| --- | --- | --- |
| ISO 12100:2010 | Safety of Machinery — Risk Assessment and Risk Reduction | Overall risk methodology — all sections |
| ISO 13849-1:2015 | Safety-Related Parts of Control Systems | Safety PLC, STO, interlocks, E-stop |
| ISO 13850:2015 | Emergency Stop Function | E-stop design — Section 5 |
| ISO 14119:2013 | Interlocking Devices for Guards | Safety door latches — Section 12 |
| ISO 14120:2015 | Guards — General Requirements | Fixed guard design |
| ISO 14118:2017 | Prevention of Unexpected Start-Up | Restart prevention — Section 12 |
| ISO 13855:2024 | Positioning of Safeguards | Light curtain positioning |
| ISO 10218-1:2011 | Safety Requirements for Industrial Robots | KUKA robot integration |
| ISO 20607:2019 | Instruction Handbook | Manual structure |
| IEC 82079-1:2019 | Preparation of Instructions for Use | Signal word hierarchy |
| IEC 60204-1:2016 | Electrical Equipment of Machines | Panel design — Section 8 |
| IEC 61496-1:2020 | Electro-Sensitive Protective Equipment | Light curtain spec |

13.2  Indian Standards & Regulations

| Standard | Title | Relevance |
| --- | --- | --- |
| IS 4571:2008 | Safety of Machinery (Indian Standard) | Indian regulatory compliance — all sections |
| IS 12100:2012 | Safety of Machinery — General Principles (Indian adoption) | Risk assessment under Indian standards framework |
| Factories Act 1948 | Indian Industrial Labour Law — Section 21: Fencing of Machinery | Legal basis for machine guarding at Tata Electronics |

13.3  Component-Level Standards

| Component | Applicable Standard | Compliance Basis |
| --- | --- | --- |
| Safety door latch (Keyence GS-51P5) | ISO 14119, IEC 60947-5-3 | Manufacturer's declaration — PLe / Cat 4 / SIL3 |
| Safety light curtain (Keyence GL-VM39FP) | IEC 61496-1, ISO 13849-1 | Manufacturer's declaration — PLe / Cat 4 / SIL3 |
| Safety PLC (FLX3 series) | ISO 13849-1 | Manufacturer's declaration |
| KUKA KR13 R850 Z200-2 CS | ISO 10218-1, IEC 60204-1 | Manufacturer's declaration — CE marked |
| Atlas QMC-21-10-HM4 | IEC 60745, ISO 11148 series | Manufacturer's declaration |
| Deprag Screw Feeder | IEC 60204-1 | Manufacturer's declaration |

13.4  Compliance Statement

This manual was prepared by Certified Safety Professional, based on the Machine Safety Assessment Report (V1.1, March 2026), DFM V1.4 (March 2026), and the standards listed above. Where the machine was not available for physical inspection, gaps are documented as Open Actions — not assumed, not ignored, not filled with generic content. The Open Actions Register closes when the machine is verified, not before.

Final review and verification is the responsibility of Indo-MIM Limited as machine manufacturer and Tata Electronics as end user. All Open Actions must be closed before this manual is released as a controlled document and the machine enters production service.

13.5  Document Control

| Field | Details |
| --- | --- |
| Document Title | Operating & Safety Manual — BG Snap Assembly Machine |
| Document Number | IndoMIM-BGSnap-OSM-001 |
| Revision | V2.0 |
| Classification | Confidential — Indo-MIM Limited / Tata Electronics |
| Language | English |
| Format | Word (.docx) — editable / PDF (.pdf) — controlled copy |
| Retention Period | Minimum 10 years from commissioning per Factories Act 1948 |
| Next Review Date | Following site visit and dry run — target V2.0 final release |

13.6  Customer Sign-Off

The following sign-off is required before this manual is released as a controlled document and the machine is placed into production service.

Indo-MIM Limited — Manufacturer Authorization

I confirm that this manual has been reviewed by Indo-MIM Limited's engineering and quality team, all technical content verified as accurate, and all Open Actions assigned to Indo-MIM reviewed and accepted for closure.

| Name: | Designation: | Signature: | Date: |
| --- | --- | --- | --- |

Tata Electronics — End User Authorization

I confirm that this manual has been reviewed by Tata Electronics' EHS and operations teams, all procedures are suitable for the facility's operating conditions, and all Open Actions assigned to Tata Electronics reviewed and accepted for closure.

| Name: | Designation: | Signature: | Date: |
| --- | --- | --- | --- |

Certified Safety Professional — Manual Author

I confirm that this manual has been prepared in accordance with applicable standards and professional obligations, and that all known safety gaps have been formally documented as Open Actions.

| Name: | Designation: | Signature: | Date: |
| --- | --- | --- | --- |

APPENDIX A — RESIDUAL RISK SUMMARY

The safeguards on this machine are good. They are not perfect. This appendix tells you what risk remains after everything that could be engineered has been engineered. These risks are real — they are low, but only because the people working with this machine respect the procedures.

All operators, setup technicians, and maintenance technicians must read this appendix in full before working with or near this machine.

[SECTION REMOVED — Residual Risk Summary appendix permanently removed per OSM standard V1.2]

| Hazard | Control Applied | Residual Risk | Requirement |
| --- | --- | --- | --- |
| Servo axis shearing (maintenance) | Interlocked safety doors (PLe/Cat4), STO, LOTO | Low | Full LOTO per Section 4 before entering |
| Electrical shock — live parts (230V AC) | Lockable main isolator LP-01, voltage verification, LOTO | Low | Full LOTO; voltage test before panel work |
| Stored energy — DC bus capacitors | 60-second wait after LP-01 isolation | Low | 60-second wait mandatory — no exceptions |
| KUKA robot impact during operation | Fixed guarding, interlocked door, light curtain (PLe/Cat4) | Low | Light curtain must be unobstructed during operation |
| KUKA robot impact during maintenance | LP-04 isolation of both robot controllers | Low | Full LOTO including LP-04 before entering S1 or S6 |
| Snap/screw feeder entanglement | Fixed guarding, interlocked access, LOTO | Low | Full LOTO before accessing feeder mechanisms |
| Tray lifter gravity hazard | STO on Z-axis servo; LP-01 and LP-03 required | Low | Isolate LP-01 and confirm LP-03 STO before reaching under tray lifter |
| Residual pneumatic pressure after LP-02 | Bleed-down via pressure gauge verification | Low | Confirm gauge at zero before reaching near pneumatic actuators |
| Ergonomic strain — tray loading/unloading | Operator training, correct posture guidance | Low | Use correct manual handling posture; observe tray weight limits |
| Unexpected restart after E-stop reset | Mandatory manual restart sequence from IPC | Low | Follow E-stop reset procedure in Section 5.4 fully |

A.2  PPE Requirements

| PPE Item | Required For | ISO 7010 |
| --- | --- | --- |
| Safety footwear (steel-capped) | All personnel at all times on shop floor | M009 |
| Protective gloves (cut-resistant) | Maintenance technicians during LOTO and component handling | M008 |
| Safety glasses | Maintenance during pneumatic work or component handling | M004 |
| High-visibility vest | Any personnel entering machine access zones | — |

APPENDIX B — MAINTENANCE LOG TEMPLATE

All completed maintenance tasks must be recorded below. One entry per task.  Result codes: S = Satisfactory  |  R = Item Replaced  |  F = Fault Found  |  D = Deferred

| Date | Task # | Description | Station | Result | Observations | Technician | Sign |
| --- | --- | --- | --- | --- | --- | --- | --- |

Maintenance records must be retained by Tata Electronics for a minimum of three years per Factories Act 1948.

APPENDIX C — LOTO LOG TEMPLATE

A new entry must be made for every LOTO event without exception. Log must remain at machine at all times.  Stations/Points: list all isolation points applied (e.g. LP-01, LP-02, LP-04 S1, LP-05).

| Date | Time Out | Time In | Stations / Points | Reason | Technician Name | Observations | Sign |
| --- | --- | --- | --- | --- | --- | --- | --- |