# TRUCK DRILLING MACHINE OPERATING MANUAL

Model: Truck Multi-Spindle Drilling Machine (48-Spindle Configuration)
 Version: 8.0
 Effective Date: January 2025
 Document Classification: Technical / Operational

## TABLE OF CONTENTS

- General & Legal Information

- Machine Identification & Overview

- Technical Specifications & Data

- Safety Information & Warnings

- Transport, Installation & Commissioning

- Operating Procedures

- Maintenance & Lubrication

- Troubleshooting & Alarm Codes

- Spare Parts & Consumables

- Appendices

## 1. GENERAL & LEGAL INFORMATION

### 1.1 Manufacturer Details

| Item | Details |
| --- | --- |
| Manufacturer | Accurate Machines & Accessories (Pvt) Ltd. |
| Address | Ambattur, Chennai – 600 098, India |
| Contact Number |  |
| Email |  |
| Website |  |

### 1.2 Machine Purpose & Application

This Truck Drilling Machine is designed for high-precision drilling of clutch facing components made of composite materials (organic and inorganic fibers with thermosetting resins). The machine conforms to Valeo quality standards and is intended for automated manufacturing environments.

Key Application: Drilling multiple holes in clutch facings as per Valeo specifications with automatic poka-yoke quality checks.

### 1.3 Regulatory Compliance & Standards

This machine is designed and manufactured in accordance with the following Indian and International standards:

- IS 4571:1968 – General Safety Requirements for Machinery

- IS 12100-1:2003 – Safety of Machinery (General Principles of Design)

- IS 12100-2:2003 – Safety of Machinery (Technical Principles and Specifications)

- IS 13598:2013 – Industrial Safety Management Systems

- ISO 14121 – Risk Assessment & Reduction

- Valeo General Equipment Specifications (D/Meth/025 Rev. 12)

## 2. MACHINE IDENTIFICATION & OVERVIEW

### 2.1 Machine Identification Data

| Parameter | Specification |
| --- | --- |
| Machine Type | Truck Multi-Spindle Drilling Machine |
| Configuration | 48-Spindle (Gang Drilling) |
| Serial Number |  |
| Year of Manufacture | 2025 |
| Control System | Mitsubishi FX5U PLC + ProFace 10" HMI |
| Electrical Supply | 3-Phase, 415V AC, 50 Hz |
| Drawing Reference | D/Meth/050/Truck Drilling/CH/01 Rev. 1 |

### 2.2 Machine Layout & Main Components

Main Subsystems:

- Loading & Feeding System – V-feeder with servo control

- Drilling Head Assembly – Multi-spindle rotating head (1460 RPM)

- Hydraulic System – Clamping and servo control circuits

- Pneumatic System – Component handling and poka-yoke actuation

- Control System – PLC, HMI, Safety interlocks, Door interlocks

- Dust Collection System – Integrated chip removal

- Safety System – Emergency stop, safety doors, interlock switches

### 2.3 Typical Product Specifications

The machine processes clutch facings with the following dimensional range:

| Parameter | Minimum | Maximum |
| --- | --- | --- |
| Clutch Facing Outside Diameter | 280 mm | 430 mm |
| Clutch Facing Thickness | 2.8 mm | 6.2 mm |
| Number of Holes | 24 Nos. | 54 Nos. |
| Hole Diameter | 7.75 mm | 14.5 mm |
| Rivet Hole Thickness | 0.9 mm | 2.1 mm |
| Hole Position Runout (Tolerance) | ±0.1 mm | ±0.1 mm |
| Concentricity (Maximum Offset) | — | 0.8 mm |

## 3. TECHNICAL SPECIFICATIONS & DATA

### 3.1 Dimensional Data

| Parameter | Specification | Unit |
| --- | --- | --- |
| Machine Length | 2.315 | Meters |
| Machine Width | 2.103 | Meters |
| Machine Height | 3.048 | Meters |
| Total Machine Weight | 1.0 | Ton (1000 kg) |

### 3.2 Power & Energy Consumption

| Parameter | Specification | Remarks |
| --- | --- | --- |
| Total Installed Power | 10 | kW |
| Motor Rating (Spindle Drive) | 22 | kW @ 1460 RPM |
| Servo Motor (Feed - Festo) | 0.4 | kW |
| Servo Motor (Drill Head - Mitsubishi) | 3.5 | kW |
| Hydraulic Pump Motor | 2 | HP (1.5 kW) |
| Pneumatic Compressor Requirement | 5–6 | Bar (minimum) |

### 3.3 Hydraulic System Specifications

| Circuit | Max. Pressure | Max. Flow | Application |
| --- | --- | --- | --- |
| Main (Spindle Clamping) | 105 BAR | 8 LPM | Workpiece clamping & servo control |
| Cooling & Lubrication | 10 BAR | 30 LPM | Tool cooling & spindle lubrication |

Hydraulic Fluid Specification:

- Type: ISO VG 46 Anti-wear Hydraulic Fluid

### 3.4 Pneumatic System Specifications

| Parameter | Specification |
| --- | --- |
| Operating Pressure | 5–6 Bar |
| Supply Source | Plant Compressed Air System |
| Filtration | 5-micron filter (minimum) |
| Desiccant Dryer | Required for humid climates |
| Solenoid Valve Rating | 24 V DC |
| Actuators | Festo pneumatic servo drives |

### 3.5 Electrical System Specifications

| Parameter | Specification |
| --- | --- |
| Mains Supply | 3-Phase, 415V AC, 50 Hz, 16A (minimum) |
| Control Voltage | 24 V DC (UPS-backed for safety functions) |
| PLC Controller | Mitsubishi FX5U Series |
| HMI | ProFace 10" touchscreen panel |
| Emergency Stop | 24 V DC solenoid cut-off circuit |
| Control Panel IP Rating | IP65 (dust-proof) |
| Safety Certification | Decra/Omron/Pilz certified |

### 3.6 Environmental Operating Conditions

| Parameter | Specification | Remarks |
| --- | --- | --- |
| Ambient Temperature Range | +5°C to +40°C | Optimal operation |
| Relative Humidity | 20% to 85% | Non-condensing |
| Noise Level | <75 dB | At full operation |

### 3.7 Performance Metrics & Acceptance Criteria

| Function | Specification | Unit | Tolerance |
| --- | --- | --- | --- |
| Spindle Rotation Speed | 1460 | RPM | Minimum |
| Production Capacity | 600 | Parts/Hour | Minimum |
| MTBF (Mean Time Between Failures) | 8 | Hours | Minimum |
| TRP (Total Run Percentage) | 85 | % | Minimum |
| MTTR (Mean Time To Repair) | 1 | Hour | Maximum |
| Changeover Time (Tooling) | 30 | Minutes | Maximum |
| Dimensional Accuracy (Thickness Under Rivet Head) | ±0.05 | mm | Cp/Cpk >1.6 |
| Concentricity (PCD Eccentricity) | — |  | Cmk = 2.0 minimum |

## 4. SAFETY INFORMATION & WARNINGS

### 4.1 General Safety Precautions

⚠️ WARNING: This is a high-speed, precision drilling machine with rotating cutting tools and moving parts. Failure to follow safety procedures may result in serious injury or death.

Before Operating this Machine:

- Read this entire manual and understand all warnings and instructions

- Receive proper training from an authorized trainer

- Always wear appropriate Personal Protective Equipment (PPE)

- Never operate this machine while fatigued or under the influence of alcohol/drugs

- Keep hands, long hair, loose clothing, and jewelry away from moving parts

- Never reach into the machine during operation

- Always use provided jigs, fixtures, and clamping devices as specified

### 4.2 Personal Protective Equipment (PPE)

Mandatory PPE:

| PPE Item | Specification | Reason |
| --- | --- | --- |
| Safety Glasses | Anti-scratch, polycarbonate lens | Chip/dust protection |
| Ear Protection | Noise level <75 dB; use when extended exposure | Hearing protection |
| Hand Gloves | Cotton or leather (remove near rotating spindle) | Oil and chip handling |
| Safety Shoes | Steel-toed, slip-resistant | Foot protection from dropped parts |
| Work Apron | Durable synthetic material | Protection from splash |
| Hair Covering | For operators with long hair | Prevent entanglement |

### 4.3 Safety Features of the Machine

The machine is equipped with the following safety systems:

- Safety Interlocked Doors – Machine cannot run if safety doors are open

- Emergency Stop (E-Stop) Button – Red mushroom button at operator station; stops all motion immediately

- Automatic Machine Stop – Triggers on detection of any failure or abnormal condition

- Light Signal Indicator – Visual alarm display on HMI and operator station

- Poka-Yoke Sensors – Prevent component overlap, detect missing holes, verify concentricity

- Dust Collection System – Integrated chip removal

- Tool Clamping Safety – Automatic tool clamping/declamping with servo verification

- Drill Head Locking Pin – Manual safety pin to lock drill head before maintenance

### 4.4 Lock-Out & Tag-Out (LOTO) Procedures

ENERGY ISOLATION POINTS:

| Energy Type | Isolation Method | Location | Lockable |
| --- | --- | --- | --- |
| Electrical | Main isolator switch | Control panel (left side) | Yes |
| Hydraulic (Main) | Ball valve on pump discharge | Hydraulic manifold | Yes |
| Hydraulic (Lubrication) | Ball valve on lubrication circuit | Lubrication manifold | Yes |
| Pneumatic | Solenoid dump valve | Pneumatic control block | No |
| Mechanical | Drill head locking pin | Above drill head assembly | Yes |

LOTO Procedure Before Maintenance:

- Press EMERGENCY STOP button – Red button at operator station

- Switch OFF Main Electrical Isolator – Located on control panel left side; turn clockwise to OFF position

- Insert padlock through isolator switch lever – To prevent accidental restart

- Isolate Hydraulic Pressure – Close main pump discharge valve on hydraulic manifold

- Isolate Pneumatic Supply – Switch HMI "Pneumatic Dump" to ON; listen for pressure release hiss

- Insert Drill Head Locking Pin – Locate pin slot above drill head; insert safety pin; secure with lock pin

- Verify Zero Energy State – Use pressure gauge to confirm hydraulic pressure drops to zero

- Place LOTO Tag – Attach personal LOTO tag to isolator and locking pin with your name and timestamp

LOTO Procedure After Maintenance:

- Verify all tools removed from machine

- Verify all access panels closed and secured

- Remove all LOTO tags and locks

- Switch ON main electrical isolator

- Slowly open hydraulic pump discharge valve

- Switch HMI "Pneumatic Dump" to OFF

- Remove drill head locking pin

- Press RESET on HMI to clear any stored alarms

- Perform pre-start checks (Section 6.1) before restarting

### 4.5 Prohibited Operations

🛑 NEVER DO THE FOLLOWING:

- Operate the machine without authorized training and certification

- Remove or disable any safety interlocks, guards, or door switches

- Reach into the machine during operation

- Run the machine with safety doors open

- Attempt to clear chips or jams while machine is running

- Lubricate or adjust tools during spindle rotation

- Modify machine parameters without supervisor approval

- Use the machine for purposes other than specified drilling operations

- Override poka-yoke error signals to force operation

- Load or unload components at unsafe speeds or positions

## 5. TRANSPORT, INSTALLATION & COMMISSIONING

### 5.1 Transport & Handling

#### 5.1.1 Lifting & Rigging

- Lifting Method: Four eye bolts (M16) installed on machine top frame for lifting

- Rigging Equipment: 2–4 ton capacity slings rated for industrial machinery

- Lifting Speed: Slow and controlled; maximum 0.5 m/s

- Personnel: Minimum 2 certified riggers required

#### 5.1.2 Transportation Safety

- Machine must be secured to transport vehicle using resilient lashings and spacers

- Cover machine with protective tarpaulin to prevent weather damage

- Avoid sudden acceleration, braking, and sharp turns during transport

- Machine should be transported in horizontal position (NO tilting)

- Remove any loose components, tool holders, or jigs before transport

- Inspect for damage upon delivery before unloading

### 5.2 Installation & Site Preparation

#### 5.2.1 Installation Requirements

| Requirement | Specification |
| --- | --- |
| Floor Loading Capacity | Minimum 1.5 ton/m² (concentrated) |
| Floor Evenness | ±5 mm tolerance over 3-meter span |
| Space Clearance | 1 meter on all sides for maintenance access |
| Environmental Conditions | +5°C to +40°C, <85% RH (non-condensing) |

#### 5.2.2 Electrical Installation

- Supply Connection: By certified electrician only; 3-Phase 415V AC, 50 Hz

- Earth Connection: Machine frame must be grounded to plant earth (<5 ohm resistance)

- Control Panel Location: Dry, accessible location; minimum 1 meter from machine for operator reach

- Emergency Stop Button: Positioned at operator station; red color, easily accessible

- UPS Backup: 24 V DC emergency stop circuit should be backed by UPS for minimum 10-second runtime

#### 5.2.3 Hydraulic System Installation

- Install main hydraulic power pack adjacent to machine (within 2 meters)

- All hose runs should have drip trays underneath

- Filtration: 10-micron system with differential pressure indicator

#### 5.2.4 Pneumatic System Installation

- Connect plant compressed air to machine input via 5-micron filter

- Supply pressure: 5–6 bar (regulated at source)

- Install pressure gauge on supply line for monitoring

- Desiccant dryer recommended for humid climates (>70% RH)

### 5.3 Commissioning Procedure

#### 5.3.1 Pre-Commissioning Checklist

- [ ] Machine physically installed and secured

- [ ] All electrical connections verified by certified electrician

- [ ] Hydraulic system filled with specified fluid; no leaks detected

- [ ] Pneumatic system pressurized and tested; all joints tight

- [ ] Control panel powered up; HMI display responsive

- [ ] All poka-yoke sensors functioning (verified with test blocks)

- [ ] Drill head rotates freely at zero pressure; no mechanical binding

- [ ] Safety doors close properly; interlock switches respond

- [ ] Emergency stop button tested; stops all motion immediately

- [ ] Dust collector system running; positive suction at spindle

#### 5.3.2 Commissioning Tests

- No-Load Spindle Rotation Test – Run spindle at 1460 RPM for 10 minutes; monitor vibration and noise

- Servo Movement Test – Jog feed servo forward/backward and drill head up/down; verify smooth motion

- Pressure Test – Monitor hydraulic pressure during clamping cycle; verify within specification

- Cycle Test (Empty) – Run 5 complete cycles without component to verify sequence and timing

- Cycle Test (Trial Components) – Run 20 cycles with trial clutch facing; measure hole accuracy and verify poka-yoke checks

- Production Run Test – Run minimum 100 parts at rated speed (600 p/h); verify MTBF and TRP targets

## 6. OPERATING PROCEDURES

### 6.1 PRE-START VERIFICATION CHECKLIST

Timing: Complete checklist before every shift start or after machine shutdown >30 minutes

#### Step 1: Verify Main Electrical Isolator is ON

- Location: Control panel, left side, red/black switch lever

- Status: Switch lever should be pointing UP (ON position)

- Action: If OFF, switch to ON position; proceed only after HMI powers up

#### Step 2: Check Electrical Earth (Ground)

- Location: Main isolator panel, earth connection bolt (marked with green/yellow label)

- Status: Verify bolt is tight and shows no corrosion or white powder deposit

#### Step 3: Verify HMI Ready & Responsive

- Location: ProFace 10" touchscreen on control panel, right side

- Action:

  - Observe startup sequence: Mitsubishi FX5U logo → Main menu screen

  - Verify all indicator lights illuminated: Power (Green), Ready (Blue), Alert Status (Yellow/Red if abnormal)

  - Touch main screen to confirm responsiveness; should show menu options

  - If screen frozen or no response, perform soft reset: Hold Reset button on HMI for 3 seconds

#### Step 4: Check Component Buffer Level (Loading Area)

- Location: Loading tray assembly (top-left of machine)

- Status: Minimum 100–150 clutch facings should be stacked in loading area

- Poka-Yoke Sensor: Located at bottom of loading stack; should be covered (facings above minimum) for green light indicator

- Action:

  - If buffer below minimum: Load 100–150 new facings manually into tray

  - If green light indicator remains red even with full buffer: Check sensor lens for dust; clean with soft cloth if needed

#### Step 5: Verify Component Loading Area is Clear & Ready

- Location: V-feeder component entry point (center-left of machine)

- Status: Area should be empty; no components jammed or stuck in feeder

- Action:

  - Visually inspect feeder path from loading tray to drill jig; no debris or broken component pieces

  - Push feeder manually by hand (machine OFF) to verify smooth movement

#### Step 6: Inspect Component Overlap Area (Drill Jig Entry)

- Location: Drill jig seating area (below spindle head)

- Status: Jig should be empty; no overlapping or stacked components visible

- Action:

  - Visually confirm jig is empty

  - Place a single test facing in jig; machine should detect single component (green light)

  - Place a second test facing on top (overlay condition); machine should detect overlap (red light and audible alarm)

  - Remove test facing; jig should return to green light

#### Step 7: Check Output/Ejection Area

- Location: Opposite side of drill jig (right side of machine)

- Status: Output collection area should be empty and ready to receive drilled components

- Action:

  - Clear any spillage or residual components from output area

  - Position collection bin at output chute (center alignment)

  - Verify bin is empty and clean

#### Step 8: Verify Clamping Position & Poka-Yoke Sensor

- Location: Drill jig clamping assembly (below spindle)

- Status: Clamp should be in fully open position (relaxed state) at machine idle

- Action:

  - Visually confirm clamp arms are fully open (180° position)

  - Green indicator light should be visible on HMI clamping status

#### Step 9: Verify Top Drill Head Locking Pin Status

- Location: Above multi-spindle drilling head, center-top

- Status: Locking pin should be INSTALLED & LOCKED during pre-start (safety requirement)

- Action:

  - Visually locate locking pin above drill head; should be inserted in locking slot (horizontal position)

  - Verify pin is fully seated; no gaps visible between pin and slot

  - Lock pin should have a padlock or keyed lock in place

#### Step 10: Check Drill Head Oil Line – Input & Output Hoses

- Location: Top-rear of drill head assembly, two hoses: Pressure (Red) & Return (Blue)

- Status: Both hoses should be tightly connected; no visible leaks, cracks, or bulging

- Action:

  - Gently tug both hoses to confirm they are firmly connected

  - Wipe both connection points with clean cloth; inspect for oil seepage

  - Monitor hose temperature (touch gently with palm) – should be warm but not burning hot (target: <50°C)

#### Step 11: Verify Dust Collector System is Running

- Location: Dust collection unit (adjacent to main machine body, typically right side)

- Status: Motor should be running and producing suction at spindle drill area

- Action:

  - Switch ON dust collector motor (if not already running)

  - Verify suction by placing hand near spindle dust outlet (machine OFF) – should feel strong air pull inward

  - Check collection bag for fullness: If >75% full, empty bag before shift to maintain suction efficiency

#### Step 12: Final Visual Inspection

- Do a complete walk-around of the machine (360° view)

- Verify all safety doors are closed and interlocked

- Confirm no loose tools, parts, or materials on machine bed or underneath

- Check floor around machine: dry, no spills, no tripping hazards

- Verify operator station is organized; emergency stop button clearly visible and accessible

### 6.2 NORMAL OPERATING CYCLE (Machine Operation Sequence)

#### 6.2.1 Cycle Initiation

Pre-Requisite: All pre-start checks (Section 6.1) completed with green status

Operator Actions:

- Ensure drill head locking pin is INSTALLED and LOCKED (safety requirement)

- Load components manually into loading tray (100–150 pieces)

- Proceed to HMI; main menu should be visible

- Touch "START PRODUCTION" button on HMI

- System will verify:

  - Buffer sensor status: Green (minimum components present)

  - Clamp position: Open (jig ready)

  - Safety doors: All closed and interlocked

- If all checks pass: Remove drill head locking pin and proceed to cycle start

#### 6.2.2 NORMAL PRODUCTION CYCLE – STEP-BY-STEP FLOW

##### PHASE 1: COMPONENT LOADING (Duration: ~2–3 seconds)

Step 1.1 – Component Loading Area → Component Feeder

- Actor: Pneumatic solenoid valve + Festo servo drive (Component Feeder)

- Action:

  - Solenoid energizes; pneumatic pressure pushes component from loading tray into V-feeder

  - V-feeder servo motor activates; component pushed into feeder channel

  - Component should align with groove poka-yoke sensor (groove detection)

- Poka-Yoke Check:

  - Groove poka-yoke sensor checks if component orientation is correct (groove present on top)

  - Status: GREEN = groove detected, component correctly oriented → proceed

  - Status: RED = no groove detected, component inverted or defective → machine stops; operator removes and reloads

##### PHASE 2: COMPONENT FEEDING – SERVO FORWARD MOVEMENT (Duration: ~4–5 seconds)

Step 2.1 – Component Feeder Servo Move Forward

- Actor: Festo servo drive (Component Feeder)

- Action:

  - Servo motor receives signal from PLC controller

  - Pushes component forward smoothly toward drill jig center

  - Motion should be controlled and slow (~50–100 mm/sec) to allow settling

##### PHASE 3: COMPONENT CENTERING & OVERLAP CHECK (Duration: ~1–2 seconds)

Step 3.1 – Overlap Detection Poka-Yoke Check

- Sensor Location: Above drill jig (height sensor / proximity sensor)

- Logic:

  - GREEN = single component detected → proceed to next step

  - RED = multiple components detected (overlap condition) → machine stops; alarm sounds

Step 3.2 – Centering Unit Accuracy Check

- Sensor Location: Centering poka-yoke sensor (points toward component edge)

- Action: Sensor verifies component is centered in jig (radial positioning)

- Tolerance: ±2 mm from jig center

- Status:

  - GREEN = component centered correctly → proceed

  - YELLOW = component slightly off-center (within tolerance) → proceed but flag for review

  - RED = component significantly off-center (>2 mm) → machine stops

##### PHASE 4: DRILL HEAD POSITIONING – SERVO MOVEMENT (Duration: ~2–3 seconds)

Step 4.1 – Drill Head Servo Move (Downward Positioning)

- Actor: Mitsubishi servo motor + hydraulic servo valve (Drill Head Movement)

- Action:

  - Servo receives signal from PLC

  - Hydraulic actuator lowers drill head smoothly from retracted position (top) toward component

  - Servo control provides smooth, controlled descent at ~100–150 mm/sec

Step 4.2 – Drill Head Contact & Clamping Sequence

- Action:

  - Drill head reaches proximity to component (10–15 mm clearance)

  - Pressure sensor on spindle detects light touch

  - Solenoid valve energizes; hydraulic pressure applies to clamping arms

  - Clamping arms close around component edges, securing it firmly in jig

- Clamping Pressure: Monitored by pressure sensor; should reach 80–105 BAR within 1 second

##### PHASE 5: DRILLING CYCLE (Duration: ~4–6 seconds)

Step 5.1 – Spindle Rotation & Drilling

- Actor: Main spindle motor (1460 RPM) + Multi-spindle gang drill head (48 spindles)

- Action:

  - Spindle motor (22 kW) energizes to rated speed: 1460 RPM

  - All 48 drill bits simultaneously contact component surface and begin cutting

  - Chips flow downward into dust collection system

  - Servo control monitors spindle load via pressure sensors

- Drilling Process:

  - Hole size: 7.75–14.5 mm depending on part configuration

  - Material: Composite (organic/inorganic fibers + thermoset resin)

  - Feed rate: Automatic servo control (adjusts based on cutting resistance)

- Monitoring:

  - Spindle current sensor: Should show 60–80% of rated motor current

  - Drill head pressure: Should remain in 80–105 BAR range (servo-controlled)

  - Spindle vibration: Should be minimal (<0.5 mm peak-to-peak)

##### PHASE 6: POST-DRILLING POKA-YOKE CHECKS (Duration: ~2–3 seconds)

Step 6.1 – Hole Presence Detection

- Sensor Type: Optical/Proximity sensors positioned around component edge

- Logic: Sensor array checks if expected number of holes are present

- Tolerance: ±2 holes acceptable

- Status:

  - GREEN = all holes detected → proceed

  - RED = holes missing → machine stops; operator removes component

Step 6.2 – Concentricity & Hole Position Check

- Sensor Type: Precision concentricity gauging sensor (optical or pneumatic)

- Logic: Measures runout/eccentricity of drilled hole pattern relative to component center

- Tolerance: Maximum 0.8 mm offset

- Status:

  - GREEN = runout <0.8 mm → proceed

  - YELLOW = runout 0.6–0.8 mm → proceed but flag for SPC (Statistical Process Control)

  - RED = runout >0.8 mm → machine stops; component rejected

Step 6.3 – Rivet Hole Thickness Verification

- Sensor Type: Thickness measurement probe (air jet or mechanical)

- Logic: Measures thickness of material surrounding rivet holes

- Tolerance: ±0.05 mm (Critical specification per Valeo)

- Status:

  - GREEN = thickness within tolerance → proceed to ejection

  - RED = thickness out of tolerance → machine stops; component flagged

##### PHASE 7: CLAMP RELEASE & COMPONENT FEEDER SERVO BACKWARD (Duration: ~3–4 seconds)

Step 7.1 – Clamping System Release

- Actor: Hydraulic solenoid valve (clamp release)

- Action:

  - Solenoid de-energizes; hydraulic pressure to clamping arms vents to tank

  - Clamp arms spring-open smoothly, releasing component

Step 7.2 – Component Feeder Servo Move Backward

- Actor: Festo servo drive (Component Feeder reverse)

- Action:

  - Servo receives reverse signal from PLC

  - Feeder servo pulls drilled component backward (away from jig) at controlled speed ~50–100 mm/sec

  - Component slides out of drill jig into ejection chute

##### PHASE 8: DRILL HEAD RETRACTION (Duration: ~2–3 seconds)

Step 8.1 – Drill Head Servo Move (Upward Retraction)

- Actor: Mitsubishi servo motor + hydraulic servo valve (Drill Head Movement)

- Action:

  - Servo receives upward command from PLC

  - Hydraulic actuator retracts drill head upward (away from jig) at ~150 mm/sec

  - Drill head should return to fully retracted position (120–150 mm above jig)

##### PHASE 10: COMPONENT EJECTION & OUTPUT CHECK (Duration: ~1–2 seconds)

Step 10.1 – Component Ejection from Output Chute

- Actor: Pneumatic ejector pin or gravity-fed chute

- Action:

  - Solenoid-controlled ejector pin (or air jet) activates automatically after feeder retracts

  - Component pushed out of jig and slides down exit chute into collection bin below

Step 10.2 – Output/Ejection Verification Poka-Yoke Sensor

- Sensor Location: At output chute exit (near collection bin)

- Sensor Type: Presence detector (optical or proximity)

- Logic: Confirms drilled component has successfully exited machine

- Status:

  - GREEN = component detected at output → cycle complete

  - RED = component stuck in chute or missing → machine stops

#### 6.2.3 CYCLE COMPLETION & REPEAT

- Total Cycle Time: ~20–25 seconds per component

- Throughput: 600 components/hour (at full speed)

- Automatic Repeat: Upon successful output verification, machine immediately triggers next component loading

- Cycle continues until:

  - Buffer level drops below minimum (sensor red); machine stops; operator loads more components

  - Emergency stop button pressed; all motion halts immediately

  - Any poka-yoke check fails; machine stops; alarm displayed on HMI

  - Operator presses "STOP PRODUCTION" button on HMI; current cycle completes, then machine stops

### 6.3 MACHINE STOP PROCEDURES

#### 6.3.1 Normal Stop (End of Shift / Controlled Shutdown)

- Press "STOP PRODUCTION" button on HMI

- Machine completes current component cycle (does not stop mid-cycle)

- Upon cycle completion:

  - Clamp releases

  - Drill head retracts to fully up position

  - Feeder servo returns to neutral

  - Spindle motor de-energizes and coasts to stop (~30–60 seconds)

  - All hydraulic pressures vent safely to tank

  - Pneumatic dump solenoid releases air pressure

- Once machine is completely still (spindle fully stopped):

  - Switch dust collector OFF (if shift ending)

  - Switch main electrical isolator to OFF position

  - Perform end-of-shift checks (Section 6.4)

#### 6.3.2 Emergency Stop (Safety Situation)

When to Use: Immediate danger, safety hazard, or uncontrolled machine movement

- Press EMERGENCY STOP button immediately – Red mushroom button at operator station

- Effect: All motion stops instantly; spindle cuts off; hydraulics vent; pneumatics dump

- Recovery Steps:

  - Assess situation; identify hazard

  - If hazard eliminated: Turn emergency stop knob clockwise to reset

  - Perform pre-start checks (Section 6.1) before resuming operation

### 6.4 END-OF-SHIFT CHECKS & SHUTDOWN

- Empty Loading Tray – Remove remaining components to storage

- Clean Component Areas – Wipe jig and feeder with clean cloth to remove debris

- Check for Leaks – Visually inspect all hose connections for oil seepage

- Monitor Dust Collector – Empty collection bag if >50% full

- Note any Issues – Record in machine logbook:

  - Any poka-yoke errors encountered during shift

  - Production count and rejection count

  - Any unusual sounds, vibrations, or temperature

  - Maintenance action items for next shift

- Switch OFF Control Systems:

  - Dust collector OFF

  - HMI STOP PRODUCTION button (if still running)

  - Main electrical isolator OFF

- Secure Machine:

  - All safety doors closed and locked (if applicable)

  - Drill head locking pin inserted and locked (safety requirement)

  - No loose components or tools left on machine bed

## MAINTENANCE & LUBRICATION

### 7.1 Maintenance Schedule

#### 7.1.1 Daily Maintenance (Before Each Shift)

| Task | Procedure |
| --- | --- |
| Visual Inspection | Check for oil leaks, loose fasteners, damage |
| Hose Inspection | Inspect all hydraulic and pneumatic hoses for cracks, bulging |
| Dust Collector | Empty collection bag if needed; check filter for blockage |
| Component Tray | Clean loading area; remove any broken component pieces |
| Drain Plug Check | Verify hydraulic tank drain plug is tight |

#### 7.1.2 Weekly Maintenance (Every 40 Operating Hours)

| Task | Procedure |
| --- | --- |
| Hydraulic Fluid Check | Check fluid level in tank; top up if below minimum mark |
| Pneumatic Filter Cleaning | Clean 5-micron filter element; replace if clogged |
| Spindle Bearing Inspection | Listen for grinding noise; monitor temperature (should be <50°C) |
| Servo Motor Check | Verify smooth jog operation of feed servo and drill head servo |
| Sensor Cleaning | Wipe sensor lenses with clean, soft cloth; remove dust |

#### 7.1.3 Monthly Maintenance (Every 160 Operating Hours)

| Task | Procedure |
| --- | --- |
| Hydraulic Filter Change | Replace main hydraulic filter element |
| Lubrication System Check | Check spindle lubrication system output |
| Motor Current Draw | Measure spindle motor current under load; compare to baseline |
| Pressure Gauge Calibration | Verify hydraulic and pneumatic gauges are accurate |
| Safety Door Testing | Test all safety door interlocks; verify machine stops when opened |

#### 7.1.4 Quarterly Maintenance (Every 640 Operating Hours / ~3 months)

| Task | Procedure |
| --- | --- |
| Gearbox Inspection | Check gearbox mounting bolts; listen for gear noise |
| Servo Calibration | Perform servo zero-point calibration per Mitsubishi manual |
| Poka-Yoke Sensor Calibration | Recalibrate all sensor thresholds per OEM specification |
| Spindle Alignment Check | Verify spindle runout with dial indicator |
| Electrical Connections | Inspect all terminal blocks and connectors for corrosion |

#### 7.1.5 Annual Maintenance (Yearly / ~2500 Operating Hours)

| Task | Procedure |
| --- | --- |
| Complete Hydraulic System Flush | Drain and replace all hydraulic fluid |
| Spindle Bearing Service | Check for wear; replace if damaged |
| PLC Battery Replacement | Replace RTC (Real-Time Clock) battery in FX5U controller (CR2032) |
| Safety Certification Renewal | Perform third-party machine safety inspection |

### 7.2 Lubrication Specification & Schedule

#### 7.2.1 Hydraulic Fluid Specification

| Parameter | Specification |
| --- | --- |
| Fluid Type | ISO VG 46 Anti-wear Hydraulic Fluid |
| Tank Capacity | Check manual specifications |
| Change Interval | Every 12 months or 2500 operating hours |
| Disposal | Collect used fluid in sealed container; send to certified recycler (NOT in drain) |

#### 7.2.2 Spindle Lubrication System

| Parameter | Specification |
| --- | --- |
| Supply Pressure | 10 BAR (lubrication circuit) |
| Flow Rate | 30 LPM (for spindle bearing lubrication) |
| Refill Interval | When sight glass shows low level |

#### 7.2.3 Pneumatic System Lubrication

| Component | Lubrication Method | Interval |
| --- | --- | --- |
| Solenoid Valves | Integrated FRL (Filter-Regulator-Lubricator) on supply line | Every 500 hours |
| Cylinder Rods | Pre-lubricated at manufacture | Check monthly |
| Servo Drive Bearings | Self-contained | Monitor for noise |

### 7.3 Spare Parts & Consumables List

#### 7.3.1 Critical Spare Parts (Stock Minimum: 2 Units Each)

| Part Name | Part Number | Function | Qty | Lead Time |
| --- | --- | --- | --- | --- |
| Hydraulic Filter Element |  | Removes contamination from hydraulic fluid | 2 | 2–3 days |
| Pneumatic Filter Element |  | Removes water and oil mist from compressed air | 2 | 2–3 days |
| Spindle Bearing Assembly |  | Supports rotating drill head at 1460 RPM | 1 | 5–7 days |
| Servo Motor (Festo Feed) |  | Powers component feeder movement | 1 | 7–10 days |
| Servo Motor (Mitsubishi Drill Head) |  | Powers drill head up/down movement | 1 | 7–10 days |
| Proximity Sensor (Poka-Yoke) |  | Detects component position, overlap, concentricity | 5 | 3–5 days |
| Solenoid Valve (Clamping) |  | Controls clamp open/close | 2 | 2–3 days |
| Solenoid Valve (Feed) |  | Controls component feeder direction | 2 | 2–3 days |
| Hydraulic Hose Assembly (Pressure) |  | Main pressure line to drill head (105 BAR) | 1 | 3–5 days |
| Hydraulic Hose Assembly (Return) |  | Return line from drill head to tank (10 BAR) | 1 | 3–5 days |
| Pressure Relief Valve |  | Protects system from overpressure | 1 | 3–4 days |
| Spindle Drive Belt |  | Transfers motor power to spindle shaft | 2 | 2–3 days |
| Coupling (Motor to Gearbox) |  | Connects motor to gearbox | 1 | 5–7 days |

#### 7.3.2 Consumable Items (Reorder as Needed)

| Item | Specification | Consumption Rate | Reorder Point |
| --- | --- | --- | --- |
| Hydraulic Fluid | ISO VG 46 | 5–10 liters/month | 20 liters stock |
| Pneumatic Filter Elements | 5-micron replacement cartridge | 1 per 500 hours | Always 2 in stock |
| Drill Bits | Carbide multi-point gang set (48 bits) | As needed | Keep 2 sets in stock |
| Safety Door Gasket | Silicone rubber seal, 3mm thickness | 1–2 per year | 2 in stock |
| Interlock Switch | Magnetic proximity type, 24V DC | 1 per 2–3 years | 1 in stock |

#### 7.3.3 Optional Spare Parts (1 Unit Each)

| Part | Purpose | Lead Time |
| --- | --- | --- |
| Complete Control Panel Module | Full HMI + PLC replacement (emergency backup) | 10–14 days |
| Spindle Assembly (Complete) | Replacement spindle head if bearing fails | 14–21 days |
| Hydraulic Pump Unit | Main pump replacement | 10–14 days |

## TROUBLESHOOTING & ALARM CODES

### 8.1 Common Issues & Solutions

#### Issue 1: Machine Does Not Start – Green Light Does Not Appear on HMI

Possible Causes & Resolution:

Main electrical isolator switch is OFF

- Check: Verify isolator switch lever points UP (ON position)

- Action: Switch to ON; observe HMI startup sequence

- Expected: Mitsubishi logo appears on screen within 5 seconds

Electrical power supply is not available

- Check: Verify 3-Phase 415V supply is present at plant distribution board

- Action: Use multimeter to check voltage at control panel terminal (should read ~415V)

- Expected: Voltage between 380–440V is acceptable

Main isolator switch is faulty (stuck)

- Check: Turn switch slowly; should hear distinct "click" when switching positions

- Action: Contact maintenance for replacement

HMI display is powered but not showing startup screen (black screen)

- Check: Is HMI backlight illuminated? (look at screen in dim light)

- Action: Press and hold RESET button on HMI for 3 seconds (soft restart)

- Expected: Mitsubishi logo and main menu should appear within 10 seconds

- If still blank: Perform cold restart – switch isolator OFF, wait 30 seconds, switch ON again

UPS (Emergency Stop backup power) has failed

- Check: If main power lost, does machine still show HMI display for 10 seconds?

- Action: Check UPS battery indicator light on control panel (should be green)

- Expected: Green light = battery OK; Red light = battery drained or faulty

Resolution Steps (In Order):

- Check electrical isolator is ON; observe HMI green light

- If no light, perform cold restart (OFF 30 sec, then ON)

- If still no response, use multimeter to verify 3-phase supply at panel

- If supply OK but no HMI: Contact maintenance for control panel troubleshooting

#### Issue 2: Buffer Sensor Showing Red Light – Machine Stops & Will Not Start Cycle

Possible Causes & Resolution:

Minimum buffer level not met

- Check: Loading tray should contain minimum 100–150 clutch facings

- Action: Manually load more components into loading tray until red light changes to green

- Expected: Green light after ~50 components added

Buffer sensor lens is dirty/dusty

- Check: Locate buffer sensor (bottom of loading stack); examine lens for dust deposit

- Action: Gently wipe sensor lens with clean, soft cloth (microfiber preferred)

- Expected: Red light changes to green; sensor responds to component presence

Buffer sensor is misaligned

- Check: Is sensor pointing at component stack, or has it been bumped?

- Action: Visually inspect sensor mounting bracket; should be perpendicular to stack direction

- Expected: Sensor should detect light reflection from top component

Sensor is malfunctioning (electronic failure)

- Check: Sensor indicator light on sensor body should blink when hand passed over lens

- Action: If no blink or constant ON/OFF: sensor requires replacement

- Expected: Blinking indicator = sensor working; steady light = possible failure

Resolution Steps (In Order):

- Load additional components until buffer light shows green

- If light remains red: Clean sensor lens and retry

- If still red: Check sensor mounting alignment

- If issue persists: Contact maintenance; sensor likely requires replacement

#### Issue 3: Clamp Will Not Close – Red Light on Clamp Status, Machine Stops

Possible Causes & Resolution:

Component is stuck or misaligned in jig

- Check: Visually inspect component position in jig; should be centered

- Action: Stop machine; manually remove component; reload carefully

- Expected: Clamp should close smoothly when component is properly seated

Clamp mechanical parts are dirty or stuck

- Check: Move clamp arms manually by hand (machine OFF); should move freely

- Action: Wipe clamp arms and contact surfaces with clean cloth; remove debris

- Expected: Clamp arms should move 180° arc freely without sticking

Hydraulic pressure is not building (clamp solenoid not energizing)

- Check: Observe hydraulic pressure gauge on manifold during clamping attempt

- Action: Pressure should rise from 0 to 80+ BAR within 1 second of cycle start

- Expected: Gauge needle moving upward = solenoid working; needle stuck at zero = problem

- Next Step: Check solenoid electrical connection; verify 24V supply to solenoid

Hydraulic leak (reduced pressure, slow clamp closure)

- Check: Inspect all hose connections around clamp assembly for oil seepage

- Action: Wipe connections with cloth; if drops visible after 2 minutes, leak confirmed

- Expected: Dry connections; no visible oil drops

- Next Step: Tighten connection nuts (use appropriate wrench); if leak continues, replace hose

Proximity sensor detects false clamp-open position

- Check: Sensor indicator light should go from red (open) to green (closed) during clamp cycle

- Action: Manually move clamp to fully closed position; sensor light should go green

- Expected: Green light = clamp fully closed and locked

- If light stays red: Sensor may be misaligned or failed; contact maintenance

Resolution Steps (In Order):

- Stop machine; remove component; manually test clamp movement

- Clean clamp arms and contact surfaces

- Reload component carefully; attempt cycle again

- If clamp still won't close: Check hydraulic pressure gauge during clamp attempt

- If pressure low: Check for leaks; tighten hose connections

- If pressure OK but clamp stuck: Contact maintenance for mechanical inspection

#### Issue 4: Spindle Does Not Rotate – Drill Head Not Spinning at Start of Cycle

Possible Causes & Resolution:

Main spindle motor is not receiving 3-phase AC power

- Check: Listen for motor humming/buzzing sound; should hear audible hum at idle

- Action: Use multimeter to verify 3-phase voltage at motor terminals (should be ~415V AC between phases)

- Expected: Approximately 415V between any two phase terminals; 0V on ground-to-phase = fault

- Next Step: If no voltage, check main contactor on motor starter panel

Main contactor (motor starter) is stuck open

- Check: Locate main contactor relay on motor starter panel (usually large black box with 3 terminals)

- Action: Observe contactor coil indicator light; should illuminate when cycle starts

- Expected: Coil light ON = contactor energized; coil light OFF = contactor not energizing

- Next Step: If light OFF, check 24V DC supply to contactor coil

Spindle motor is mechanically jammed

- Check: Attempt to manually rotate spindle shaft by hand (machine OFF, E-stop pressed)

- Action: Grasp spindle shaft or pulley; try to turn gently (should rotate freely with minimal resistance)

- Expected: Spindle rotates freely; no binding sensation

- If stuck/grinding noise: Internal bearing failure; stop immediately, contact maintenance

Spindle thermal overload protection has tripped

- Check: Feel motor housing with palm – should be warm but not burning hot (<50°C)

- Action: If motor is very hot, allow 30–60 minutes cooldown with fan running

- Expected: Motor temperature drops back to <40°C; thermal reset occurs automatically

- Next Step: If motor keeps overheating on restart, check for bearing friction (grinding noise)

Variable Frequency Drive (VFD) or soft starter has fault

- Check: Observe VFD status lights on motor control panel (usually green = OK, red = fault)

- Action: If red fault light, note the error code displayed (if any); record for maintenance

- Expected: Green light = normal operation

- Next Step: Contact maintenance with error code; VFD may require reset or replacement

Resolution Steps (In Order):

- Listen for motor humming sound; if present, spindle driver is energized (power OK)

- If no hum: Check 3-phase voltage at motor using multimeter

- If voltage OK but no spin: Check motor manually; should rotate freely by hand

- If motor stuck: Stop immediately; contact maintenance

- If motor rotates freely but won't run: Check main contactor and motor starter panel

- If all above OK: Contact maintenance for VFD/soft starter diagnostics

#### Issue 5: Poka-Yoke Sensor Showing Repeated Red Errors – Components Rejected

Error Type A: Groove Poka-Yoke (Component Loading) – Red Light

- Meaning: Component loaded is inverted (no groove detected)

- Cause: Operator placed component upside-down in feeder

- Action:

  - Stop machine

  - Remove inverted component from feeder

  - Reload component with groove facing UP (toward feeder sensor)

- Verification: Test with one component; if groove detectable, green light shows

Error Type B: Overlap Poka-Yoke (Drill Jig Entry) – Red Light

- Meaning: Two or more components stacked in jig (height sensor detected overlap)

- Cause: Component not fully ejected from jig before next component entered

- Action:

  - Stop machine

  - Manually remove stacked components from jig

  - Check component feeder servo: should retract fully after each cycle

  - Verify servo air pressure is adequate (5–6 bar minimum)

  - Reload single component; attempt cycle

- Prevention: Ensure pneumatic pressure does NOT drop below 5 bar during operation

Error Type C: Concentricity Poka-Yoke (Hole Position Check) – Red Light

- Meaning: Drilled holes are eccentric (runout >0.8 mm) relative to component center

- Cause: Component centering failed; component was off-center in jig during drilling

- Action:

  - Inspect component loading: Component should sit flush in V-feeder groove

  - Check if V-feeder servo moved fully forward (should push component to jig center)

  - Verify servo position sensor feedback (PLC display should show servo at maximum extension)

  - Check jig centering pins: Should grip component edges without excessive play

  - Re-test with new component batch

- Maintenance Check: If issue persists, centering pins may be worn; contact maintenance

Error Type D: Hole Presence Poka-Yoke – Red Light

- Meaning: Expected holes are missing or blocked; optical sensor did not detect >2 holes

- Cause: One or more spindle bits did not drill; bits may be broken or dull

- Action:

  - Stop machine

  - Inspect drill bits for breakage (broken tips visible)

  - Check spindle speed: Should be exactly 1460 RPM (verify on HMI display)

  - If bits intact and speed OK: bits may be dull (worn); recommend replacement

  - Replace damaged bits with spares from maintenance kit

  - Run test cycle with new bits

Error Type E: Rivet Hole Thickness Poka-Yoke – Red Light

- Meaning: Material thickness under rivet area is out of tolerance (±0.05 mm)

- Cause: Component material defect (thinner than specification) OR drill bits cutting too much material

- Action:

  - Inspect failed component: Is it thinner than normal batch?

  - If all components in batch are thin: Reject entire batch; notify material supplier

  - If only isolated failures: Component may have internal voids or delamination

  - Check drill bit wear: Dull bits may cut oversized holes; replace bits

  - Re-test with new bit set

General Resolution Steps for Any Poka-Yoke Red Error:

- Stop machine immediately (press STOP PRODUCTION on HMI)

- Identify which poka-yoke check failed (HMI should display error code or description)

- Remove failed component and any subsequent stuck components

- Perform corrective action per error type above

- Clear error by pressing RESET on HMI (should go from red to green)

- Reload component batch; restart production

- If error repeats: Escalate to maintenance supervisor; do NOT force machine to run with repeated errors

### 8.2 HMI Alarm Codes & Messages

Refer to HMI Main Screen documentation for specific alarm codes and their meanings.

Common Alarm Display Locations on HMI:

- Main operation screen shows real-time status

- Alarm message section displays error codes and descriptions

- Alert lights indicate priority level (red = critical, yellow = warning)

### 8.3 When to Call Maintenance

| Situation | Action |
| --- | --- |
| Spindle makes grinding noise | Bearing failure likely – Call immediately |
| Hydraulic leak (visible oil drops) | System pressure compromised – Call immediately |
| Electrical shock felt when touching machine | Electrical hazard; E-stop & isolate – Call immediately |
| Machine vibration increases suddenly | Mechanical imbalance – Call immediately |
| Repeated poka-yoke errors not resolved | Sensor calibration required – Call maintenance |
| Emergency stop button not resetting | Safety circuit fault – Call immediately |
| Hose rupture with fluid spray | SHUT DOWN IMMEDIATELY – Call immediately |
| Control panel smoking or burning smell | EVACUATE – Call emergency services – Call immediately |

## 9. RECIPE PARAMETERS & CONFIGURATION

### 9.1 Machine Recipe/Program Settings

HMI Main Operation Screen displays the key operating parameters during production.

Description of HMI Main Menu Display (Reference Image):

The HMI screen shows four primary information sections:

- Upper Section – Servo Position Monitoring:

  - FEED SERVO POS: Displays current component feeder position in millimeters (0.000 mm = neutral/home position)

  - SPINDLE SERVO POS: Displays current drill head vertical position in millimeters (0.00 mm = fully retracted)

  - Both readings update in real-time during cycle operation

- Middle Section – Production Counters & Status:

  - CYCLE TIME: Shows elapsed time for current production cycle in seconds (13.5 sec example shown)

  - JOB COUNT: Displays total components produced in current batch (177 shown with RESET button)

  - SPC COUNT: Statistical Process Control counter for quality tracking (177 shown with SPC RESET button)

- Right Side – Door & System Status Indicators:

  - Individual indicator lights show status of:

    - REAR DOOR: Red (closed) or Green (open)

    - LH DOOR (Left-Hand): Red (closed) or Green (open)

    - RH DOOR (Right-Hand): Red (closed) or Green (open)

    - MAIN DOOR: Red (closed) or Green (open)

  - All doors must show RED (closed) for machine to operate

- Control Buttons Section:

  - SPINDLE SERVO RESET: Resets spindle position to zero

  - FEEDER SERVO RESET: Resets feeder position to zero

  - CYCLE STOP: Stops current production cycle

  - CONTINUE CYCLE: Resumes production after stoppage

Additional Operating Parameters (Programmable via Setup Menu):

| Parameter | Default Value | Range | Unit | Purpose |
| --- | --- | --- | --- | --- |
| Spindle Speed | 1460 | 1000–1460 | RPM | Controls drilling speed |
| Component Feed Rate (Forward) | 50 | 20–100 | mm/sec | Speed of component feeder servo moving forward |
| Component Feed Rate (Backward) | 50 | 20–100 | mm/sec | Speed of component feeder servo moving backward |
| Drill Head Descent Speed | 100 | 50–150 | mm/sec | Speed of drill head moving down toward component |
| Drill Head Ascent Speed | 150 | 50–150 | mm/sec | Speed of drill head retracting upward |
| Clamping Pressure (Target) | 90 | 70–105 | BAR | Hydraulic pressure for component clamping |
| Drilling Dwell Time | 4 | 2–8 | Seconds | Time spindle stays in contact with component drilling |
| Cycle Timeout | 60 | 30–120 | Seconds | Maximum time allowed for single cycle before abort |
| Buffer Minimum Level | 100 | 50–200 | Pieces | Minimum components required in loading tray |
| Pressure Relief Limit | 105 | 100–110 | BAR | Maximum hydraulic pressure before safety relief valve opens |

How to Change a Recipe Parameter:

- Press RECIPES button on HMI main menu

- Select Edit Mode (password required)

- Navigate to desired parameter using touch buttons

- Press INCREASE or DECREASE to adjust value

- Confirm new value (should show on screen)

- Press SAVE to store in PLC memory

- Press RUN to apply new settings on next cycle

⚠️ WARNING: Never adjust spindle speed above 1460 RPM or clamping pressure above 105 BAR without supervisor approval. These are safety limits set by machine design.

### 9.2 Calibration & Zero-Point Adjustment

#### 9.2.1 Servo Zero-Point Calibration (Component Feeder)

When Required: If component feeder does not return to starting position (drifts), or if servo moves too far forward/backward

Procedure:

- Set machine to SETUP mode on HMI

- Move component feeder servo to fully retracted position (HOME position)

- Connect Mitsubishi servo diagnostic tool to PLC RS-232 port

- Run "Zero Calibration" routine in servo software

- Servo will move forward to mechanical stop, then return to zero position

- Confirm zero position on screen; save in PLC memory

- Test with manual jog commands; verify forward/backward movement is symmetric

#### 9.2.2 Drill Head Servo Calibration

When Required: If drill head does not stop at proper depth, or if servo is not providing smooth up/down motion

Procedure:

- Set machine to SETUP mode

- Connect Mitsubishi servo tool to PLC

- Command drill head to move DOWN slowly; observe when tip reaches reference point

- Mark reference position on PLC display (zero point for drilling)

- Command drill head to move UP to fully retracted position

- Servo should move exactly same distance in both directions (verify on position display)

- Adjust scaling factor if asymmetric movement detected

- Save calibration; test with manual jog

#### 9.2.3 Poka-Yoke Sensor Threshold Adjustment

When Required: If sensors are falsely triggering errors, or missing legitimate faults

For Proximity Sensors (Presence Detection):

- Access HMI SETUP → SENSOR CALIBRATION menu

- Place reference object (component) at sensor detection zone

- Press TEACH button; sensor "learns" object distance

- Threshold automatically set to trigger 10–20 mm before physical contact

- Test with actual component; verify green light only when component present

For Pressure Sensors (Clamping):

- Pressurize hydraulic system to idle pressure (~30 BAR)

- Access HMI SETUP → PRESSURE THRESHOLDS

- Record baseline pressure reading

- Adjust minimum clamp pressure threshold: Should be 70 BAR (80% of rated 90 BAR)

- Adjust maximum clamp pressure threshold: Should be 105 BAR (relief valve limit)

- System will alarm if pressure exceeds 105 BAR or falls below 70 BAR during clamp

- Save settings

## 10. APPENDICES

### 10A. Electrical Wiring Diagram

Electrical Circuit Components:

The machine electrical system consists of:

- 3-Phase 415V AC main supply from plant distribution board

- Main isolator switch (16A capacity) with LOTO capability

- Motor soft starter/VFD for spindle motor control (22 kW Schneider VFD)

- 24V DC emergency stop circuit (critical safety circuit with UPS backup)

- UPS battery backup for emergency stop function (minimum 10-second runtime)

- Motor starters and contactors for pump, servo, and auxiliary motors

- Mitsubishi FX5U PLC for machine control and sequencing

- ProFace 10" HMI touchscreen for operator interface

- SICK SFO-4 Safety Relay for door interlock and safety function control

- 24V DC power supply (SMPS) for all control circuits

Refer to attached electrical drawing for complete circuit diagram and terminal connections.

### 10B. Hydraulic System Schematic

Hydraulic System Components:

The machine hydraulic system includes:

- Main hydraulic pump (motor-driven) with flow rate 8 LPM @ 1460 RPM

- Pressure relief valve (105 BAR setting) for main circuit protection

- Directional control solenoid valves for clamping and servo control

- Servo-controlled pressure reducing valves (pilot-operated) for smooth motion

- Pressure gauges on main circuit (0-150 BAR) and lubrication circuit (0-20 BAR)

- Hydraulic tank with drain and filler ports, level sight glass

- Return line filter (10-micron) for fluid cleaning

- Separate lubrication circuit (10 BAR, 30 LPM) for spindle cooling and bearing lubrication

Refer to attached hydraulic circuit drawing for complete schematic and component layout.

### 10C. Pneumatic System Layout

Pneumatic System Components:

- Plant compressed air source (5–6 BAR regulated supply)

- Line-mounted 5-micron filter for air quality

- Pressure regulator with gauge for supply monitoring

- Manual isolation ball valves at key points

- Solenoid dump valve for pneumatic LOTO function

- Individual hose runs to Festo servo drives for component feeder control

### 10D. Machine Dimensional Drawings

Reference Drawing Number: D/Meth/050/Truck Drilling/CH/01 Rev. 1 (Accurate Machines)

Key Dimensions:

- Overall Length: 2.315 M

- Overall Width: 2.103 M

- Overall Height: 3.048 M

- Total Weight: 1.0 Ton

### 10E. PLC Program Flow Chart

PLC Controller: Mitsubishi FX5U Series

Key Specifications:

- Scan Time: 10 ms (100 cycles per second)

- Backup Battery: CR2032 (RTC battery)

- Program Memory: Sufficient for all control functions

- Safety Features: Real-time monitoring of all sensor inputs and safety interlocks

### 10F. Component Specifications & Tolerances

Clutch Facing Dimension Reference (Per Valeo D/Meth/025 Rev. 12):

| Dimension | Mini | Maxi | Tolerance |
| --- | --- | --- | --- |
| Outer Diameter | 280 mm | 430 mm | ±0.5 mm |
| Thickness | 2.8 mm | 6.2 mm | ±0.2 mm |
| No. of Holes | 24 | 54 | Exact count |
| Hole Diameter | 7.75 mm | 14.5 mm | ±0.1 mm |
| Hole Position Runout | — | — | Max 0.8 mm |
| Rivet Hole Thickness | 0.9–2.1 mm | — | ±0.05 mm |

### 10G. Machine Operating Log Template

| Date | Shift | Operator Name | Start Time | End Time | Total Parts Produced | Rejected Parts | Defect Code | Maintenance Actions | Supervisor Sign |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 10H. Machine Maintenance Log Template

| s | Maintenance Task | Duration (mins) | Components Replaced/Serviced | Technician Name | Status |
| --- | --- | --- | --- | --- | --- |

### 10I. Accident/Incident Report Form

Machine Incident Report – Truck Drilling Machine

Date & Time: _______________
 Location: _______________
 Operator Name: _______________
 Operator ID: _______________

Incident Type (Circle): Minor Injury | Serious Injury | Equipment Damage | Near-Miss | Other

Description of Incident:

Root Cause (If Known):

Immediate Action Taken:

Witness Names & Signatures:

- _______________ Signature: _______________

- _______________ Signature: _______________

Supervisor Review & Comments:

Supervisor Signature: _______________ Date: _______________

Machine Status After Incident (Circle): Operational | Minor Repair Needed | Out of Service | Requires Safety Inspection

### 10J. Glossary of Terms

Poka-Yoke: Japanese term meaning "mistake-proof." Automatic error detection system that stops machine if abnormal condition detected.

Servo Motor: Precision electric motor that responds to PLC commands for precise position/velocity control.

Solenoid Valve: Electromagnetic device that opens/closes fluid passages when energized.

MTBF (Mean Time Between Failures): Average operating time between equipment breakdowns. Target: 8 hours minimum.

TRP (Total Run Percentage): Percentage of scheduled operating time that machine actually runs without stoppage. Target: 85% minimum.

MTTR (Mean Time To Repair): Average time required to fix machine after failure. Target: maximum 1 hour.

Proximity Sensor: Non-contact sensor that detects object presence/position using electromagnetic or optical technology.

LOTO (Lock-Out/Tag-Out): Safety procedure to isolate energy sources and prevent accidental machine restart during maintenance.

Runout: Amount of deviation from perfect concentricity; measured as radial distance from true center. Tolerance: 0.8 mm maximum.

Eccentricity: Off-center condition of a circular component; detected by concentricity poka-yoke sensor.

### 10K. Emergency Contact Numbers

| Contact Type | Name | Mobile | Email | Availability |
| --- | --- | --- | --- | --- |
| Manufacturer (Technical Support) |  |  |  |  |
| Maintenance Supervisor |  |  |  | 24/7 (on-call) |
| Plant Safety Officer |  |  |  | 24/7 (on-call) |
| Electrical Technician |  |  |  | Business hours |
| Hydraulics Specialist |  |  |  | Business hours |
| Plant Emergency Services |  |  |  | 24/7 |

### 10L. Training Certification Record

Employee Name: _______________
 Employee ID: _______________
 Department: _______________
 Training Date: _______________
 Trainer Name & Signature: _______________

Training Topics Completed (Check):

- [ ] Machine Safety & Hazard Awareness

- [ ] Emergency Stop & LOTO Procedures

- [ ] Pre-Start Verification Checklist (Section 6.1)

- [ ] Normal Operating Cycle (Section 6.2)

- [ ] Troubleshooting Common Issues (Section 8.1)

- [ ] Maintenance Schedule & Procedures (Section 7)

- [ ] Poka-Yoke Error Codes (Section 8.2)

- [ ] Component Loading & Quality Verification

- [ ] Equipment Shutdown & End-of-Shift Procedures

- [ ] Hands-On Practical Training (Supervised)

Trainer Assessment: (Circle) PASS | CONDITIONAL PASS | NOT READY

Trainer Signature: _______________ Date: _______________

Plant Supervisor Sign-Off (Authorization to Operate Independently):
 _______________ Date: _______________

## DOCUMENT CONTROL

| Version | Date | Changes Made | Author | Approved By |
| --- | --- | --- | --- | --- |
| 1.0 | January 2025 | Initial document creation |  |  |
| 8.0 | January 2025 | Cleaned version - removed coolant, placeholders, added HMI descriptions |  |  |

Document Custodian: Accurate Machines & Accessories (Pvt) Ltd.
 Next Review Date: January 2026
 Distribution: Digital copy on plant intranet; printed manuals at operator stations

END OF MANUAL

M/s ACCURATE MACHINES AND ACCESSORIES PRIVATE LIMITED

397, SIDCO Industrial Estate,
 Ambattur, Chennai - 600 098, INDIA