# TRUCK DRILLING MACHINE OPERATING MANUAL

Model: Truck Multi-Spindle Drilling Machine (48-Spindle Configuration)

Effective Date: January 2026

Document Classification: Technical / Operational

# TABLE OF CONTENTS

- General & Legal Information

- Machine Identification & Overview

- Technical Specifications & Data

- Safety Information & Warnings

- Transport, Installation & Commissioning

- Operating Procedures

- Maintenance & Lubrication

- Troubleshooting & Alarm Codes

- Spare Parts & Consumables

- Appendices

# GENERAL & LEGAL INFORMATION

## Manufacturer Details

| Item | Details |
| --- | --- |
| Manufacturer | Accurate Machines & Accessories (Pvt) Ltd. |
| Address | No.397, Sidco Industrial Estate, Ambattur, Chennai – 600 098, India |

| Contact Number | Mr.V.Purushothaman - 98402 46804 Director. |
| --- | --- |
| Email | info@acmaa.net |
| Website | www.acmaa.net |

## Machine Purpose & Application

This Truck Drilling Machine is designed for high-precision drilling of clutch facing components made of composite materials (organic and inorganic fibers with thermosetting resins). The machine conforms to Valeo quality standards and is intended for automated manufacturing environments.

Key Application: Drilling multiple holes in clutch facings as per Valeo specifications with automatic poka-yoke quality checks.

## Regulatory Compliance & Standards

This machine is designed and manufactured in accordance with the following Indian and International standards:

- IS 4571:1968 – General Safety Requirements for Machinery

- IS 12100-1:2003 – Safety of Machinery (General Principles of Design)

- IS 12100-2:2003 – Safety of Machinery (Technical Principles and Specifications)

- IS 13598:2013 – Industrial Safety Management Systems

- ISO 14121 – Risk Assessment & Reduction

- Valeo General Equipment Specifications (D/Meth/025 Rev. 12)

## Warranty & Liability

Machine Warranty Valid for 1 year since the date of Supply & Invoicing.

# MACHINE IDENTIFICATION & OVERVIEW

## Machine Identification Data

| Parameter | Specification |
| --- | --- |
| Machine Type | Truck Multi-Spindle Drilling Machine |
| Configuration | 54-Spindle (Gang Drilling) |
| Serial Number | 2710 |
| Year of Manufacture | 2025 |
| Control System | Mitsubishi FX5U PLC + ProFace 10" HMI |
| Electrical Supply | 3-Phase with neutral, 415V AC, 50 Hz |
| Drawing Reference | D/Meth/050/Truck Drilling/CH/01 Rev. 1 |

## Machine Layout & Main Components

### Main Subsystems:

- Loading & Feeding System – V-feeder with servo control

- Drilling Head Assembly – Multi-spindle rotating head (1460 RPM)

- Hydraulic System – Drill head, Fixture clamping & Spindle cooling

- Pneumatic System – During Drill head changeover

- Control System – PLC, HMI, Safety interlocks, Door interlocks

- Dust Collection System – Suction type

- Safety System – Emergency stop, safety doors, interlock switches

## Typical Product Specifications

The machine processes clutch facings with the following dimensional range:

| Parameter | Minimum | Maximum |
| --- | --- | --- |
| Clutch Facing Outside Diameter | 280 mm | 430 mm |
| Clutch Facing Thickness | 2.8 mm | 6.2 mm |
| Number of Holes | 24 Nos. | 54 Nos. |
| Hole Diameter | 7.75 mm | 14.5 mm |
| Rivet Hole Thickness | 0.9 mm | 2.1 mm |
| Hole Position Runout (Tolerance) | ±0.1 mm | ±0.1 mm |
| Concentricity (Maximum Offset) | — | 0.8 mm |

# TECHNICAL SPECIFICATIONS & DATA

## Dimensional Data

| Parameter | Specification | Unit |
| --- | --- | --- |
| Machine Length | 2.315 | Meters |
| Machine Width | 2.103 | Meters |
| Machine Height | 3.048 | Meters |
| Total Machine Weight | 4.5 | Ton (1000 kg) |

## Power & Energy Consumption

| Parameter | Specification | Remarks |
| --- | --- | --- |
| Total Installed Power | 30 | kW |
| Motor Rating (Spindle Drive) | 22 | kW @ 1460 RPM |
| Servo Motor (Feed) | 0.4 | kW |
| Servo Motor (Drill Head Movement) | 3.5 | kW |
| Hydraulic Pump Motor | 1.5 | kW |
| Pneumatic Compressor Requirement | 5–6 | Bar (minimum) |

## Hydraulic System Specifications

| Circuit | Max. Pressure | Max. Flow | Application |
| --- | --- | --- | --- |
| Main (Spindle Clamping) | 40 BAR | 8 LPM | Drill head & Fixture clamping by Hydraulic cylinders |
| Cooling & Lubrication | 2 BAR | 30 LPM | Spindle cooling via Hydraulic power pack Slide Lubrication |

### Hydraulic Fluid Specification:

- Type: Hydrol 68

- Capacity: 100 Ltrs.

- Replacement Interval: Once every 6 Months

## Pneumatic System Specifications

| Parameter | Specification |
| --- | --- |
| Operating Pressure | 5–6 Bar |
| Supply Source | Plant Compressed Air System |
| Filtration | 5-micron filter (minimum) |
| Desiccant Dryer | Required for humid climates |
| Solenoid Valve Rating | 24 V DC |

## Electrical System Specifications

| Parameter | Specification |
| --- | --- |
| Mains Supply | 3-Phase with neutral, 415V AC, 50 Hz, 63A |
| Control Voltage | 24 V DC |
| PLC Controller | Mitsubishi FX5U Series – 80MT-ESS |
| HMI | ProFace 10" touchscreen panel – PFX ET 6500WAD |
| Emergency Stop | 24 V DC (Safety relay control) |
| Control Panel IP Rating | IP65 (dust-proof) |
| Safety Certification | Sick certified |

## Environmental Operating Conditions

| Parameter | Specification | Remarks |
| --- | --- | --- |
| Ambient Temperature Range | +5°C to +40°C | Optimal operation |
| Relative Humidity | 20% to 85% | Non-condensing |
| Noise Level | <75 dB | At full operation |

## Performance Metrics & Acceptance Criteria

| Function | Specification | Unit | Tolerance |
| --- | --- | --- | --- |
| Spindle Rotation Speed | 1460 | RPM | Minimum |
| Production Capacity | 240 to 720 | Parts/Hour | Minimum to Maximum |
| MTBF (Mean Time Between Failures) | 8 | Hours | Minimum |
| TRP (Total Run Percentage) | 85 | % | Minimum |
| MTTR (Mean Time To Repair) | 1 | Hour | Maximum |
| Changeover Time (Tooling) | 30 | Minutes | Maximum |
| Dimensional Accuracy (Thickness Under Rivet Head) | ±0.05 | mm | Cp/Cpk >1.6 |
| Concentricity (PCD Eccentricity) | — | — | Cmk = 2.0 minimum |

# SAFETY INFORMATION & WARNINGS

# Machine Alarm Screen

## General Safety Precautions

ı. WARNING: This is a high-speed, precision drilling machine with rotating cutting tools and moving parts. Failure to follow safety procedures may result in serious injury or death.

### Before Operating this Machine:

- Read this entire manual and understand all warnings and instructions

- Receive proper training from an authorized trainer

- Always wear appropriate Personal Protective Equipment (PPE)

- Never operate this machine while fatigued or under the influence of alcohol/drugs

- Keep hands, long hair, loose clothing, and jewelry away from moving parts

- Never reach into the machine during operation

- Always use provided jigs, fixtures, and clamping devices as specified

## Personal Protective Equipment (PPE)

### Mandatory PPE:

| PPE Item | Specification | Reason |
| --- | --- | --- |
| Safety Glasses | Anti-scratch, polycarbonate lens | Chip/dust protection |
| Ear Protection | Noise level <75 dB; use when extended exposure | Hearing protection |
| Hand Gloves | Cotton or leather (remove near rotating spindle) | Oil and chip handling |
| Safety Shoes | Steel-toed, slip-resistant | Foot protection from dropped parts |
| Work Apron | Durable synthetic material | Protection from coolant splash |
| Hair Covering | For operators with long hair | Prevent entanglement |
| Mask | Dust Protection Mask | Helps to prevent dust. |

## Safety Features of the Machine

The machine is equipped with the following safety systems:

- Safety Interlocked Doors – Machine cannot run if safety doors are open

- Emergency Stop (E-Stop) Button – Red mushroom button at operator station; stops all motion immediately

- Automatic Machine Stop – Triggers on detection of any failure or abnormal condition

- Light Signal Indicator – Visual alarm display on HMI and operator station

- Poka-Yoke Sensors – Prevent component overlap, detect missing holes, verify concentricity

- Dust Collection System – Operator to check Manually before Machine start

- Tool Clamping Safety – Manual tool clamping/declamping

- Drill Head Locking Pin – Manual safety pin to lock drill head before maintenance

## Lock-Out & Tag-Out (LOTO) Procedures

### ENERGY ISOLATION POINTS:s

| ### Types of Hazards | ### Symbol |
| --- | --- |
| ### Electrical |  |
| ### Pneumatic |  |
| ### Hydraulic |  |

| Energy Type | Isolation Method | Location | Lockable |  | Procedure |
| --- | --- | --- | --- | --- | --- |
| Electrical | Main isolator switch | Control panel (left side) | Yes |  | Switch OFF + Lock with padlock |
| Hydraulic (Main) | Ball valve on pump discharge | Hydraulic manifold | Yes |  | Close fully + Lock if possible |
| Pneumatic | Solenoid dump valve | Pneumatic control block | No | Energize "Dump" switch on HMI |  |
| Mechanical | Drill head locking pin | Above drill head assembly | Yes | Insert pin + Lock pin slot |  |

### LOTO Procedure Before Maintenance:

- Press EMERGENCY STOP button – Red button at operator station

- Switch OFF Main Electrical Isolator – Located on control panel left side; turn clockwise to OFF position

- Insert padlock through isolator switch lever – To prevent accidental restart

- Isolate Hydraulic Pressure – Close main pump discharge valve on hydraulic manifold; lock if slot available

- Isolate Pneumatic Supply – Switch HMI "Pneumatic Dump" to ON; listen for pressure release hiss

- Insert Drill Head Locking Pin – Locate pin slot above drill head; insert safety pin; secure with lock pin

- Verify Zero Energy State – Use pressure gauge to confirm hydraulic pressure drops to zero; listen for silence

- Place LOTO Tag – Attach personal LOTO tag to isolator and locking pin with your name and timestamp

- Remove Power Cord (Optional) – For extended maintenance, unplug power cord at wall socket

### LOTO Procedure After Maintenance:

- Verify all tools removed from machine

- Verify all access panels closed and secured

- Remove all LOTO tags and locks

- Switch ON main electrical isolator

- Slowly open hydraulic pump discharge valve

- Switch HMI "Pneumatic Dump" to OFF

- Remove drill head locking pin

- Press RESET on HMI to clear any stored alarms

- Perform pre-start checks (Section 6.1) before restarting

## Prohibited Operations

### NEVER DO THE FOLLOWING:

- Operate the machine without authorized training and certification

- Remove or disable any safety interlocks, guards, or door switches

- Reach into the machine during operation

- Run the machine with safety doors open

- Attempt to clear chips or jams while machine is running

- Lubricate or adjust tools during spindle rotation

- Modify machine parameters without supervisor approval

- Use the machine for purposes other than specified drilling operations

- Override poka-yoke error signals to force operation

- Load or unload components at unsafe speeds or positions

### Counterbalance Calculation – Important Safety Information & Disclaimer

The counterbalance system for the vertical suspended load has been designed and calculated based on the following parameters:

#### Counter Balance Calculation Data

| SL.NO | DESCRIPTION | VALUE |
| --- | --- | --- |
| 1 | Mechanical Stopper to Base Plate | 491 mm |
| 2 | Mechanical Stopper to Chain Top Mounting (Home Position) | 1531 mm |
| 3 | Gear Box Ratio | 1:10 |
| 4 | Drive Maximum Speed | 3000 RPM (Actual: 600 RPM) |
| 5 | Chain Length | 1300 mm |
| 6 | Counter Balance Weight | 916 kg |
| 7 | Mechanical Stopper to Motor Mounting (Home Position) | 339 mm |

This counterbalance calculation is valid only when all the above parameters remain unchanged.

#### Manufacturer Disclaimer

- The counterbalance calculation provided is based on the machine configuration and load data supplied at the time of design.

- The manufacturer guarantees the safety of the counterbalance system only if the machine is operated and maintained strictly as per this manual.

- Any modification to the suspended load, counterweight, chain, mechanical stoppers, gearbox, drive parameters, or any other component of the system will invalidate the counterbalance calculation and may result in serious injury or equipment damage.

- The user is fully responsible for:

  - Regular inspection and verification of the actual suspended load weight.

  - Periodic checking of the counterweight (916 kg) and its secure mounting.

  - Inspection of chains, sprockets, mechanical stoppers, and mounting points.

  - Ensuring no unauthorized modifications are made to the system.

#### Mandatory User Responsibilities

The operator / end user must:

- Verify the suspended tool / load weight does not exceed the designed counterbalanced load.

- Perform daily visual inspection of the chain, counterweight, and mechanical stoppers.

- Conduct periodic (at least every 3 months or as per local regulations) thorough inspection and load verification by a competent person.

- Immediately stop using the machine if any abnormality is observed in the counterbalance system.

- Maintain proper records of all inspections and maintenance.

Note: The machine is equipped with a Mechanical Block Tool Holder with administrative control as an additional engineering safety measure. This does not relieve the user from the responsibility of regular verification and maintenance of the counterbalance system.

Manufacturer Liability Statement -

The manufacturer shall not be liable for any injury, death, or property damage resulting from unauthorized modifications, overloading, improper maintenance, or failure to verify the suspended load and counterbalance system as specified in this manual. Compliance with the above requirements is the sole responsibility of the user/owner.

# TRANSPORT, INSTALLATION & COMMISSIONING

## Transport & Handling

### Lifting & Rigging

      - Lifting Method: Four eye bolts (M16) installed on machine top frame for lifting

      - Rigging Equipment: 8-10 ton capacity slings rated for industrial machinery

      - Lifting Speed: Slow and controlled; maximum 0.5 m/s

      - Personnel: Minimum 2 certified riggers required

### Lifting Points Location:

      - Front-left and Rear-right

### Transportation Safety

      - Machine must be secured to transport vehicle using resilient lashings and spacers

      - Cover machine with protective tarpaulin to prevent weather damage

      - Avoid sudden acceleration, braking, and sharp turns during transport

      - Machine should be transported in horizontal position (NO tilting)

      - Remove any loose components, tool holders, or jigs before transport

      - Inspect for damage upon delivery before unloading

## Installation & Site Preparation

### Installation Requirements

| Requirement | Specification |
| --- | --- |
| Floor Loading Capacity | Minimum 1.5 ton/m² (concentrated) |
| Floor Evenness | ±5 mm tolerance over 3-meter span |
| Anchoring | Describe anchoring method if applicable] |
| Space Clearance | 1 meter on all sides for maintenance access |
| Environmental Conditions | +5°C to +40°C, <85% RH (non-condensing) |

### Electrical Installation

      - Supply Connection: By certified electrician only; 3-Phase 415V with Neutral AC, 50 Hz

      - Earth Connection: Machine frame must be grounded to plant earth (<5 ohm resistance)

      - Control Panel Location: Dry, accessible location; minimum 1 meter from machine for operator reach

      - Emergency Stop Button: Positioned at operator station; red color, easily accessible

### Hydraulic System Installation

      - Install main hydraulic power pack adjacent to machine (within 2 meters)

      - All hose runs should have drip trays underneath

      - Tank capacity: 100 liters

      - Filtration: 10-micron system with differential pressure indicator

### Pneumatic System Installation

      - Connect plant compressed air to machine input via 5-micron filter

      - Supply pressure: 5–6 bar (regulated at source)

      - Install pressure gauge on supply line for monitoring

      - Desiccant dryer recommended for humid climates (>70% RH)

## Commissioning Procedure

### Pre-Commissioning Checklist

      - [ ] Machine physically installed and secured

      - [ ] All electrical connections verified by certified electrician

      - [ ] Hydraulic system filled with specified fluid; no leaks detected

      - [ ] Pneumatic system pressurized and tested; all joints tight

      - [ ] Control panel powered up; HMI display responsive

      - [ ] All poka-yoke sensors functioning (verified with test blocks)

      - [ ] Drill head rotates freely at zero pressure; no mechanical binding

      - [ ] Safety doors close properly; interlock switches respond

      - [ ] Emergency stop button tested; stops all motion immediately

      - [ ] Dust collector system running; positive suction at spindle

### Commissioning Tests

- No-Load Spindle Rotation Test – Run spindle at 1460 RPM for 10 minutes; monitor vibration and noise

- Servo Movement Test – Jog feed servo forward/backward and drill head up/down; verify smooth motion

- Pressure Test – Monitor hydraulic pressure during clamping cycle; verify within specification

- Cycle Test (Empty) – Run 30 complete cycles without component to verify sequence and timing

- Cycle Test (Trial Components) – Run 20 cycles with trial clutch facing; measure hole accuracy and verify poka-yoke checks

- Production Run Test – Run minimum 100 parts at rated speed (270 p/h); verify MTBF and TRP targets

# OPERATING PROCEDURES

## PRE-START VERIFICATION CHECKLIST

Timing: Complete checklist before every shift start or after machine shutdown >30 minutes

### Procedure Step-by-Step:

Step 1: Verify Main Electrical Isolator is ON

- Location: Control panel, left side, red/black switch lever

- Status: Switch lever should be pointing UP (ON position)

- Action: If OFF, switch to ON position; proceed only after HMI powers up

### Step 2: Check Electrical Earth (Ground)

- Location: Main isolator panel, earth connection bolt (marked with green/yellow label)

- Status: Verify bolt is tight and shows no corrosion or white powder deposit

- Tool Required: Insulated screwdriver or multimeter for resistance check (target: <5 ohm to plant earth)

- Action: If loose or corroded, tighten or clean; report to maintenance if resistance >5 ohm

### Step 3: Verify HMI Ready & Responsive

- Location: ProFace 10" touchscreen on control panel, right side

- Action:

  - Observe startup sequence: Mitsubishi FX5U logo → Main menu screen

  - Verify all indicator lights illuminated: Power (Green), Ready (Blue), Alert Status (Yellow/Red if abnormal)

  - Touch main screen to confirm responsiveness; should show menu options

  - If screen frozen or no response, perform soft reset: Hold Reset button on HMI for 3 seconds

  - If still unresponsive, switch OFF isolator, wait 30 seconds, switch ON again (cold restart)

### Step 4: Check Component Buffer Level (Loading Area)

- Location: Loading tray assembly (top-left of machine)

- Status: Minimum 100–150 clutch facings should be stacked in loading area

- Poka-Yoke Sensor: Located at bottom of loading stack; should be covered (facings above minimum) for green light indicator

- Action:

  - If buffer below minimum: Load 100–150 new facings manually into tray

  - If green light indicator remains red even with full buffer: Check sensor lens for dust; clean with soft cloth if needed; report to maintenance if light remains red

  - Do NOT start machine if buffer sensor shows red alert

### Step 5: Verify Component Loading Area is Clear & Ready

- Location: V-feeder component entry point (center-left of machine)

- Status: Area should be empty; no components jammed or stuck in feeder

- Action:

  - Visually inspect feeder path from loading tray to drill jig; no debris or broken component pieces

  - Push feeder manually by hand (machine OFF) to verify smooth movement (servo powered OFF at this stage)

  - Verify groove poka-yoke sensor: Should show green light indicator when facing with groove is placed against feeder (sensor at +45° angle on feeder)

  - If feeder movement is jerky or sticky, report to maintenance; do NOT proceed

### Step 6: Inspect Component Overlap Area (Drill Jig Entry)

- Location: Drill jig seating area (below spindle head)

- Status: Jig should be empty; no overlapping or stacked components visible

- Overlap Poka-Yoke Sensor: Located above jig platform; detects when two components are stacked (height sensor)

- Action:

  - Visually confirm jig is empty

  - Place a single test facing in jig

  - Machine should detect single component: Green light on HMI overlay sensor indicator

  - Place a second test facing on top of first (overlay condition)

  - Machine should detect overlap: Red light and audible alarm on HMI

  - Remove test facing; jig should return to green light

  - If sensor does not respond to overlay condition, report to maintenance; do NOT proceed

### Step 7: Check Output/Ejection Area

- Location: Opposite side of drill jig (right side of machine)

- Status: Output collection area should be empty and ready to receive drilled components

- Ejection Path: Should be clear; no obstructions, no spillage from previous batch

- Collection Bin: Positioned directly under output chute; secure and ready

- Action:

  - Clear any spillage or residual components from output area

  - Position collection bin at output chute (center alignment)

  - Verify bin is empty and clean; ready to receive drilled facings

### Step 8: Verify Clamping Position & Poka-Yoke Sensor

- Location: Drill jig clamping assembly (below spindle)

- Status: Clamp should be in fully open position (relaxed state) at machine idle

- Clamping Poka-Yoke: Proximity sensor detects clamp position

- Action:

  - Visually confirm clamp arms are fully open (180° position)

  - Green indicator light should be visible on HMI clamping status

  - If clamp is partially closed or stuck, manually open by hand; report to maintenance if resistance felt

  - Attempt manual clamping cycle (if available on HMI test menu): Press "Test Clamp" button; verify clamp closes smoothly, then opens after 2-second hold

  - If clamp does not respond or moves jerkily, do NOT proceed; report to maintenance

### Step 9: Verify Top Drill Head Locking Pin Status

- Location: Above multi-spindle drilling head, center-top

- Status: Locking pin should be INSTALLED & LOCKED during pre-start (safety requirement)

- Purpose: Prevents accidental drill head movement during component loading or inspection

- Action:

  - Visually locate locking pin above drill head; should be inserted in locking slot (horizontal position)

  - Verify pin is fully seated; no gaps visible between pin and slot

  - Lock pin should have a padlock or keyed lock in place

  - If pin is missing or loose: DO NOT PROCEED; locate spare pin from maintenance kit and install; lock with padlock

  - This pin MUST remain installed until all component loading is complete and operator is ready to trigger first cycle start

### Step 10: Check Drill Head Oil Line – Input & Output Hoses

- Location: Top-rear of drill head assembly, two hoses: Pressure (Red) & Return (Blue)

- Status: Both hoses should be tightly connected; no visible leaks, cracks, or bulging

- Pressure Line (Red):

  - Should be firm to touch (pressurized during cycle only)

  - Connection nut should not be loose; use spanner for verification if needed

  - No weeping or drops of oil visible

- Return Line (Blue):

  - Should feel warm during idle (passive return flow)

  - Connection should be tight; no splashing or spraying

  - Should return to reservoir tank smoothly

- Action:

  - Gently tug both hoses to confirm they are firmly connected

  - Wipe both connection points with clean cloth; inspect for oil seepage

  - If either hose shows leaking, cracks, or bulging: STOP IMMEDIATELY; report to maintenance; do NOT use machine

  - Monitor hose temperature (touch gently with palm) – should be warm but not burning hot (target: <50°C)

  - If excessively hot, notify maintenance; may indicate internal blockage or pump issue

### Step 11: Verify Dust Collector System is Running

- Location: Dust collection unit (adjacent to main machine body, typically right side)

- Status: Motor should be running and producing suction at spindle drill area

- Visual Indicators:

  - Dust collector motor light (green indicator) should be illuminated on control panel

  - Sound check: Audible vacuum suction sound from dust lines near spindle

  - Filter observation: Dust should be visibly accumulating in collection bag/cartridge

- Action:

  - Switch ON dust collector motor (if not already running): Locate "Dust Collector" switch on HMI main menu or control panel; select ON

  - Verify suction by placing hand near spindle dust outlet (machine OFF) – should feel strong air pull inward

  - Check collection bag for fullness: If >75% full, empty bag before shift to maintain suction efficiency

  - Verify filter element is clean (visual check through sight glass): Whitish dust layer

= normal; dark/clogged appearance = replace filter

  - If no suction detected: Check filter blockage first; then check for hose disconnection; report to maintenance if issue persists

### Step 12: Final Visual Inspection

- Do a complete walk-around of the machine (360° view)

- Verify all safety doors are closed and interlocked

- Confirm no loose tools, parts, or materials on machine bed or underneath

- Check floor around machine: dry, no spills, no tripping hazards

- Verify operator station is organized; emergency stop button clearly visible and accessible

## NORMAL OPERATING CYCLE (Machine Operation Sequence)

### Cycle Initiation

Pre-Requisite: All pre-start checks (Section 6.1) completed with green status

### Operator Actions:

- Ensure drill head locking pin is INSTALLED and LOCKED (safety requirement)

- Load components manually into loading tray (100–150 pieces) – repeat as needed

- Proceed to HMI; main menu should be visible

- Touch "START PRODUCTION" button on HMI

- System will verify:

  - Buffer sensor status: Green (minimum components present)

  - Clamp position: Open (jig ready)

  - Safety doors: All closed and interlocked

- If any check fails: Red alarm light + audible buzzer; correct issue and retry

- If all checks pass: Remove drill head locking pin and proceed to cycle start

### NORMAL PRODUCTION CYCLE – STEP-BY-STEP FLOW

PHASE 1: COMPONENT LOADING (Duration: ~2–3 seconds)

Step 1.1 – Component Loading Area → Component Feeder

      - Actor: Pneumatic solenoid valve + Festo servo drive (Component Feeder)

### Action:

        - Solenoid energizes; Electrical servo actuator pushes component from loading tray into V-feeder

        - V-feeder servo motor activates; component pushed into feeder channel

        - Component should align with groove Manually

### Poka-Yoke Check:

        - Groove Manual checks if component orientation is correct (groove present on top)

      - Timing: 2–3 seconds

      - Outcome: Component now seated in V-feeder, ready for center positioning

PHASE 2: COMPONENT FEEDING – SERVO FORWARD MOVEMENT (Duration: ~4–5 seconds)

### Step 2.1 – Component Feeder Servo Move Forward

      - Actor: Festo servo drive (Component Feeder)

### Action:

        - Servo motor receives signal from PLC controller

        - Pushes component forward smoothly toward drill jig center

        - Motion should be controlled and slow (~50–100 mm/sec) to allow settling

      - Feedback: Linear position sensor on feeder confirms forward movement

      - Timing: 3–4 seconds

      - Outcome: Component pushed to center of drill jig

PHASE 3: COMPONENT CENTERING & OVERLAP CHECK (Duration: ~1–2 seconds)

### Step 3.1 – Overlap Detection Poka-Yoke Check

      - Sensor Location: Above drill jig (height sensor / proximity sensor)

      - Action: Sensor detects component height and checks if only ONE component is in jig

### Poka-Yoke Logic:

        - GREEN = single component detected → proceed to next step

        - RED = multiple components detected (overlap condition) → machine stops;

alarm sounds; operator must remove overlapped component and restart

      - Timing: ~1 second

      - Outcome: Single component confirmed in jig; ready for drilling

### Step 3.2 – Centering Unit Accuracy Check

        - Component centering automatic & auto-aligning during drilling.

        - After drilling to check manually for center drilling accuracy

        - Offset range of Centricity - +/- 0.8 mm

PHASE 4: DRILL HEAD POSITIONING – SERVO MOVEMENT (Duration: ~2–3 seconds)

### Step 4.1 – Drill Head Servo Move (Downward Positioning)

      - Actor: Mitsubishi servo motor + hydraulic servo valve (Drill Head Movement)

### Action:

        - Servo receives signal from PLC

        - Hydraulic actuator lowers drill head smoothly from retracted position (top) toward component

        - Servo control provides smooth, controlled descent at ~100–150 mm/sec

        - Linear position sensor on drill head confirms downward movement

      - Timing: 2–3 seconds

      - Outcome: Drill head positioned above component, ready for contact

### Step 4.2 – Drill Head Contact & Clamping Sequence

      - Action:

        - Drill head reaches proximity to component (10–15 mm clearance)

        - Pressure sensor on spindle detects light touch

        - Solenoid valve energizes; hydraulic pressure applies to clamping arms

        - Clamping arms close around component edges, securing it firmly in jig

      - Clamping Poka-Yoke Sensor: Confirms clamp is in fully closed position (proximity sensor)

        - Status: GREEN = clamp closed → proceed to drilling

        - Status: RED = clamp stuck or partial closure → machine stops; operator checks for obstruction

      - Clamping Pressure: Monitored by pressure sensor; should reach 35–40BAR within 1 second

        - If pressure does not reach 35 BAR: Poka-yoke alarm; stop cycle; check for leaks

      - Timing: 1 second

      - Outcome: Component firmly clamped; ready for drilling

PHASE 5: DRILLING CYCLE (Duration: ~4–6 seconds)

### Step 5.1 – Spindle Rotation & Drilling

      - Actor: Main spindle motor (1460 RPM) + Multi-spindle gang drill head (54 spindles)

### Action:

        - Spindle motor (3-phase AC induction motor, ~22 kW) energizes to rated speed:

### 1460 RPM

        - All 54 drill bits simultaneously contact component surface and begin cutting

        - Hydraulic lubrication pump supplies Oil lubricant at 2 BAR, 30 LPM through drill head

        - Chips (asbestos burr) flow downward into dust collection system

### Drilling Process:

        - Hole size: 7.75–14.5 mm depending on part configuration

        - Material: Composite (organic/inorganic fibers + thermoset resin)

        - Feed rate: Automatic servo control (adjusts based on drilling resistance)

### Monitoring:

        - Spindle current sensor: Should show 60–80% of rated motor current

        - Drill head oil flow: Should remain in 26 to 30 LPM range

        - Acoustic monitoring: Normal drilling sound; no grinding or unusual noise

      - Duration: 4–6 seconds (depends on hole depth and material hardness)

      - Outcome: All 54 holes drilled successfully through component

PHASE 6: COMPONENT FEEDER SERVO BACKWARD (Duration: ~3–4

seconds)

### Step 6.1 – Component Feeder Servo Move Backward

      - Actor: Festo servo drive (Component Feeder reverse)

### Action:

        - Servo receives reverse signal from PLC

        - Feeder servo pulls drilled component backward (away from jig) at controlled speed ~50–350 mm/sec

        - Component slides out of drill jig into ejection chute

      - Timing: 2–3 seconds

      - Outcome: Drilled component now positioned for ejection

PHASE 7: DRILL HEAD RETRACTION (Duration: ~2–3 seconds)

### Step 7.1 – Drill Head Servo Move (Upward Retraction)

      - Actor: Mitsubishi servo motor + hydraulic servo valve (Drill Head Movement)

### Action:

        - Servo receives upward command from PLC

        - Electrical servo actuator retracts drill head upward (away from jig) at ~70 mm/sec

      - Timing: 2–3 seconds

      - Outcome: Drill head fully retracted; machine ready for next component

PHASE 8: COMPONENT EJECTION & OUTPUT CHECK (Duration: ~1–2 seconds)

### Step 8.1 – Component Ejection from Output Chute

      - Actor: Pneumatic ejector pin or gravity-feed chute

### Action:

        - Electrical feeder servo actuator activates automatically after feeder retracts

        - Component pushed out of jig and slides down exit chute into collection bin below

        - Gravity and smooth chute surface ensure component reaches output bin without damage

      - Timing: 1 second

      - Outcome: Drilled component lands in output collection bin

### Step 8.2 – Output/Ejection Verification Poka-Yoke Sensor

      - Sensor Location: At output chute exit (near collection bin)

      - Sensor Type: Presence detector (optical or proximity)

      - Logic: Confirms drilled component has successfully exited machine

      - Outcome: Component successfully ejected and confirmed in output bin

### CYCLE COMPLETION & REPEAT

      - Total Cycle Time: ~12–15 seconds per component

      - Throughput: 240 components/hour (at full speed)

      - Automatic Repeat: Upon successful output verification, machine immediately triggers next component loading

      - Cycle continues until:

        - Buffer level drops below minimum (sensor red); machine stops; operator loads more components

        - Emergency stop button pressed; all motion halts immediately

        - Any poka-yoke check fails; machine stops; alarm displayed on HMI

        - Operator presses "STOP PRODUCTION" button on HMI; current cycle completes, then machine stops

## MACHINE STOP PROCEDURES

### Normal Stop (End of Shift / Controlled Shutdown)

- Press "STOP PRODUCTION" button on HMI

- Machine completes current component cycle (does not stop mid-cycle)

- Upon cycle completion:

  - Feeder servo returns to neutral

- Once machine is completely still (spindle fully stopped):

  - Switch dust collector OFF (if shift ending)

  - Switch main electrical isolator to OFF position

  - Perform end-of-shift checks (Section 6.4)

### Emergency Stop (Safety Situation)

When to Use: Immediate danger, safety hazard, or uncontrolled machine movement

- Press EMERGENCY STOP button immediately – Red mushroom button at operator station

- Effect: All motion stops instantly; spindle cuts off; hydraulics vent; pneumatics dump

### Recovery Steps:

  - Assess situation; identify hazard

  - If hazard eliminated: Turn emergency stop knob clockwise to reset

  - Perform pre-start checks (Section 6.1) before resuming operation

  - If hazard unresolved: Leave E-stop pressed; call maintenance supervisor

## END-OF-SHIFT CHECKS & SHUTDOWN

- Empty Loading Tray – Remove remaining components to storage

- Clean Component Areas – Wipe jig and feeder with clean cloth to remove debris

- Check for Leaks – Visually inspect all hose connections for oil seepage

- Monitor Dust Collector – Empty collection bag if >50% full; this ensures tomorrow's startup suction

- Note any Issues – Record in machine logbook:

  - Any poka-yoke errors encountered during shift

  - Production count and rejection count

  - Any unusual sounds, vibrations, or temperature

  - Maintenance action items for next shift

### Switch OFF Control Systems:

  - Dust collector OFF

  - HMI STOP PRODUCTION button (if still running)

  - Main electrical isolator OFF

### Secure Machine:

  - All safety doors closed and locked (if applicable)

  - Drill head locking pin inserted and locked (safety requirement)

  - No loose components or tools left on machine bed

# MAINTENANCE & LUBRICATION

## Maintenance Schedule

### Daily Maintenance (Before Each Shift)

| Task | Procedure |
| --- | --- |
| Visual Inspection | Check for oil leaks, loose fasteners, damage |
| Hose Inspection | Inspect all hydraulic and pneumatic hoses for cracks, bulging |
| Dust Collector | Empty collection bag if needed; check filter for blockage |
| Component Tray | Clean loading area; remove any broken component pieces |
| Drain Plug Check | Verify hydraulic tank drain plug is tight |
| Feeder Servo Actuator | Cleaning required every shift, Start & End of Shift. |

### Monthly Maintenance

| Task | Procedure | Tool/Material |
| --- | --- | --- |
| Hydraulic Fluid Check | Check fluid level in tank; top up if below minimum mark | Fluid type |
| Pneumatic Filter Cleaning | Clean 5-micron filter element; replace if clogged | Replacement filter, compressed air |
| Servo Motor Check | Verify smooth jog operation of feed servo and drill head servo | Jog buttons on HMI |
| Sensor Cleaning | Wipe sensor lenses with clean, soft cloth; remove dust | Lens cleaning cloth |

### Weekly Maintenance

| Task | Procedure | Details |
| --- | --- | --- |
| Lubrication System Check | Check lubrication pump output; verify coolant flow to spindle | Visual verification of mist/flow |
| Motor Current Draw | Measure spindle motor current under load; compare to baseline | Clamp meter; baseline: A |
| Safety Door Testing | Test all safety door interlocks; verify machine stops when opened | Open each door during cycle |

    - Quarterly Maintenance

| Task | Procedure | Details |
| --- | --- | --- |
| Gearbox Inspection | Check gearbox mounting bolts; listen for gear noise | Torque wrench; Bolt torque spec |
| Poka-Yoke Sensor Calibration | Recalibrate all sensor thresholds per OEM specification | Mitsubishi PLC programming kit |
| Hydraulic Filter Change | Replace main hydraulic filter element | Filter part number |
| Pressure Gauge Calibration | Verify hydraulic and pneumatic gauges are accurate | Calibration kit; |
| Spindle Alignment Check | Verify spindle runout with dial indicator | Dial indicator; max runout |

| Electrical Connections | Inspect all terminal blocks and connectors for corrosion | Contact cleaner, dielectric grease |
| --- | --- | --- |

    - Annual Maintenance

| Task | Procedure | Details |
| --- | --- | --- |
| Complete Hydraulic System Flush | Drain and replace all hydraulic fluid | Fluid capacity liters |
| Spindle Bearing Service | Bearing type– check for wear; replace if damaged | Bearing kit; service procedure manual |
| PLC Battery Replacement | Replace RTC (Real-Time Clock) battery in FX5U controller | Battery type: CR2032; location |
| Safety Certification Renewal | Perform third-party machine safety inspection (Sick) | Certifying agency contact |

## Lubrication Specification & Schedule

### Hydraulic Fluid Specification

| Parameter | Specification |
| --- | --- |
| Fluid Type | Hydrol 68 (or) Servo 68 |
| Equivalent Brands | HLP 68 (or) RAJ 68 |

| Tank Capacity | 100 Liters |
| --- | --- |
| Refill Quantity | 90 Liters per refill (check dipstick) |
| Min/Max Level | Min mark 250 mm; Max mark 400 mm on sight glass |
| Change Interval | Every 06 months |
| Disposal | Collect used fluid in sealed container; send to certified recycler (NOT in drain) |

### Spindle Lubrication (Oil)

| Parameter | Specification |
| --- | --- |
| Supply Pressure | 2 BAR (lubrication circuit) |
| Flow Rate | 30 LPM (for cooling and spindle bearing lubrication) |
| Refill Tank Capacity | 100 Liters |

### Pneumatic System Lubrication

| Component | Lubrication Method | Interval |
| --- | --- | --- |
| Solenoid Valves | Integrated FRL (Filter-Regulator-Lubricator) on supply line | Every 500 hours |

## Spare Parts & Consumables List

### Critical Spare Parts

| S.No. | DESCRIPTION | PART NO | MAKE | UNIT | QTY |
| --- | --- | --- | --- | --- | --- |
| HORIZONTAL SERVO SLIDE ARRANGEMENT |  |  |  |  |  |
| 1 | Servo drive | CMMT-AS-C4-3A-MP-S1 | FESTO | NOS | 1 |
| 2 | Servo motor | EMMT-AS-80-L-LS-RMB | FESTO | NOS | 1 |
| 3 | Gear unit | EMGA-80-P-G5-EAS-80 | FESTO | NOS | 1 |
| 4 | Toothed b.axis | ELGA-TB-RF-80-700-0H | FESTO | NOS | 1 |
| 5 | Axial kit | EAMM-A-N48-80G-G2 | FESTO | NOS | 1 |
| 6 | Motor cable | NEBM-M23G15-EH-5-Q9N-R3LEG14 | FESTO | NOS | 1 |
| 7 | Prox.switch | SIES-8M-PO-24V-K-7,5-OE | FESTO | NOS | 2 |
| 8 | Switch lug | SF-EGC-1-80 | FESTO | NOS | 1 |
| 9 | Profile mounting | MUE-70/80 | FESTO | NOS | 4 |
| 10 | Breaking resister | CACR-KL2-100-W1800 | FESTO | NOS | 1 |

| S.No. | DESCRIPTION | PART NO | MAKE | UNIT | QTY |
| --- | --- | --- | --- | --- | --- |
| VERTICAL ADJUSTMENT ARRANGEMENT |  |  |  |  |  |
| 1 | Plumber block | UCFL201D1 | NTN | NOS | 1 |
| 2 | 125 Diameter hand wheel | PHLK125-12 | MISUMI | NOS | 1 |

| S.No. | DESCRIPTION | PART NO | MAKE | UNIT | QTY |
| --- | --- | --- | --- | --- | --- |
| VERTICAL SERVO SLIDE ARRANGEMENT |  |  |  |  |  |
| 1 | 16.7 Nm, 3.5 kW, 2000 RPM plain shaft with brake 200 V class servo motor | HG-SR352B | MITSUBISHI | NOS | 1 |
| 2 | Amplifier | MR-J4-350B | MITSUBISHI | NOS | 1 |  |
| 3 | 5m Encoder Cable, Standard | MR-J3ENSCBL5M-L | MITSUBISHI | NOS | 1 |  |
| 4 | Connector | MS3106F2222S+TB | MITSUBISHI | NOS | 1 |  |
| 5 | Brake Connector | MR-BKCNS1 | MITSUBISHI | NOS | 1 |  |
| 6 | Connector Set | MR-J3CN1 | MITSUBISHI | NOS | 1 |  |
| 7 | Gearbox | FE145 RATIO 1/10 Rated output torque 356 Nm Backlash <8arcmin | LIMING | NOS | 1 |  |
| 8 | Disc coupling | DRW 90C 20X20 OD-Ø94.5mm ,L-98mm,Bore 1-Ø30mm-Keyway-8x3.3,Bore 2- Ø35 | DURI | NOS | 1 |  |
| 9 | Lock nut (M40X1.5) | KM-8 | SKF | NOS | 4 |  |
| 10 | Lock washer (M40) | MB-8 | SKF | NOS | 2 |  |
| 11 | Deep groove ball bearing | 6208-2z | SKF | NOS | 2 |  |
| 12 | Angular contact ball bearing | 7208_be-2rzp_2 | SKF | NOS | 2 |  |
| 13 | Oil seal | 65X50X8 | STD | NOS | 4 |  |
| 14 | Ground ballscrew | FDW Ground ball bearing (50-10B1) L-1500mm | HIWIN | NOS | 1 |  |
| 15 | Ground ballscrew nut | R50-10B2-FDT-1250-1550-0.008 | HIWIN | NOS | 1 |  |
| 16 | Rail | RGR45R1500H | HIWIN | NOS | 2 |  |
| 17 | Rail block | RGW45HCZAH | HIWIN | NOS | 6 |  |
| 18 | Spindle unit motor | BBL 3 phase AC IE3 Class motor 22KW/30 HP 4 Pole/1465 RPM / B5-flange /s1 duty /Frame-160 L | BIJLEE | NOS | 1 |  |

| S.No. | DESCRIPTION | PART NO | MAKE | UNIT | QTY |
| --- | --- | --- | --- | --- | --- |
| COUNTER BALANCE ARRANGEMENT |  |  |  |  |  |
| 1 | Deep groove ball bearing | 6205-2z | SKF | NOS | 4 |
| 2 | 3/4 " chain | (Chain no-12.0 , Pitch-19.05) | STD | NOS | 14 FEET |
| 3 | Lock nut (M25X1.5) | KM-5 | SKF | NOS | 8 |
| 4 | Lock washer (M25) | MB-5 | SKF | NOS | 4 |
| FIXTURE HOLDING ARRANGEMENT |  |  |  |  |  |
| 1 | Air lifter | ISR-38-700-A | MISUMI | NOS | 2 |
| 2 | Rocker clamp | RCS-3006 | KJF | NOS | 4 |
| DRILL HEAD HOLDING ARRANGEMENT |  |  |  |  |  |
| 1 | Swing cylinder L.H | LHA0551-LH | LJAYA | NOS | 2 |
| 2 | Swing cylinder R.H | LHA0551-RH | LJAYA | NOS | 2 |
| 3 | Flow control valve | BLZ200-A | LJAYA | NOS | 4 |
| SAFETY DOOR LOCK & SENSOR ARRANGEMENT |  |  |  |  |  |
| 1 | Safety door lock | IE10-S2 | SICK | NOS | 1 |
| 2 | Safety door lock actuator | I10-M0233 | SICK | NOS | 1 |
| 3 | Safety interlock switches | RE13-SAC | SICK | NOS | 8 |
| 4 | Safety controller | FLX3-CPUC100 | SICK | NOS | 1 |
| 5 | Sensor cable | YF8U14-050VA3XLEAX | SICK | NOS | 10 |
| 6 | Sensor cable | YF2A14-050VB3XLEAX | SICK | NOS | 5 |
| 7 | Poka yoke sensor feeding area | GTB6-P4231 | SICK | NOS | 2 |
| 8 | Poka yoke sensor drilling area | IME12-4BPSZC0S | SICK | NOS | 2 |
| 9 | Poka yoke sensor | IME18-8BPSZC0S | SICK | NOS | 3 |

### Consumable Items (Reorder as Needed)

| Item | Specification | Consumption Rate | Reorder Point |
| --- | --- | --- | --- |
| Hydraulic Fluid | Hydrol 68 | 5–10 liters/month | 20 liters stock |
| Pneumatic Filter Elements | 5-micron replacement cartridge | 1 per 500 hours | Always 2 in stock |
| Interlock Switch | Magnetic proximity type, 24V DC Solenoid switch 24v DC | 1 per 2–3 years | 1 in stock |

### Optional Spare Parts (1 Unit Each)

| Part | Purpose |
| --- | --- |
| Complete Control Panel Module | Full HMI + PLC replacement (emergency backup) |
| Spindle Assembly (Complete) | Replacement spindle head if bearing fails |
| Hydraulic Pump Unit | Main pump replacement |

### Spare Parts Contact & Ordering

| Vendor | Contact Info |
| --- | --- |
| Accurate Machines (Manufacturer) | [PLACEHOLDER: Phone, Email] |

# TROUBLESHOOTING & ALARM CODES

## Common Issues & Solutions

### Issue 1: Machine Does Not Start – Green Light Does Not Appear on HMI Possible Causes:

- Main electrical isolator switch is OFF

  - Check: Verify isolator switch lever points UP (ON position)

  - Action: Switch to ON; observe HMI startup sequence

  - Expected: Mitsubishi logo appears on screen within 5 seconds

### Electrical power supply is not available

  - Check: Verify 3-Phase with neutral 415V supply is present at plant distribution board

  - Action: Use multimeter to check voltage at control panel terminal (should read

~415V)

  - Expected: Voltage between 380–440V is acceptable

### Main isolator switch is faulty (stuck)

  - Check: Turn switch slowly; should hear distinct "click" when switching positions

  - Action: Do NOT force; contact maintenance for replacement

  - Expected: Smooth switching action with audible click

### HMI display is powered but not showing startup screen (black screen)

  - Check: Is HMI backlight illuminated? (look at screen in dim light)

  - Action: Press and hold RESET button on HMI for 3 seconds (soft restart)

  - Expected: Mitsubishi logo and main menu should appear within 10 seconds

  - If still blank: Perform cold restart – switch isolator OFF, wait 30 seconds, switch ON again

### Resolution Steps (In Order):

- Check electrical isolator is ON; observe HMI green light

- If no light, perform cold restart (OFF 30 sec, then ON)

- If still no response, use multimeter to verify 3-phase supply with neutral at panel

- If supply OK but no HMI: Contact maintenance for control panel troubleshooting

### Issue 2: Buffer Sensor Showing Red Light – Machine Stops & Will Not Start Cycle Possible Causes:

- Minimum buffer level not met

  - Check: Loading tray should contain minimum 100–150 clutch facings

  - Action: Manually load more components into loading tray until red light changes to green

  - Expected: Green light after ~50 components added

### Buffer sensor lens is dirty/dusty

  - Check: Locate buffer sensor (bottom of loading stack); examine lens for dust deposit

  - Action: Gently wipe sensor lens with clean, soft cloth (microfiber preferred)

  - Expected: Red light changes to green; sensor responds to component presence

### Buffer sensor is misaligned

  - Check: Is sensor pointing at component stack, or has it been bumped?

  - Action: Visually inspect sensor mounting bracket; should be perpendicular to stack direction

  - Expected: Sensor should detect light reflection from top component

### Sensor is malfunctioning (electronic failure)

  - Check: Sensor indicator light on sensor body should blink when hand passed over lens

  - Action: If no blink or constant ON/OFF: sensor requires replacement

  - Expected: Blinking indicator = sensor working; steady light = possible failure

  - Next Step: Contact maintenance for sensor replacement (lead time: 2–3 days)

### Resolution Steps (In Order):

- Load additional components until buffer light shows green

- If light remains red: Clean sensor lens and retry

- If still red: Check sensor mounting alignment

- If issue persists: Contact maintenance; sensor likely requires replacement

### Issue 3: Spindle Does Not Rotate – Drill Head Not Spinning at Start of Cycle Possible Causes:

- Main spindle motor is not receiving 3-phase AC power

  - Check: Listen for motor humming/buzzing sound; should hear audible hum at idle

  - Action: Use multimeter to verify 3-phase with neutral voltage at motor terminals (should be

~415V AC between phases)

  - Expected: Approximately 415V between any two phase terminals; 0V on ground-to-phase = fault

  - Next Step: If no voltage, check main contactor on motor starter panel

### Main contactor (motor starter) is stuck open

  - Check: Locate main contactor relay on motor starter panel (usually large black box with 3 terminals)

  - Action: Observe contactor coil indicator light; should illuminate when cycle starts

  - Expected: Coil light ON = contactor energized; coil light OFF = contactor not energizing

  - Next Step: If light OFF, check 24V DC supply to contactor coil

### Spindle motor is mechanically jammed

  - Check: Attempt to manually rotate spindle shaft by hand (machine OFF, E-stop pressed)

  - Action: Grasp spindle shaft or pulley; try to turn gently (should rotate freely with minimal resistance)

  - Expected: Spindle rotates freely; no binding sensation

  - If stuck/grinding noise: Internal bearing failure; stop immediately, contact maintenance

### Spindle thermal overload protection has tripped

  - Check: Feel motor housing with palm – should be warm but not burning hot (<50°C)

  - Expected: Motor temperature drops back to <40°C; thermal reset occurs automatically

  - Next Step: If motor keeps overheating on restart, check for bearing friction (grinding noise)

### Resolution Steps (In Order):

- Listen for motor humming sound; if present, spindle driver is energized (power OK)

- If no hum: Check 3-phase with neutral voltage at motor using multimeter

- If voltage OK but no spin: Check motor manually; should rotate freely by hand

- If motor stuck: Stop immediately; contact maintenance

- If motor rotates freely but won't run: Check main contactor and motor starter panel

### Issue 5: Poka-Yoke Sensor Showing Repeated Red Errors – Components Rejected Possible Causes & Corresponding Poka-Yoke Check:

### Error Type A: Overlap Poka-Yoke (Drill Jig Entry) – Red Light

  - Meaning: Two or more components stacked in jig (height sensor detected overlap)

  - Cause: Component not fully ejected from jig before next component entered

### Action:

    - Stop machine

    - Manually remove stacked components from jig

    - Check component feeder servo: should retract fully after each cycle

    - Verify servo air pressure is adequate (5–6 bar minimum)

    - Reload single component; attempt cycle

  - Prevention: Ensure pneumatic pressure does NOT drop below 5 bar during operation

### Resolution Steps for Any Poka-Yoke Red Error:

- Stop machine immediately (press STOP PRODUCTION on HMI)

- Identify which poka-yoke check failed (HMI should display error code or description)

- Remove failed component and any subsequent stuck components

- Perform corrective action per error type above

- Clear error by pressing RESET on HMI (should go from red to green)

- Reload component batch; restart production

- If error repeats: Escalate to maintenance supervisor; do NOT force machine to run with repeated errors

## HMI Alarm Codes & Messages

## When to Call Maintenance

| Situation | Call Maintenance Immediately |
| --- | --- |
| Spindle makes grinding noise | YES – Bearing failure likely |
| Hydraulic leak (visible oil drops) | YES – System pressure compromised |
| Electrical shock felt when touching machine | YES – Electrical hazard; E-stop & isolate |
| Machine vibration increases suddenly | YES – Mechanical imbalance |
| Repeated poka-yoke errors not resolved | YES – Sensor calibration required |
| Emergency stop button not resetting | YES – Safety circuit fault |
| Hose rupture with fluid spray | YES – SHUT DOWN IMMEDIATELY |
| Control panel smoking or burning smell | YES – EVACUATE & EVACUATE – Fire risk |

# RECIPE PARAMETERS & CONFIGURATION

## Machine Recipe/Program Settings

NOTE: The following parameters are programmable via HMI menu. Changes should be made by authorized personnel only (Supervisor or above).

| Parameter | Default Value | Range | Unit | Purpose |
| --- | --- | --- | --- | --- |
| Spindle Speed | 900 | 900– 1460 | RPM | Controls drilling speed |
| Component Feed Rate (Forward) | 50 | 20–100 | mm/sec | Speed of component feeder servo moving forward |
| Component Feed Rate (Backward) | 50 | 20–100 | mm/sec | Speed of component feeder servo moving backward |
| Drill Head Descent Speed | 100 | 50–150 | mm/sec | Speed of drill head moving down toward component |
| Drill Head Ascent Speed | 150 | 50–150 | mm/sec | Speed of drill head retracting upward |
| Clamping Pressure (Target) | 40 | 35–50 | BAR | Hydraulic pressure for component clamping |
| Drilling Dwell Time | 4 | 2–8 | Seconds | Time spindle stays in contact with component drilling |
| Coolant Flow Rate | 30 | 15–30 | LPM | Lubrication system flow to spindle |
| Cycle Timeout | 60 | 30–120 | Seconds | Maximum time allowed for single cycle before abort |
| Buffer Minimum Level | 100 | 50–200 | Pieces | Minimum components required in loading tray |
| Pressure Relief Limit | 105 | 100– 110 | BAR | Maximum hydraulic pressure before safety relief valve opens |

### How to Change a Recipe Parameter:

- Press RECIPES button on HMI main menu

- Select Edit Mode

- Navigate to desired parameter using touch buttons

- Press INCREASE or DECREASE to adjust value

- Confirm new value (should show on screen)

- Press SAVE to store in PLC memory

- Press RUN to apply new settings on next cycle

ı. WARNING: Never adjust spindle speed above 1460 RPM or clamping pressure above 105 BAR without supervisor approval. These are safety limits set by machine design.

## Calibration & Zero-Point Adjustment

### Servo Zero-Point Calibration (Component Feeder)

When Required: If component feeder does not return to starting position (drifts), or if servo moves too far forward/backward

### Procedure (Requires Mitsubishi Servo Tool):

- Set machine to SETUP mode on HMI

- Move component feeder servo to fully retracted position (HOME position)

- Connect Mitsubishi servo diagnostic tool to PLC RS-232 port

- Run "Zero Calibration" routine in servo software

- Servo will move forward to mechanical stop, then return to zero position

- Confirm zero position on screen; save in PLC memory

- Test with manual jog commands; verify forward/backward movement is symmetric

### Drill Head Servo Calibration

When Required: If drill head does not stop at proper depth, or if servo is not providing smooth up/down motion

### Procedure:

- Set machine to SETUP mode

- Connect Mitsubishi servo tool to PLC

- Command drill head to move DOWN slowly; observe when tip reaches component

- Mark reference position on PLC display (zero point for drilling)

- Command drill head to move UP to fully retracted position

- Servo should move exactly same distance in both directions (verify on position display)

- Adjust scaling factor if asymmetric movement detected

- Save calibration; test with manual jog

### Poka-Yoke Sensor Threshold Adjustment

When Required: If sensors are falsely triggering errors, or missing legitimate faults

### For Proximity Sensors (Presence Detection):

- Access HMI SETUP → SENSOR CALIBRATION menu

- Place reference object (component) at sensor detection zone

- Press TEACH button; sensor "learns" object distance

- Threshold automatically set to trigger 10–20 mm before physical contact

- Test with actual component; verify green light only when component present

### For Pressure Sensors (Clamping):

- Pressurize hydraulic system to idle pressure (30 to 40 BAR)

- Access HMI SETUP → PRESSURE THRESHOLDS

- Record baseline pressure reading

- Adjust minimum clamp pressure threshold: Should be 30 BAR (80% of rated 70 BAR)

- Adjust maximum clamp pressure threshold: Should be 40 BAR (relief valve limit)

- System will alarm if pressure exceeds 60 BAR or falls below 30 BAR during clamp

- Save settings

# APPENDICES

## 10A. Electrical Wiring Diagram

### Main Electrical Components:

- 3-Phase with neutral 415V AC supply from plant distribution board

- Main isolator switch (80A capacity)

- Motor soft starter or VFD for spindle motor

- 24V DC emergency stop circuit (critical safety circuit)

- Emergency stop circuit

- Control transformer 415V AC

## 10B. Hydraulic System Schematic

### Key Components:

- Hydraulic pump (motor-driven)

- Main relief valve (40 BAR setting)

- Directional control solenoid valves

- Servo-controlled pressure reducing valves (pilot-operated)

- Pressure gauges (main and lubrication circuits)

- Tank with drain and filler plug

- Return line filter

## 10C. Pneumatic System Layout

### Key Components:

- Plant compressed air source (5–6 BAR)

- Line-mounted 5-micron filter

- Pressure regulator with gauge

- Manual isolation Shut-OFF valves

- Solenoid dump valve (for LOTO)

## 10D. Machine Dimensional Drawings

Reference Drawing Number: D/Meth/050/Truck Drilling/CH/01 Rev. 1 (Accurate Machines)

## 10E. PLC Program Flow Chart

PLC Controller: Mitsubishi FX5U - 80MTESS Scan Time: 10 ms (100 cycles per second) Backup Battery: CR2032 (RTC battery)

## 10F. Component Specifications & Tolerances

### Clutch Facing Dimension Reference (Per Valeo D/Meth/025 Rev. 12):

| Dimension | Mini | Maxi | Tolerance |
| --- | --- | --- | --- |
| Outer Diameter | 280 mm | 430 mm | ±0.5 mm |
| Thickness | 2.8 mm | 6.2 mm | ±0.2 mm |
| No. of Holes | 24 | 54 | Exact count |
| Hole Diameter | 7.75 mm | 14.5 mm | ±0.1 mm |
| Hole Position Runout | — | — | Max 0.8 mm |
| Rivet Hole Thickness | 0.9–2.1 mm | — | ±0.05 mm |

## 10G. Machine Operating Log Template

| Date | Shift | Opera tor Name | Start Time | End Time | Total Parts Produ ced | Rejec ted Parts | Def ect Cod e | Mainten ance Actions | Superv isor Sign |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## 10I. Machine Maintenance Log Template

| Date | Maintena nce Task | Durati on (mins) | Components Replaced/Ser viced | Technic ian Name | Parts Cost | Status (Complete/Pen ding) |
| --- | --- | --- | --- | --- | --- | --- |

## 10J. Accident/Incident Report Form

Machine Incident Report – Truck Drilling Machine Date & Time:

Location: 	 Operator Name: 			 Operator ID:

Incident Type (Circle):

- Minor Injury | Serious Injury | Equipment Damage | Near-Miss | Other

Description of Incident:

Root Cause (If Known):

Immediate Action Taken:

Witness Names & Signatures:

- Signature:

- Signature:

Supervisor Review & Comments:

Supervisor Signature: 	Date:

Machine Status After Incident (Circle):

  - Operational | Minor Repair Needed | Out of Service | Requires Safety Inspection

## 10K. Glossary of Terms

Poka-Yoke: Japanese term meaning "mistake-proof" or "fool-proof." Automatic error detection system that stops machine if abnormal condition detected.

Servo Motor: Precision electric motor that responds to PLC commands for precise position/velocity control (used for component feeder and drill head movement).

Solenoid Valve: Electromagnetic device that opens/closes fluid passages when energized (used for clamping, feeding, and air control).

MTBF (Mean Time Between Failures): Average operating time between equipment breakdowns (target: 8 hours minimum for this machine).

TRP (Total Run Percentage): Percentage of scheduled operating time that machine actually runs without stoppage (target: 85% minimum).

MTTR (Mean Time To Repair): Average time required to fix machine after failure (target: maximum 1 hour).

Proximity Sensor: Non-contact sensor that detects object presence/position using electromagnetic or optical technology.

LOTO (Lock-Out/Tag-Out): Safety procedure to isolate energy sources and prevent accidental machine restart during maintenance.

Runout: Amount of deviation from perfect concentricity; measured as radial distance from true center (tolerance: 0.8 mm maximum).

Eccentricity: Off-center condition of a circular component; detected by concentricity poka-yoke sensor.

## 10L. Emergency Contact Numbers

| Contact Type | Name | Mobile | Email |
| --- | --- | --- | --- |
| Manufacture r (Technical Support) |  |  |  |
| Maintenance Supervisor |  |  |  |
| Plant Safety Officer |  |  |  |
| Electrical Technician |  |  |  |
| Hydraulics Specialist |  |  |  |
| Plant Emergency Services |  |  |  |
| Poison Control / Medical |  |  |  |

## 10M. Training Certification Record

Employee Name: 			 Employee ID: 		 Department:

Training Date:

Trainer Name & Signature:

Training Topics Completed (Check):

  - [ ] Machine Safety & Hazard Awareness

  - [ ] Emergency Stop & LOTO Procedures

  - [ ] Pre-Start Verification Checklist (Section 6.1)

  - [ ] Normal Operating Cycle (Section 6.2)

  - [ ] Troubleshooting Common Issues (Section 8.1)

  - [ ] Maintenance Schedule & Procedures (Section 7)

  - [ ] Poka-Yoke Error Codes (Section 8.2)

  - [ ] Component Loading & Quality Verification

  - [ ] Equipment Shutdown & End-of-Shift Procedures

  - [ ] Hands-On Practical Training (Supervised)

Trainer Assessment: (Circle)

PASS – Employee demonstrates competency in all areas

CONDITIONAL PASS – Employee requires additional supervision in:

NOT READY – Further training required; retest scheduled for:

Trainer Signature: 	Date:

Plant Supervisor Sign-Off (Authorization to Operate Independently):

Date:

END OF MANUAL

For questions or clarifications, contact:

[PLACEHOLDER: Manufacturer support details]