OPERATING & SAFETY MANUAL

TILTING MACHINE

Clutch Plate Positioning & Tilting Station

Manufactured by: Karthick Engineering & Maintenance Consultants (KEMC) No 22/50, Manickavasagam Street, Kumaran Nagar, Padi, Chennai – 600050 End User: Valeo India Pvt. Ltd. Machine Safety Assessment: Independent Safety Consultant Document Version: 1.0 ¦ Date: February 2026 Safety Evaluation Date: 16 February 2026

READ THIS MANUAL BEFORE OPERATING THIS MACHINE Failure to follow safety instructions may result in serious injury or death.

# Document Control

| Document Title | Operating & Safety Manual – Tilting Machine |
| --- | --- |
| Document Number | KEMC-TM-OSM-001 |
| Version | 1.0 |
| Date of Issue | February 2026 |
| Prepared by | KEMC Technical Documentation Team |
| Reviewed by | KEMC Engineering |
| Approved by | KEMC Management / Valeo Safety Officer |
| Safety Assessment Reference | Third-Party Machine Safety Evaluation – 16/02/2026 |
| Next Review Date | February 2027 (or after any machine modification) |

## Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | February 2026 | KEMC Documentation | Initial release |

ℹ NOTE: This document is to be kept readily available at the machine workstation at all times. Copies must be provided to all personnel operating, maintaining, or supervising this machine.

# Section 1: Machine Identification

Figure 1.1: Tilting Machine – Front View

## 1.1 Machine Identification

| Machine Name | Tilting Machine |
| --- | --- |
| Manufacturer | Karthick Engineering & Maintenance Consultants (KEMC) |
| End User | Valeo India Pvt. Ltd. |
| Manufacturing Date | February 2026 |
| Machine Type | Assembly Machine – Clutch Plate Tilting Station |
| Safety Assessment Date | 16 February 2026 |

# Section 5: Safety Devices

## 5.1 Fixed Perimeter Guard

Figure 5.1: Fixed Mesh Perimeter Guard

| Guard Type | Fixed Mesh Guard |
| --- | --- |
| Height | 700 mm |
| Mesh Gap | 20 mm |
| Status | Implemented |

## 5.2 Light Curtain – Operator Access

Figure 5.2: Type 4 Safety Light Curtain

| Device Type | Type 4 Safety Light Curtain |
| --- | --- |
| Top Beam Height | 1400 mm |
| Bottom Beam Height | 900 mm |
| Resolution | 14 mm |

## 5.3 Control Panel & Cycle Start

Figure 5.3: TEKNIC Control Panel with START Button

⚠ OPEN ACTION – Safety Enhancement Recommended The safety assessment (dated 16/02/2026) recommends upgrading to a TWO-HAND CYCLE START CONTROL per ISO 13851. This would require simultaneous actuation of two separated buttons, ensuring both operator hands are clear before the cycle begins. Current Status: Single START button. Two-hand control PENDING. Until implemented, operator MUST ensure: (1) Both hands withdrawn. (2) Light curtain clear. (3) No personnel nearby. ONLY THEN press START.

## 5.4 Emergency Stop

Figure 5.4: Red Mushroom-Head E-Stop Button

## 5.5 Pneumatic Isolation

Figure 5.5: Blue Pneumatic Isolation Valve

## 5.6 Main Power Isolator

Figure 5.6: Main Power Isolator with LOTO Provision

# Section 6: Machine Components

## 6.1 Tilting Mechanism

Figure 6.1: Electric Motor with Worm Gearbox (Tilting Drive)

## 6.2 Pneumatic System

Figure 6.2: JANATICS FRL Unit with Pressure Gauge (5 bar)

## 6.3 Tilting Fixture

Figure 6.3: Tilting Fixture with Clamping Cylinders

## 6.4 HMI Display

Figure 6.4: HMI Display Panel

## 6.5 Safety Signage

Figure 6.5: ISO 7010 Safety Warning Labels