[LOGO PLACEHOLDER — Insert Valeo Friction Materials India Private Limited company logo here]

[LOGO PLACEHOLDER — Insert Independent Safety Consultant logo or branding here, if applicable]

OPERATING AND SAFETY MANUAL

Low Energy Test Bench

Clutch Endurance Testing Machine

| Field | Details |
| --- | --- |
| Customer | Valeo Friction Materials India Private Limited |
| Address | No. 16A, Sengundram Industrial Area, Melrosapuram (via Singaperumal Koil), Chengalpattu / Kancheepuram District, Chennai – 603 204, Tamil Nadu, India |
| Machine | Low Energy Test Bench — Clutch Endurance Test |
| Location | R&D Building, R&D Cell |
| Document No. | [Insert document number] |
| Revision | 00 |
| Date | March 2026 |
| Prepared by | Independent Safety Consultant — B11 LMSS Certified |
| Classification | Confidential — For internal use by authorised Valeo personnel only |

[IMAGE PLACEHOLDER — Insert front view photograph of the Low Energy Test Bench. Source: SICK India risk assessment report, Section 4.4, Page 12.]

# Table of Contents

Table of Contents	2

1  Document Information and Scope	5

1.1  Purpose of this manual	5

1.2  Scope of this manual	5

1.3  Applicable standards and regulations	5

1.4  Revision history	6

1.5  Disclaimer	6

1.6  Open actions — document level	6

2  Machine Identification	8

2.1  Machine data	8

2.2  Technical specifications	8

2.3  Test workpiece description	9

2.4  Environmental conditions	9

2.5  Control system overview	10

2.6  Safety devices fitted	10

3  Machine Description and Functional Overview	12

3.1  Machine description	12

3.2  Functional description — how the machine works	12

3.3  Operating modes	13

3.4  Machine layout and zoning	13

3.5  Process flow summary	13

3.6  Persons present at the machine	14

4  Control System and Software	15

4.1  Overview	15

4.2  Operator panel — button functions and layout	15

4.3  NI card digital input mapping	16

4.4  NI card digital output mapping	17

4.5  Analogue I/O mapping	17

4.6  Safety relay — Pilz PNOZ X3	17

4.7  MORPHEE software — system login procedure	18

4.8  MEditor — new test file creation procedure	18

4.9  MORPHEE — test launch procedure	19

4.10  MORPHEE — calibration verification	19

5  Pre-Start Verification Checklist	20

5.1  Purpose	20

5.2  Pre-start verification checklist	20

A — Electrical supply checks	20

B — Pneumatic supply checks	20

C — Exhaust and dust collection checks	20

D — Machine guarding and safety checks	21

E — Software and workstation checks	21

F — PPE and housekeeping checks	21

5.3  Pre-start sign-off	22

5.4  Abnormal conditions — do not start	22

6  Normal Operating Cycle Procedures	23

6.1  Overview	23

6.2  Cover plate preparation	23

6.3  Machine starting procedure	24

6.4  Clutch assembly loading procedure	24

6.5  Clutch displacement setting procedure	25

6.6  Test program creation and launch	26

6.6.1  Creating a new test file in MEditor	26

6.6.2  Launching the test in MORPHEE	26

6.7  Test monitoring	26

6.8  Inspection at 6,000 cycles	27

6.9  Tested clutch removal procedure	27

6.10  Machine stopping procedure	28

7  Safety Procedures	29

7.1  General safety rules	29

7.2  Personal protective equipment	29

7.3  Emergency stop procedure	29

In the event of an emergency:	30

To reset after E-stop:	30

7.4  Door interlock system	30

7.5  Lockout/Tagout (LOTO) procedure	30

7.5.1  Energy isolation points	31

7.5.2  Tag-out procedure (isolation — before maintenance)	31

7.5.3  Tag-in procedure (restoration — after maintenance)	31

7.6  Reaction to abnormalities	32

7.7  Hot zone and thermal hazards	32

7.8  Dust hazard	33

7.9  Emergency contacts	33

7.10  Accident and near-miss reporting	34

7.11  Training requirements	34

8  Alarms and Troubleshooting	35

8.1  Overview	35

8.2  Indicator lamp status guide	35

8.3  Alarm and fault reference table	35

8.4  VFD trip response procedure	36

8.5  Machine log requirements	37

9  Maintenance and Calibration	38

9.1  Maintenance overview	38

9.2  Maintenance schedule summary	38

9.3  Spare parts list	39

9.4  Dust collection system inspection	40

9.5  General housekeeping	40

9.6  Air filter replacement procedure	40

9.7  Calibration procedures	41

9.7.1  Torque calibration	41

9.7.2  Temperature calibration	42

9.7.3  RPM calibration	42

9.8  Third-party maintenance specialist	42

10  Commissioning and Decommissioning	44

10.1  Purpose	44

10.2  Commissioning checklist	44

10.3  First test run validation	45

10.4  Decommissioning and isolation procedure	45

10.5  Long-term shutdown procedure	46

10.6  Disposal of test components	46

Open Actions Register	47

Abbreviations	49

# 1  Document Information and Scope

## 1.1  Purpose of this manual

This Operating and Safety Manual has been prepared for the Low Energy Test Bench (LE Test Bench) installed in the R&D Building, R&D Cell at Valeo Friction Materials India Private Limited, Chennai. The manual serves two primary purposes: to provide clear, step-by-step operational guidance for personnel who work on or around the machine, and to serve as a formal safety compliance document that supports the organisation's obligations under applicable Indian industrial standards and international machine safety requirements.

The manual is intended for use by machine operators, maintenance personnel, plant engineers, and any other authorised persons who interact with the LE Test Bench. Visitors and third-party vendors who enter the machine area must also be made aware of the safety requirements described in this document.

## 1.2  Scope of this manual

This manual covers the following areas in relation to the Low Energy Test Bench:

- Machine identification and technical specifications, including all known equipment and system details. Where data is unavailable due to the retrofit nature of the machine, open actions are formally documented with placeholders pending customer verification.

- Control system and software description, including the operator panel layout, MORPHEE test management software, NI USB-6001 data acquisition system, variable frequency drive, and Pilz PNOZ X3 safety relay architecture.

- Pre-start verification procedures to be completed before each shift or test run.

- Normal operating cycle procedures covering cover plate preparation, clutch loading, displacement setting, test launch, monitoring, inspection at 6,000 cycles, clutch removal, and machine shutdown.

- Safety procedures including PPE requirements, emergency stop, door interlock operation, and full Lockout/Tagout (LOTO) procedure.

- Alarms and troubleshooting guidance for common fault conditions.

- Maintenance schedule and calibration procedures for torque, temperature, and RPM systems.

- Commissioning and decommissioning guidance.

This manual does not replace or supersede: the original equipment manufacturer's documentation (if available), Valeo's internal quality management system procedures, Valeo's EHS management system requirements, or statutory requirements under the Factories Act 1948 and applicable state regulations.

## 1.3  Applicable standards and regulations

| Standard / Regulation | Title | Relevance |
| --- | --- | --- |
| ISO 20607:2019 | Safety of machinery — Operator's handbook — General drafting principles | Manual structure and content requirements |
| ISO 12100:2010 | Safety of machinery — General principles for design — Risk assessment and risk reduction | Risk-based safety approach |
| IEC 82079-1:2019 | Preparation of instructions for use of products | Technical documentation quality |
| IS 4571:1985 | Code of practice for safety of machinery | Indian national standard for machine safety |
| IS 12100 | Safety of machinery — General principles for design | Aligned with ISO 12100 |
| Factories Act 1948 | Indian statutory requirement for safe workplaces | Legal obligation on employer and operators |
| ANSI B11 LMSS | Machine Safety Standards (USA) — Risk assessment methodology | Consultant's certification standard used in assessment |

## 1.4  Revision history

| Rev | Date | Description of change | Approved by |
| --- | --- | --- | --- |
| 00 | March 2026 | First issue — initial release | [Insert name] |

## 1.5  Disclaimer

This manual has been prepared by an Independent Safety Consultant who is B11 LMSS certified, based on information provided by Valeo Friction Materials India Private Limited and on-site observations made during the risk assessment and site review visits. The consultant has exercised professional care in preparing this document; however, the accuracy of all technical specifications, machine parameters, and process details is the responsibility of Valeo Friction Materials India Private Limited.

Where data was unavailable at the time of preparation, open actions have been formally documented with placeholders. Valeo Friction Materials India Private Limited is responsible for closing all open actions and verifying all placeholder data before this manual is considered complete and ready for unrestricted distribution.

This document must not be reproduced, distributed, or disclosed to third parties without the written consent of Valeo Friction Materials India Private Limited. This document is classified as confidential.

## 1.6  Open actions — document level

OPEN ACTION OA-007: Customer to confirm the document number format and assign a formal document number to this manual in accordance with Valeo's document control procedure. Responsible party: Valeo Friction Materials India Private Limited. Target close date: [Insert].

# 2  Machine Identification

## 2.1  Machine data

| Field | Details |
| --- | --- |
| Machine name | Low Energy Test Bench — Clutch Endurance Test |
| Machine type | Testing machine (automated endurance test bench) |
| Manufacturer | Not available — retrofit machine relocated from another Valeo facility |
| Model number | Not available — no nameplate present |
| Serial number | Not available — no nameplate present |
| Asset number | Not available — pending Valeo asset register confirmation |
| Manufacturing date | 8 August 2025 (re-commissioning date at current location) |
| Control panel manufacturer | New Craft Technology, Job No. 1639, Rev R0, December 2025 |
| Asset location — line | R&D Cell |
| Asset location — building | R&D Building |
| Factory | Valeo Friction Materials India Private Limited |
| Street address | No. 16A, Sengundram Industrial Area, Melrosapuram (via Singaperumal Koil) |
| District | Chengalpattu / Kancheepuram District |
| City | Chennai |
| State | Tamil Nadu |
| Country | India |
| PIN code | 603 204 |

OPEN ACTION OA-001: Machine nameplate is not available. The LE Test Bench is a retrofit machine relocated from another Valeo location. Serial number, model number, and original manufacturer details are not confirmed. Valeo Friction Materials India Private Limited is responsible for providing or verifying these details. This open action must be closed before the manual is considered complete.

## 2.2  Technical specifications

| Parameter | Specification |
| --- | --- |
| Drive motor type | AC induction motor controlled by variable frequency drive (VFD) |
| Drive motor rating (kW) | [Insert — to be confirmed from motor nameplate] — refer Open Action OA-002 |
| Gear motor (bed traverse) | [Insert — to be confirmed from motor nameplate] — refer Open Action OA-002 |
| Main power supply | 415V AC, 3-phase, 50 Hz |
| Control supply voltage | 24V DC (SMPS — switch mode power supply) |
| Compressed air supply | 4 to 6 bar (regulated) |
| Machine footprint (approx.) | 3 m × 2.5 m |
| Test cycle clamping force range | 450 to 500 DaN |
| Maximum test speed | [Insert — refer VFD parameter settings] |
| Total installed electrical load | [Insert — to be confirmed from panel schedule] |
| Protection class — control panel | IP54 (minimum) |

OPEN ACTION OA-002: Main drive motor electrical ratings (kW, voltage, full load current, RPM) and gear motor ratings are not available in any provided documentation. These must be verified from motor nameplates or electrical panel records by Valeo Friction Materials India Private Limited. Responsible party: Valeo Friction Materials India Private Limited. Target close date: [Insert].

## 2.3  Test workpiece description

The LE Test Bench is designed to test automotive clutch assemblies. Each test assembly consists of three primary components:

- Flywheel: 17 mm, fitted to the machine adapter using an M5 bolt

- Sim washer: 7.7 mm, assembled between the flywheel and cover plate

- Cover plate assembly: bolted to the flywheel and sim washer using M8 bolts, with thermocouple installed prior to testing

The friction disc is manufactured from impregnated yarn made of glass fibre infused with metal. Test variants include multiple clutch assembly configurations used in Valeo's R&D programme.

The cover plate preparation process requires heating the assembly in an oven at 300°C for 1.5 hours prior to testing. A thermocouple is installed in the cover plate by drilling a 4.8 mm hole at the centre surface and at the outer punched rivet location for temperature monitoring during the test.

## 2.4  Environmental conditions

| Condition | Requirement |
| --- | --- |
| Ambient temperature | 20°C to 35°C (stable room temperature recommended) |
| Relative humidity | 40% to 70% RH, non-condensing |
| Ventilation | Adequate air circulation required; forced exhaust ventilation mandatory for high-temperature operations |
| Dust control | Dust-free environment; avoid corrosive gases or oil mist |
| Power supply quality | Stable 3-phase supply with proper earthing; surge protection device recommended (refer Open Action OA-004) |
| Noise | Acoustic enclosure recommended if noise levels exceed 80 dB during operation |

## 2.5  Control system overview

The control system of the Low Energy Test Bench was designed and installed by New Craft Technology (drawing reference: NT1639, Rev R0, December 2025). The system comprises:

- Two National Instruments USB-6001 DAQ controllers (NI Card 1 and NI Card 2) as the primary I/O platform

- MORPHEE test management software running on a dedicated Windows XP workstation

- Variable frequency drive (VFD) for main motor speed control

- Pilz PNOZ X3 dual-channel safety relay for E-stop and door interlock monitoring

- Siemens operator panel with push buttons and clamp torque display

- Three-phase gear motor with K1/K2 contactors for clutch bed traverse

## 2.6  Safety devices fitted

| Safety device | Description and location |
| --- | --- |
| E-stop Button 1 | Operator panel — front of machine |
| E-stop Button 2 | Left side of machine enclosure |
| E-stop Button 3 | Cabin workstation area |
| Door interlock — Door 1 | Main access door — solenoid lock, PNOZ monitored |
| Door interlock — Door 2 | Side access door — solenoid lock, PNOZ monitored |
| Door interlock — Door 3 | Rear access door — solenoid lock, PNOZ monitored |
| Pilz PNOZ X3 safety relay | Dual-channel E-stop and door interlock monitoring — control panel |
| Guard G001 | Fixed mesh guard — drive end |
| Guard G002 | Fixed mesh guard — non-drive end |
| Guard G003 | Fixed mesh guard — top |
| Guard G004 (acrylic) | Busbar protection guard — refer Open Action OA-003 |
| Air pressure sensor | Monitors pneumatic supply — low pressure triggers fault |
| Proximity sensor | Detects clutch assembly position |
| Limit switch | Bed travel limit — prevents over-travel of clutch assembly bed |

OPEN ACTION OA-003: Busbar acrylic guard G004 dimensions were recorded as 'xxx mm' in the SICK India risk assessment report. Fabrication status and actual dimensions are not confirmed. This guard must be confirmed as fabricated and installed at commissioning. Responsible party: Valeo Friction Materials India Private Limited / New Craft Technology. Target close date: [Insert].

OPEN ACTION OA-004: Overvoltage surge protective device was recommended in the SICK India risk assessment report but has not been confirmed as installed. Valeo Friction Materials India Private Limited must confirm installation status or formally accept the risk of not installing this device. Responsible party: Valeo Friction Materials India Private Limited. Target close date: [Insert].

# 3  Machine Description and Functional Overview

## 3.1  Machine description

The Low Energy Test Bench (LE Test Bench) is an automated endurance testing machine used exclusively within the Research and Development Cell at Valeo Friction Materials India Private Limited, Chennai. The machine is designed to simulate the repeated engage and disengage cycles experienced by automotive clutch assemblies during their operational life, allowing Valeo's R&D team to evaluate the durability, wear characteristics, torque performance, and thermal behaviour of clutch components under controlled laboratory conditions.

The machine is a retrofit unit relocated from another Valeo facility. The original manufacturer and nameplate data are not available (refer Open Action OA-001). The control system was fully rebuilt by New Craft Technology in December 2025 and is based on a National Instruments DAQ platform with MORPHEE test management software running on a dedicated Windows XP workstation.

The machine occupies approximately 3 m × 2.5 m of floor space and is installed within a dedicated R&D cell alongside the EM-1 Bench, oven, working table, and measuring table. The operator workstation is physically separated from the machine zone by a desk partition, which limits the operator's exposure to noise and airborne dust generated during testing.

## 3.2  Functional description — how the machine works

The LE Test Bench performs clutch endurance testing through the following sequence of operations.

Before testing begins, the cover plate and flywheel assembly is prepared by bolting together a 17 mm flywheel with a 7.7 mm sim washer using M8 bolts, then placing the assembly in an oven at 300°C for 1.5 hours. A thermocouple is fitted into the cover plate by drilling a 4.8 mm hole at the centre surface and at the outer punched rivet location. This allows continuous temperature monitoring during the test cycle.

The prepared clutch assembly — consisting of the flywheel, friction disc, and cover plate — is then loaded onto the test bed by the operator in manual mode. The flywheel is fitted to the adapter using an M5 bolt. The friction disc is aligned using the M8 Aline key bolt and centre shaft spline alignment, and the cover plate is tightened down. The clutch assembly bed is moved forward using the lead screw rotation wheel until the drive shaft is connected. The thermocouple plug is connected, and the bed is secured using the M10 slinckey fixing bolt. The operator then checks clutch displacement using the clamp and de-clamp function, verifying that the displayed clamping force falls within the required range of 450 to 500 DaN.

Once the clutch is loaded and displacement is verified, the operator creates or selects a test program file using MEditor software, then launches the test via MORPHEE software. The test runs automatically, executing repeated engage and disengage cycles while the system continuously records torque, temperature, and RPM data. The operator monitors the process from the workstation cabin, checking filter operation and MORPHEE display readings at the start and end of each shift.

At 6,000 cycles, the test is paused. The friction disc is removed, cleaned using compressed air, and inspected for wear depth. Surface bedding percentage is assessed visually on both faces. If the friction disc meets the acceptance criteria, it is reassembled and the test continues. Scrap components are handed to the EHS team for disposal in accordance with Valeo waste handling guidelines.

Post-test, the clutch assembly is fully removed. Tested components are then sent to the quality inspection station where cover assembly clamping load, friction disc thickness, flywheel and disc roughness (Ra), flatness, concavity, and friction disc depth are all measured and recorded.

## 3.3  Operating modes

The machine operates in two modes.

In Manual Mode, the operator directly controls individual machine functions using the Siemens operator panel push buttons. This mode is used during clutch loading, displacement setting, and any maintenance or inspection task that requires door access. In manual mode, the door interlocks can be overridden by the designated Operator 2 using the USER PW access level on the MORPHEE system, allowing the door to be opened while the machine is not running. The operator must exercise particular caution in manual mode as cylinder movement is still possible when the door is open.

In Automatic Mode, the machine executes the selected test program independently without operator intervention. All door interlocks are active in automatic mode and the machine will not operate unless all three doors are confirmed closed and locked by the Pilz PNOZ X3 safety relay chain. The operator monitors the test from the cabin workstation.

## 3.4  Machine layout and zoning

[IMAGE PLACEHOLDER — Insert machine layout drawing showing R&D cell arrangement including LE Test Bench, EM-1 Bench, oven, working table, measuring table, and operator cabin. Source: Excel data collection sheet, Layout tab.]

[IMAGE PLACEHOLDER — Insert front isometric view photograph of the Low Energy Test Bench showing the machine enclosure, operator panel, and cabin workstation. Source: SICK India risk assessment, Section 4.4, Page 12.]

## 3.5  Process flow summary

| Step | Activity |
| --- | --- |
| 1 | Cover plate preparation — oven heating at 300°C for 1.5 hours, thermocouple fitting |
| 2 | Clutch assembly loading onto test bed in Manual mode |
| 3 | Clutch displacement setting — verify 450 to 500 DaN clamping force |
| 4 | Test program creation in MEditor (if new file) or selection of existing file |
| 5 | Test launch via MORPHEE in Automatic mode |
| 6 | Continuous automatic test execution — repeated engage/disengage cycles |
| 7 | Shift monitoring — operator checks filter and MORPHEE readings |
| 8 | Inspection at 6,000 cycles — friction disc removal, cleaning, wear measurement |
| 9 | Reassembly and continuation, or clutch removal if worn beyond acceptance criteria |
| 10 | Post-test removal of clutch assembly and transfer to quality inspection station |

## 3.6  Persons present at the machine

| Role | Responsibilities |
| --- | --- |
| Operator 1 | Test monitoring from cabin workstation during automatic mode. Shift log sign-off. Basic pre-start checks. |
| Operator 2 | All manual mode activities — clutch loading, displacement setting, inspection at 6,000 cycles, clutch removal. Holds the key switch. Must be present for all door access activities. |
| Maintenance Person | Technical maintenance activities under LOTO. Calibration coordination. Panel access. |
| EHS Lead | Safety oversight. LOTO permit authority. Work permit issuance for third-party specialists. |
| R&D Deputy Manager | Technical authority. Commissioning sign-off. Test specification approval. |
| Third-party Specialist (New Craft Technology) | Electrical and control system maintenance and troubleshooting under work permit. |

# 4  Control System and Software

## 4.1  Overview

The control system of the Low Energy Test Bench was designed and installed by New Craft Technology in December 2025 (drawing reference: NT1639-NEW CRAFT WIRING, Rev R0, Job No. 1639, 20/12/2025). The system uses two National Instruments USB-6001 data acquisition controllers as the primary I/O platform, with MORPHEE test management software running on a dedicated Windows XP workstation as the test execution and data recording interface.

The control architecture separates machine safety functions from test execution functions. Safety is handled entirely in hardware through the Pilz PNOZ X3 dual-channel safety relay, which monitors the emergency stop chain and door interlock status independently of the software. Test execution, motor control, and data acquisition are managed through the NI DAQ system and MORPHEE software.

[IMAGE PLACEHOLDER — Insert PLC control panel general arrangement drawing. Source: NT1639-NEW CRAFT WIRING, Rev R0, Page 02. Shows panel layout with NI cards, VFD, relays, SMPS units, MCBs, and terminal blocks.]

## 4.2  Operator panel — button functions and layout

[IMAGE PLACEHOLDER — Insert annotated control panel photograph. Source: Excel Testing sheet. Shows Siemens OP panel with all buttons and clamp torque display labelled.]

| Button / Display | Label | Function | Mode |
| --- | --- | --- | --- |
| Red mushroom-head push button | E-STOP (ARRET) | Emergency stop — immediately de-energises control circuit via Pilz PNOZ X3. Twist to release. | All modes |
| Illuminated push button (white) | RESET | Resets the safety relay after E-stop or fault. Press only after confirming machine and area are safe. | All modes |
| Key switch | C | Selects Auto or Manual operating mode. Key held by Operator 2 at all times. | All modes |
| Push button | F1 | Manual rotation of the clutch (hold-to-run). Used during displacement setting. | Manual only |
| Push button | F2 | Clutch engage and disengage. Press to engage, press again to disengage. | Manual only |
| Push button | F3 | Clutch displacement forward — moves the clutch bed forward. | Manual only |
| Push button | K3 | Clutch displacement reverse — moves the clutch bed in reverse. | Manual only |
| Push button | F5 | Clutch displacement position zero restart — resets the position reference to zero. | Manual only |
| Push button | F7 | Position Auto Set — sets the automatic position reference for test execution. | Manual only |
| Digital display | Clamp Torque Display | Shows current clamping force in DaN. Target: 450 to 500 DaN. | All modes |

## 4.3  NI card digital input mapping

NI Card 1 handles 24 digital inputs across three ports (Port 0, Port 1, Port 2).

| NI Card | Port / Channel | Signal name | Description |
| --- | --- | --- | --- |
| NI Card 1 | Port 0 / DI 0.0 | Control ON | Control circuit ON push button input |
| NI Card 1 | Port 0 / DI 0.1 | VFD Trip | VFD fault / trip status signal |
| NI Card 1 | Port 0 / DI 0.2 | Gear Motor MPCB Trip | Gear motor protection circuit breaker trip signal |
| NI Card 1 | Port 0 / DI 0.3 | Safety Door Lock 1 | Door 1 interlock solenoid lock status |
| NI Card 1 | Port 0 / DI 0.4 | Safety Door Lock 2 | Door 2 interlock solenoid lock status |
| NI Card 1 | Port 0 / DI 0.5 | Safety Door Lock 3 | Door 3 interlock solenoid lock status |
| NI Card 1 | Port 1 / DI 1.0 | Engage PB | Engage push button — operator panel |
| NI Card 1 | Port 1 / DI 1.1 | Disengage PB | Disengage push button — operator panel |
| NI Card 1 | Port 1 / DI 1.2 | Auto Mode Selector | Auto/Manual mode key switch input |
| NI Card 1 | Port 1 / DI 1.3 | E-Stop Chain | E-stop chain status via PNOZ X3 output |
| NI Card 1 | Port 1 / DI 1.4 | Proximity Sensor | Clutch assembly position detection |
| NI Card 1 | Port 1 / DI 1.5 | Limit Switch | Bed travel limit switch — prevents over-travel |
| NI Card 1 | Port 2 / DI 2.0 | Air Pressure Sensor | Pneumatic supply pressure monitoring (4–6 bar) |
| NI Card 1 | Port 2 / DI 2.1 | Main Motor ON Selector | Main motor enable selector switch |
| NI Card 1 | Port 2 / DI 2.2 | Blower Selector | Dust collection blower selector switch |

## 4.4  NI card digital output mapping

| NI Card | Channel | Signal name | Description |
| --- | --- | --- | --- |
| NI Card 2 | DO 0.0 | Motor Forward | Main motor forward run command to VFD |
| NI Card 2 | DO 0.1 | Motor Reverse | Main motor reverse run command (where applicable) |
| NI Card 2 | DO 0.2 | Engage Output | Engage actuator solenoid output |
| NI Card 2 | DO 0.3 | Disengage Output | Disengage actuator solenoid output |
| NI Card 2 | DO 0.4 | Blower Control | Dust collection blower motor control output |
| NI Card 2 | DO 0.5 | Indicator Lamp — Red | Red indicator lamp — fault / E-stop active |
| NI Card 2 | DO 0.6 | Indicator Lamp — Yellow | Yellow indicator lamp — auto mode / warning |
| NI Card 2 | DO 0.7 | Indicator Lamp — Green | Green indicator lamp — control circuit healthy |
| NI Card 2 | DO 1.0 | VFD Run Command | VFD enable / run command |
| NI Card 2 | DO 1.1 | Door Lock Coil | Solenoid door lock energise output |
| NI Card 2 | DO 1.2 | VFD Speed Reference (Analogue) | Analogue output — motor speed setpoint to VFD |

## 4.5  Analogue I/O mapping

| Channel | Signal | Description |
| --- | --- | --- |
| NI Card 2 — AI 0 | Torque (Load cell) | Analogue input from torque load cell — 0 to 10V or 4 to 20 mA (confirm with calibration record) |
| NI Card 2 — AI 1 | VFD Speed Feedback | Analogue feedback from VFD — actual motor speed |
| NI Card 2 — AI 2 | Temperature Channel 1 | Thermocouple input — cover plate centre surface |
| NI Card 2 — AI 3 | Temperature Channel 2 | Thermocouple input — outer punched rivet location |
| NI Card 2 — AO 0 | VFD Speed Reference | Analogue output — motor speed setpoint from NI card to VFD |

## 4.6  Safety relay — Pilz PNOZ X3

The Pilz PNOZ X3 is a dual-channel safety relay that provides independent monitoring of the emergency stop chain and machine door interlock status. The relay is wired as follows:

- Three emergency stop buttons are wired in series across both input channels (CH.1 and CH.2). If any single E-stop button is pressed, both channels open simultaneously, cutting power to the control relay chain.

- The safety relay output feeds into the MY3N relay chain, which controls the control ON circuit, the main motor enable, and the door lock coil.

- The RESET button is wired to the safety relay restart input. The relay will only reset if all E-stop buttons are fully released (twisted out) and all door interlocks are confirmed closed and locked.

- If the safety relay fails to reset after all conditions are met, the Pilz PNOZ X3 unit must be inspected by the maintenance person. Do not attempt to bypass the safety relay under any circumstances.

## 4.7  MORPHEE software — system login procedure

MORPHEE is the test management software used to launch, monitor, and record all clutch endurance tests. The following procedure must be followed each time the MORPHEE software is started.

[IMAGE PLACEHOLDER — Insert screenshot of MORPHEE login screen showing User Name and Password fields. Source: Excel Testing sheet, Image 8.]

- Switch on the Windows XP workstation in the cabin. Wait for the desktop to load fully.

- Double-click the MORPHEE icon on the desktop to launch the software.

- At the login screen, enter the appropriate User Name and Password for the required access level. Two access levels are available: USER PW (Operator 2 — full manual mode and door override access) and OBSERVER PW (Operator 1 — monitoring and display only).

- After login, the MORPHEE main screen will display. Confirm that all channel readings (torque, temperature, RPM) are displaying values and not showing error codes.

- If MORPHEE displays an error on startup, do not proceed. Record the error message and refer to Section 8.6 for MORPHEE error response procedure.

## 4.8  MEditor — new test file creation procedure

MEditor is used to create new test program files before launching MORPHEE. If an existing test file is being used, this procedure can be skipped.

[IMAGE PLACEHOLDER — Insert screenshot of MEditor file creation screen showing folder structure and file parameters. Source: Excel Testing sheet, Image 9.]

- On the Windows desktop, double-click the MEditor icon.

- In MEditor, open the required folder and select the reference file to base the new test on.

- Save the reference file with a new file name using the Save As function. Use the naming convention approved by the R&D team.

- In the new file, modify the required test parameters — number of cycles, speed setpoint, engage/disengage timing, temperature limit, and torque limit — as per the test specification.

- Save the file. Close MEditor.

- The new file is now ready to be selected in MORPHEE for test launch.

## 4.9  MORPHEE — test launch procedure

The following procedure must be followed after the pre-start checklist (Section 5) and clutch loading and displacement setting (Sections 6.4 and 6.5) have been completed.

[IMAGE PLACEHOLDER — Insert screenshot of MORPHEE main screen showing test file selection and Validation button. Source: Excel Testing sheet, Image 13.]

- In MORPHEE, navigate to the test file folder and select the required test program file.

- Confirm that the mode key switch (C) on the operator panel is in the Auto position.

- Confirm all three machine doors are closed and locked — the door interlock indicators on the MORPHEE screen must show all doors as locked.

- Press the Validation button in MORPHEE to initiate the pre-launch self-check.

- MORPHEE will confirm all input conditions are met. If any condition is not met, an error will be displayed — resolve before proceeding.

- Once validation is confirmed, press the Start button. The test will start automatically and MORPHEE will begin recording torque, temperature, and RPM data.

## 4.10  MORPHEE — calibration verification

Before conducting a new test, or as part of the periodic calibration schedule, the MORPHEE system calibration must be verified. Calibration procedures are described in Section 9.7. Calibration monitoring screens are accessed as follows.

- Open the MORPHEE file from the desktop.

- Click on Moniteurs (Monitors) in the main menu.

- Select Consignes (Setpoints) file.

- The Temperature and Torque calibration screen will display current values. Enter the manual load and temperature values and verify that the MORPHEE display matches the reference instruments.

- Enter the manual RPM in the CVM field. Verify the MORPHEE display against the tachometer reading.

[IMAGE PLACEHOLDER — Insert screenshots of MORPHEE calibration screens showing Moniteurs, Consignes, and calibration data entry fields. Source: Excel Quality Instruction sheet, calibration procedure images.]

# 5  Pre-Start Verification Checklist

## 5.1  Purpose

The pre-start verification checklist must be completed by the authorised operator before every test run and at the start of every shift. Its purpose is to confirm that the machine, utilities, and safety systems are in a known safe and ready state before any test program is launched. No test program may be started until all checklist items have been verified and any deficiencies resolved.

If any item on this checklist cannot be confirmed as satisfactory, the operator must not start the machine. The deficiency must be reported to the maintenance team and recorded in the machine log before any further action is taken.

## 5.2  Pre-start verification checklist

### A — Electrical supply checks

| Item | Check description | Verified | Initials |
| --- | --- | --- | --- |
| A1 | Confirm main power panel SSB-02 is switched ON and the Low Energy Bench supply is energised | ☐ |  |
| A2 | Confirm the machine power panel rotary isolator is in the ON position | ☐ |  |
| A3 | Confirm control voltage 24V DC is available — green power indicator lamp on panel is illuminated | ☐ |  |
| A4 | Confirm no MCB or MPCB in the control panel is in the tripped position | ☐ |  |
| A5 | Confirm the electrical cabinet door is fully closed and latched | ☐ |  |

### B — Pneumatic supply checks

| Item | Check description | Verified | Initials |
| --- | --- | --- | --- |
| B1 | Confirm the compressed air supply line valve is open | ☐ |  |
| B2 | Confirm air pressure at the regulator/filter unit reads between 4 and 6 bar | ☐ |  |
| B3 | Confirm the pneumatic filter condition — no visible water or contamination in the filter bowl | ☐ |  |
| B4 | Confirm the pneumatic LOTO valve is in the open/run position (not isolated) | ☐ |  |

### C — Exhaust and dust collection checks

| Item | Check description | Verified | Initials |
| --- | --- | --- | --- |
| C1 | Confirm the exhaust line motor starter is switched ON — green button depressed | ☐ |  |
| C2 | Confirm the dust collection exhaust system is running and airflow is felt at the exhaust outlet | ☐ |  |
| C3 | Confirm the air filter is clean and not overdue for replacement (refer maintenance schedule) | ☐ |  |

### D — Machine guarding and safety checks

| Item | Check description | Verified | Initials |
| --- | --- | --- | --- |
| D1 | Confirm all fixed guards (G001, G002, G003) are in position and securely fastened | ☐ |  |
| D2 | Confirm Guard G004 (acrylic busbar guard) is in position — refer Open Action OA-003 | ☐ |  |
| D3 | Confirm all three machine doors are closed and door interlock solenoids are engaged | ☐ |  |
| D4 | Test all three E-stop buttons — press each in turn, confirm red lamp illuminates, then twist to release and reset | ☐ |  |
| D5 | Confirm the Pilz PNOZ X3 safety relay has been successfully reset — all channel LEDs are illuminated | ☐ |  |
| D6 | Confirm no persons are inside the machine enclosure or hazard zone | ☐ |  |

### E — Software and workstation checks

| Item | Check description | Verified | Initials |
| --- | --- | --- | --- |
| E1 | Confirm the Windows XP workstation is switched on and desktop is visible | ☐ |  |
| E2 | Confirm MORPHEE software has been started and is not displaying any startup error | ☐ |  |
| E3 | Confirm all MORPHEE channel readings (torque, temperature, RPM) are displaying values without error codes | ☐ |  |
| E4 | Confirm the required test program file is selected in MORPHEE | ☐ |  |

### F — PPE and housekeeping checks

| Item | Check description | Verified | Initials |
| --- | --- | --- | --- |
| F1 | Confirm all PPE required for this shift is available and in serviceable condition (safety glasses, hearing protection, dust mask, heat-resistant gloves, safety shoes) | ☐ |  |
| F2 | Confirm the machine area is free from loose tools, components, or obstructions in the 1 m clearance zone | ☐ |  |
| F3 | Confirm the machine log is available at the workstation and previous shift entry is complete | ☐ |  |

## 5.3  Pre-start sign-off

After completing all items above, the operator must sign off the pre-start checklist in the machine log before starting the machine.

| Field | Entry | Field | Entry |
| --- | --- | --- | --- |
| Date |  | Shift |  |
| Operator name |  | Operator signature |  |
| All items satisfactory? | Yes / No | Deficiencies found and actions taken |  |

## 5.4  Abnormal conditions — do not start

The machine must not be started under any of the following conditions. If any of these conditions are observed, the operator must immediately inform the maintenance team.

- Any E-stop button is latched in the depressed position and cannot be released by twisting

- Any door interlock is not engaging or a door cannot be confirmed as locked

- Air pressure is below 4 bar or above 6 bar and cannot be adjusted to the correct range

- Any MCB or MPCB in the control panel is in the tripped position

- The MORPHEE system is displaying a fault or error that has not been cleared

- Any guard or shield is missing, damaged, or not in the correct position

- The Pilz PNOZ X3 safety relay is not showing all channel indicators as healthy after reset

- There is visible smoke, burning smell, oil leakage, or any other abnormal condition at or near the machine

# 6  Normal Operating Cycle Procedures

## 6.1  Overview

This section describes the complete normal operating cycle for the Low Energy Test Bench from machine start-up through to shutdown. The procedures must be followed in the sequence shown. All pre-start checks in Section 5 must be completed and signed off before beginning any procedure in this section.

All clutch loading, displacement setting, and manual mode operations must be performed by Operator 2 only. Test program monitoring may be performed by Operator 1 under supervision. Operator 2 must be present for all manual mode door access activities.

## 6.2  Cover plate preparation

Cover plate preparation must be completed before the clutch assembly is loaded onto the test bed. This procedure is performed at the oven and measuring table, not at the test bench itself.

⚠ WARNING: The oven operates at 300°C. Use heat-resistant gloves when handling components from the oven. Do not touch the cover plate or flywheel assembly with bare hands after removal from the oven. Allow components to cool to a safe handling temperature before fitting the thermocouple.

- Take the flywheel (17 mm) and the sim washer (7.7 mm) and assemble them together with the cover plate assembly using M8 mm bolts.

[IMAGE PLACEHOLDER — Insert photograph showing flywheel, sim washer, and cover plate assembly with M8 bolts. Source: Excel Testing sheet, cover plate preparation image 1.]

- Place the assembled cover plate and flywheel assembly into the oven. Set oven temperature to 300°C and heat for 1.5 hours.

[IMAGE PLACEHOLDER — Insert photograph showing assembled components placed in oven. Source: Excel Testing sheet, cover plate preparation image 2.]

- While the assembly is in the oven, prepare the thermocouple for installation. Drill a 4.8 mm hole in the cover plate at the following two locations: at the centre surface of the cover plate, and at the outer punched rivet of the cover plate.

[IMAGE PLACEHOLDER — Insert photograph showing thermocouple hole drilling locations on cover plate. Source: Excel Testing sheet, cover plate preparation image 3.]

- After the oven cycle is complete, remove the assembly using heat-resistant gloves. Allow to cool to safe handling temperature.

- Fit the thermocouple into the drilled holes as per the approved installation photograph. Ensure the thermocouple is seated correctly in both locations.

[IMAGE PLACEHOLDER — Insert photograph showing thermocouple correctly installed in cover plate — centre surface and outer punched rivet locations. Source: Excel Testing sheet, cover plate preparation image 4.]

- The cover plate assembly is now ready for clutch loading onto the test bench.

## 6.3  Machine starting procedure

- Confirm all pre-start checks in Section 5 are complete and signed off.

- At the main distribution panel (SSB-02), confirm the Low Energy Bench supply breaker is in the ON position.

- At the machine control panel, turn the rotary isolator to the ON position.

- Release all three E-stop buttons by twisting clockwise until they pop out. Confirm all three are fully released.

- Press the RESET button on the operator panel. The Pilz PNOZ X3 safety relay will reset. Confirm the green indicator lamp illuminates.

- Press the Control ON button. The control circuit will energise. The green lamp will remain illuminated.

- At the exhaust line panel, confirm the motor starter is ON (green button depressed). Confirm airflow from the dust collection exhaust outlet.

- On the cabin workstation, confirm MORPHEE is running and all channel readings are healthy.

- The machine is now ready for clutch loading. Proceed to Section 6.4.

## 6.4  Clutch assembly loading procedure

⚠ WARNING: Clutch loading must be performed in Manual mode only. Confirm the mode key switch (C) is in the Manual position before opening any door. Operator 2 must hold the key at all times.

- Confirm the mode key switch (C) on the operator panel is in the Manual position.

- Open the machine access door. Confirm the door interlock solenoid releases to permit door opening.

- Place the friction disc onto the drive shaft, aligning the disc using the M8 Aline key bolt and centre shaft spline alignment.

[IMAGE PLACEHOLDER — Insert photograph showing friction disc loaded onto drive shaft with alignment key. Source: Excel Testing sheet, clutch setting procedure image 1.]

- Fit the flywheel to the machine adapter using the M5 bolt. Tighten to the specified torque.

[IMAGE PLACEHOLDER — Insert photograph showing flywheel fitted to adapter. Source: Excel Testing sheet, clutch setting procedure image 2.]

- Position the cover plate assembly over the friction disc. Tighten down the cover plate using M8 bolts in a cross-pattern sequence.

[IMAGE PLACEHOLDER — Insert photograph showing cover plate being tightened down. Source: Excel Testing sheet, clutch setting procedure image 3.]

- Using the lead screw rotation wheel, move the clutch assembly bed forward until the drive shaft is fully connected.

- Connect the thermocouple plug to the thermocouple installed in the cover plate.

- Tighten the bed M10 slinckey fixing bolt to secure the clutch assembly bed in position.

[IMAGE PLACEHOLDER — Insert photograph showing clutch assembly bed, thermocouple plug location, and bed fixing bolt. Source: Excel Testing sheet, clutch setting procedure image — Clutch Assembly, Thermocouple Plug, and Bed Fixing Bolt labelled.]

- Close the machine door. Confirm the door interlock engages — the solenoid lock must be confirmed as active before proceeding.

- Proceed to Section 6.5 for clutch displacement setting.

## 6.5  Clutch displacement setting procedure

The clutch displacement must be verified and set to the correct range of 450 to 500 DaN before the test can be launched. This procedure uses the Siemens operator panel controls in Manual mode.

[IMAGE PLACEHOLDER — Insert annotated photograph of Siemens operator panel with all button labels: F1, F2, F3, K3, F5, F7, C mode key, Reset, E-stop, and Clamp Torque display. Source: Excel Testing sheet, clutch displacement setting procedure image.]

[IMAGE PLACEHOLDER — Insert clutch displacement setting flowchart. Source: Excel Testing sheet, displacement setting flowchart image.]

- Confirm the mode key switch (C) is in Manual mode.

- Press F2 to disengage the clutch.

- Press and hold F1 to manually rotate the friction disc.

- Press F2 to engage the clutch.

- Check the clamp torque display on the operator panel.

Interpret the display reading as follows:

- If the displayed value is less than 450 DaN: Press the F3 key to move the clutch displacement forward. Return to Step 3 and repeat.

- If the displayed value is between 450 and 500 DaN: The displacement is within the acceptable range. Proceed to Step 6.

- If the displayed value is greater than 500 DaN: Press the K3 key to move the clutch displacement in reverse. Return to Step 3 and repeat.

- Once the displacement reads 450 to 500 DaN, press F5 to set the position to zero (Position Reset).

- Press K3 to move the displacement to -400.

- Press F5 again for a second Position Reset.

- Press F7 for Position Auto Set. This sets the automatic position reference for the test execution cycle.

- Turn the mode key switch (C) to Auto mode. The machine is now ready for test launch.

## 6.6  Test program creation and launch

### 6.6.1  Creating a new test file in MEditor

If a new test program file is required, complete the following procedure in MEditor before launching MORPHEE. If an existing test file is being used, skip to Section 6.6.2.

- On the Windows desktop, double-click the MEditor icon.

- In MEditor, open the required folder and select the reference file to base the new test on.

- Save the reference file with a new file name using Save As. Use the naming convention approved by the R&D team.

- Modify the required test parameters as per the test specification.

- Save the file. Close MEditor.

### 6.6.2  Launching the test in MORPHEE

- In MORPHEE, navigate to the test file folder and select the required test program file.

- Confirm the mode key switch (C) is in the Auto position.

- Confirm all three machine doors are closed and locked.

- Press the Validation button in MORPHEE.

- Once validation is confirmed, press the Start button. The test will start automatically.

[IMAGE PLACEHOLDER — Insert screenshot of MORPHEE Validation button screen. Source: Excel Testing sheet.]

## 6.7  Test monitoring

During automatic test execution, the operator must monitor the machine from the cabin workstation. The following checks must be performed at the start and end of each shift.

- Check the MORPHEE display — confirm torque, temperature, and RPM readings are within the expected range and no alarms are active.

- Check the dust collection exhaust system is running — confirm airflow at the exhaust outlet.

- Check the air pressure display is reading between 4 and 6 bar.

- Record the cycle count displayed in MORPHEE in the machine shift log.

- Check the area around the machine is clear and no persons have entered the 1 m clearance zone.

Do not enter the 1 m clearance zone around the machine while the test is running in automatic mode.

## 6.8  Inspection at 6,000 cycles

⚠ WARNING: Before opening the machine door for inspection, ensure the test program has been paused or stopped in MORPHEE and all rotating parts have come to a complete stop. Wait for the guard locking unit to release the door automatically — this confirms the internal temperature has reduced to a safe level.

⚠ CAUTION: Internal temperatures may be extremely high immediately after test completion. Do not touch internal components with bare hands. Wear heat-resistant gloves.

- At 6,000 cycles, pause or stop the test program in MORPHEE.

- Wait for all rotating parts to stop completely.

- Wait for the door guard locking unit to release automatically. Do not attempt to override the door lock.

- Open the machine door. Access the clutch assembly.

- Using compressed air, clean the friction disc to remove all test dust and debris. Wear appropriate respiratory PPE during this activity.

- Measure and record the wear depth of the friction disc.

- Visually assess the friction disc surface bedding percentage on both faces. Record the result.

- If the friction disc meets the acceptance criteria, reassemble the clutch (refer to Sections 6.4 Steps 3 to 9) and relaunch the test using Section 6.6.2.

- If the friction disc does not meet the acceptance criteria, remove the full clutch assembly (refer to Section 6.9) and hand scrapped components to the EHS team for disposal.

[IMAGE PLACEHOLDER — Insert photograph showing friction disc inspection at 6,000 cycles. Source: Excel Testing sheet.]

## 6.9  Tested clutch removal procedure

⚠ WARNING: This procedure must be performed in Manual mode only. Confirm the test program is fully stopped and all rotating parts are stationary before opening the machine door.

- Pause or stop the test program in MORPHEE. Confirm all motion has stopped.

- Wait for the guard locking unit to release automatically. Open the clutch assembly cover and door.

- Remove the bed fixing bolt.

[IMAGE PLACEHOLDER — Insert photograph showing bed fixing bolt location during clutch removal. Source: Excel Testing sheet, tested clutch removing procedure image.]

- Using the lead screw rotation wheel, move the clutch assembly bed in reverse until the drive shaft is disconnected.

- Disconnect the thermocouple plug.

- Remove the friction disc, cover plate, and flywheel from the test bed.

- Transfer the tested clutch assembly components to the quality inspection station.

- At the quality inspection station, measure and record: cover assembly clamping load, friction disc thickness, flywheel and disc roughness (Ra), flywheel flatness and concavity, cover plate flatness and concavity, and friction disc depth.

## 6.10  Machine stopping procedure

Follow this procedure at the end of the shift or when stopping the machine for any reason.

- If a test is running, pause or stop the test program in MORPHEE. Wait for all motion to stop.

- Wait for the guard locking unit to release automatically before opening any door.

- If the clutch assembly is to remain loaded for the next shift, close and secure all doors and leave the machine in the ready state.

- If the clutch assembly is to be removed, follow Section 6.9.

- At the cabin workstation, save all MORPHEE data files and close the MORPHEE software.

- On the operator panel, confirm the mode key switch (C) is in Manual. The key must be removed by Operator 2 and kept securely.

- At the machine control panel, turn the rotary isolator to the OFF position.

- At the main distribution panel SSB-02, switch OFF the Low Energy Bench supply breaker.

- At the exhaust line panel, switch OFF the motor starter.

- Record the shift log entry — cycle count, test status, any deficiencies or observations — and sign off.

# 7  Safety Procedures

## 7.1  General safety rules

The following safety rules apply to all persons who enter the LE Test Bench operating area at any time. These rules are non-negotiable and must be observed at all times regardless of whether the machine is running or stopped.

- Only authorised and trained personnel are permitted to operate, maintain, or service the LE Test Bench.

- No person may enter the 1 metre machine clearance zone during automatic operation.

- Safety covers, guards, and door interlocks must never be bypassed, defeated, or removed during machine operation.

- The machine must never be operated with a known fault. Any fault or abnormality observed must be reported immediately.

- No food, drink, or personal items are permitted in the machine area.

- Visitors must not enter the machine clearance zone without escort and EHS clearance.

## 7.2  Personal protective equipment

| PPE Item | Specification | When required | Who | Notes |
| --- | --- | --- | --- | --- |
| Safety glasses | EN166 / IS 5983 compliant, impact-resistant | All times in machine area | All personnel | Mandatory — no exceptions |
| Hearing protection | Ear plugs or ear muffs — SNR 25 dB minimum | During all test runs | All personnel in area | Machine generates significant noise during testing |
| Dust mask / FFP2 respirator | FFP2 or equivalent — glass fibre rated | Friction disc handling, 6,000 cycle inspection, compressed air cleaning | Operator 2 | Glass fibre dust is a health hazard |
| Heat-resistant gloves | Rated to minimum 300°C | Oven handling, post-test clutch removal | Operator 2 | Mandatory for all oven and clutch removal activities |
| Safety shoes | Steel-toed, oil-resistant sole | All times in machine area | All personnel | Clutch components can fall during loading/removal |
| Anti-static wrist strap | When handling NI DAQ equipment | Maintenance activities near NI cards | Maintenance person only | Prevents damage to NI equipment |

## 7.3  Emergency stop procedure

The LE Test Bench has three emergency stop buttons. Their locations are:

- E-stop 1: Operator panel — front face of machine enclosure

- E-stop 2: Left side of machine enclosure

- E-stop 3: Cabin workstation area

[IMAGE PLACEHOLDER — Insert photograph showing all three E-stop button locations on the machine. Source: SICK India risk assessment, Section 5, E-stop map diagram.]

### In the event of an emergency:

- Press the nearest E-stop button immediately. The machine will stop within seconds. The control circuit will de-energise and the red indicator lamp will illuminate.

- Shout a warning to any other persons in the area.

- If a person is in contact with the machine or trapped, do not touch the person — call for help immediately.

- Call the EHS Lead and the maintenance person using the emergency contact list (Section 7.9).

- Do not restart the machine until the cause of the emergency has been investigated and resolved.

### To reset after E-stop:

- Confirm the area is safe and all persons are clear of the machine.

- Identify and resolve the cause of the emergency stop.

- Twist all three E-stop buttons clockwise until they pop out and are fully released.

- Press the RESET button on the operator panel. The Pilz PNOZ X3 safety relay will reset.

- Press the Control ON button to re-energise the control circuit.

- Restart the test only after confirming that it is safe to do so and after recording the E-stop event in the machine log.

## 7.4  Door interlock system

All three machine doors are fitted with solenoid-type door interlock locks monitored by the Pilz PNOZ X3 safety relay. The following rules apply:

- In Automatic mode, all three doors must be closed and locked before the machine will start. The machine will not run if any door is open.

- In Manual mode, doors can be opened using the USER PW access level in MORPHEE by Operator 2. The operator must confirm the machine is not in motion before opening any door.

- Never attempt to force open a locked door. If a door lock fails to release, stop the machine and call the maintenance person.

- Never reach into the machine through a partially open door while the machine is running.

## 7.5  Lockout/Tagout (LOTO) procedure

The LOTO procedure must be performed before any maintenance activity, guard removal, or any task requiring physical access to the interior of the machine. The LOTO procedure involves two stages: tag-out (isolation and energy removal) and tag-in (restoration of energy).

### 7.5.1  Energy isolation points

| Energy source | Isolation point | Isolation method |
| --- | --- | --- |
| Electrical — main supply | Rotary isolator on machine control panel | Turn to OFF, apply padlock and LOTO tag |
| Electrical — mains supply | Main breaker at SSB-02 panel for Low Energy Bench | Switch OFF, apply padlock and LOTO tag |
| Pneumatic — compressed air | Pneumatic LOTO valve on air supply line to machine | Turn to CLOSED, apply padlock and LOTO tag, bleed residual pressure via exhaust valve |

### 7.5.2  Tag-out procedure (isolation — before maintenance)

- Inform the shift supervisor and EHS Lead that the machine is being isolated for maintenance.

- If the machine is running, stop it using the normal stopping procedure in Section 6.10.

- At the machine control panel, turn the rotary isolator to the OFF position.

- At the SSB-02 panel, switch OFF the Low Energy Bench supply breaker.

- Apply a personal padlock to the rotary isolator and to the SSB-02 supply breaker. Each person working on the machine must apply their own personal padlock.

- Attach a LOTO tag to each padlock. The tag must state: the name of the person who applied it, the date and time, the reason for isolation, and the contact number.

- At the pneumatic supply LOTO valve, turn the valve to the CLOSED/ISOLATED position. Operate the exhaust valve to bleed all residual pneumatic pressure from the system. Confirm zero pressure on the pressure gauge.

- Apply a padlock and LOTO tag to the pneumatic LOTO valve.

- Attempt to press the Control ON button and confirm the machine does not start.

- Record the LOTO tag-out details in the machine log before commencing any work.

### 7.5.3  Tag-in procedure (restoration — after maintenance)

- Confirm all maintenance work is complete and all tools have been removed from the machine.

- Confirm all guards and covers have been replaced and secured.

- Inform all persons working on the machine that power is about to be restored.

- Remove the pneumatic LOTO valve padlock and tag (by the person who applied it only). Open the pneumatic LOTO valve. Confirm pressure builds to the correct range (4 to 6 bar).

- Remove the SSB-02 breaker padlock and tag. Switch ON the Low Energy Bench supply breaker.

- Remove the rotary isolator padlock and tag. Turn the rotary isolator to the ON position.

- Perform the full pre-start verification checklist (Section 5) before returning the machine to service.

- Record the LOTO tag-in completion in the machine log, including the name of the person who restored energy and the time.

## 7.6  Reaction to abnormalities

In the event of the following abnormal conditions, the operator must take the actions listed below immediately.

| Abnormal condition | Immediate action | Secondary action | Do NOT |
| --- | --- | --- | --- |
| Total power failure | Remain calm. Do not press E-stop (already de-energised). Note cycle count from MORPHEE if visible before shutdown. | Call maintenance person. Record event in log. Wait for power restoration. | Attempt to manually force-open machine doors. |
| Partial power failure / brownout | Stop the test in MORPHEE if possible. Press E-stop. Switch OFF rotary isolator. | Call maintenance person. Do not restart until power quality is confirmed stable. | Restart immediately without investigating cause. |
| Fire at or near the machine | Press nearest E-stop. Shout warning to evacuate area. Activate fire alarm. | Follow Valeo evacuation procedure. Do not use water on electrical fire. Use CO2 extinguisher. | Re-enter the area until fire is confirmed extinguished and area is declared safe. |
| Fire alarm (general building) | Stop test in MORPHEE. Press E-stop. Evacuate by nearest exit. Leave machine — do not stop to shut down. | Muster at designated assembly point. Report to EHS Lead. | Re-enter building until all-clear is given by fire officer. |
| Smoke from machine or panel | Press E-stop immediately. Do not open panel doors. Evacuate area. | Call maintenance person and EHS Lead. Do not restart until cause is investigated. | Attempt to open control panel or machine enclosure to investigate smoke. |
| Clutch assembly separates or breaks during test | Machine should E-stop automatically via torque/RPM fault. If not, press E-stop immediately. Evacuate area. | Inspect machine enclosure only after all motion has stopped and machine is fully isolated per LOTO. Call maintenance person. | Enter machine area while any part may still be in motion. |

## 7.7  Hot zone and thermal hazards

⚠ WARNING: The LE Test Bench generates significant heat during testing. Internal temperatures can reach 300°C at the clutch assembly surface during sustained test runs. The oven used for cover plate preparation operates at 300°C. Thermal burns are a significant risk if correct PPE and procedures are not followed.

The following thermal hazard rules apply at all times:

- Heat-resistant gloves rated to 300°C minimum must be worn when handling components from the oven or when accessing the clutch assembly immediately after test completion.

- Do not touch the flywheel, cover plate, or friction disc with bare hands after a test run. Allow components to cool before handling.

- Do not attempt to open the machine door immediately after stopping the test. The guard locking unit will release the door automatically when the internal temperature has reduced to a safe level. Wait for the automatic release.

- Do not place any flammable materials within the 1 m clearance zone around the machine during testing.

## 7.8  Dust hazard

⚠ CAUTION: The friction disc is manufactured from glass fibre yarn infused with metal. Dust generated during testing and during the 6,000 cycle inspection is a significant respiratory hazard. An FFP2 respirator must be worn at all times during friction disc handling, cleaning with compressed air, and any activity that may disturb accumulated test dust.

The following dust hazard controls apply:

- The dust collection exhaust system must be running at all times during testing. Do not operate the machine if the exhaust system is not running.

- Never use compressed air to blow dust in an uncontrolled manner. When cleaning the friction disc at 6,000 cycles, direct compressed air into the exhaust extraction zone.

- Dispose of all accumulated test dust and used air filters through the Valeo EHS team. Do not dispose of in general waste.

- Wash hands thoroughly after all friction disc handling activities. Do not eat, drink, or touch the face while wearing contaminated gloves.

## 7.9  Emergency contacts

| Role | Contact |
| --- | --- |
| EHS Lead — Valeo Friction Materials India Pvt. Ltd. | [Insert name and phone number] |
| R&D Deputy Manager | [Insert name and phone number] |
| Maintenance Person (in-house) | [Insert name and phone number] |
| Third-party Specialist — New Craft Technology | [Insert name and phone number — Job No. 1639] |
| Plant Security | [Insert phone number] |
| Ambulance / Emergency services | 108 (Tamil Nadu) |
| Fire services | 101 |
| Local police | 100 |

## 7.10  Accident and near-miss reporting

All accidents, near-misses, and dangerous occurrences at or near the LE Test Bench must be reported to the EHS Lead immediately. The operator must complete the Valeo accident/near-miss report form and submit it to the EHS Lead within 24 hours of the event. The machine must not be restarted after any accident until the EHS Lead has cleared the machine for operation.

## 7.11  Training requirements

All operators and maintenance personnel must complete the following training before being authorised to work on or near the LE Test Bench.

| Training requirement | Frequency |
| --- | --- |
| Machine-specific operating procedure training (based on this manual) | Initial — before first operation; refresher bi-yearly |
| LOTO procedure training | Initial — before first maintenance activity; refresher bi-yearly |
| Emergency response procedure training (E-stop, fire, evacuation) | Initial; refresher bi-yearly |
| PPE usage and inspection training | Initial; refresher bi-yearly |
| Glass fibre dust hazard awareness | Initial; refresher bi-yearly |
| Thermal hazard awareness | Initial; refresher bi-yearly |
| [IMAGE PLACEHOLDER — Insert training record sign-off sheet. This should be designed as a physical log book page for the R&D cell.] |  |

# 8  Alarms and Troubleshooting

## 8.1  Overview

This section provides guidance for responding to alarms, faults, and abnormal conditions on the Low Energy Test Bench. For major electrical faults and VFD faults, the third-party maintenance specialist must be contacted. Operators must not attempt to investigate inside the control cabinet or machine enclosure without completing the LOTO procedure in Section 7.5.

## 8.2  Indicator lamp status guide

| Lamp colour | State | Meaning | Action required |
| --- | --- | --- | --- |
| Green | ON steady | Control circuit energised — machine ready | None — normal state |
| Green | OFF | Control circuit not energised | Check main power, rotary isolator, and Control ON button |
| Yellow | ON steady | Machine in automatic mode, test running | None — normal state during test |
| Yellow | Flashing | Fault or warning condition detected | Check MORPHEE display for fault code |
| Red | ON steady | Emergency stop activated or safety relay fault | Identify and resolve E-stop cause, then reset per Section 7.3 |
| Red | Flashing | VFD trip or motor protection fault | Refer to Section 8.4 |

## 8.3  Alarm and fault reference table

| Fault | Symptom | Likely cause | Who resolves | Corrective action |
| --- | --- | --- | --- | --- |
| Control circuit does not energise | Green lamp does not illuminate | Isolator OFF, MCB tripped, E-stop latched, or safety relay not reset | Operator | Check rotary isolator is ON. Check all MCBs are UP. Confirm all E-stops released. Press RESET then Control ON. |
| Safety relay not resetting | Red lamp stays ON after E-stop release | E-stop not fully released, door interlock open, or safety relay fault | Operator / Maintenance | Confirm all three E-stops are twisted out. Confirm all three doors are closed and locked. Check Pilz PNOZ X3 indicators. |
| VFD trip | Red lamp flashing, motor stops, VFD display shows fault code | Overcurrent, overvoltage, overtemperature, or phase loss | Operator (minor) / Maintenance (persistent) | Note VFD fault code. Refer to Section 8.4. Do not reset VFD repeatedly without investigating cause. |
| Gear motor MPCB trip | Bed movement not possible, MPCB tripped indicator | Motor overload, jam, or short circuit in bed traverse circuit | Maintenance | Do not attempt to reset MPCB without investigating cause. Call maintenance person. |
| Door interlock not engaging | Machine will not start in auto mode, door lock solenoid does not click | Door not fully closed, solenoid fault, or wiring fault | Operator (physical door) / Maintenance (solenoid) | Confirm door is physically fully closed. If solenoid fails to engage after confirming door closed, call maintenance. |
| Air pressure fault | Yellow lamp, MORPHEE fault display, machine stops or will not start | Supply pressure below 4 bar or above 6 bar, supply valve closed, or regulator fault | Operator (adjust pressure) / Maintenance (regulator fault) | Check supply valve is open. Adjust regulator to 4 to 6 bar. If pressure cannot be maintained, call maintenance. |
| MORPHEE will not start | Black screen or error on startup | Windows XP corruption, software fault, or NI card communication fault | Maintenance (IT / specialist) | Do not attempt to reinstall software. Record the error message. Call the New Craft Technology specialist. |
| Torque reading zero or incorrect | MORPHEE shows 0 DaN or erratic torque during test | Load cell disconnection, calibration drift, or NI card AI fault | Maintenance | Stop test. Do not use test data until torque is verified. Perform torque calibration check per Section 9.7.1. |
| Temperature reading zero or over-range | MORPHEE temperature channel shows 0 or maximum value | Thermocouple disconnection, damaged thermocouple, or NI card AI fault | Operator (check plug) / Maintenance (replacement) | Stop test. Check thermocouple plug is connected. If connected and reading is still incorrect, call maintenance. |
| Calibration non-conformance | Calibration check shows reading outside tolerance | Calibration drift, sensor damage, or measurement system fault | Maintenance / Third-party calibration vendor | Refer to Section 8.7. Do not use any test data recorded after the last confirmed valid calibration. |

## 8.4  VFD trip response procedure

- Note the fault code displayed on the VFD front panel display. Record the code and time in the machine log.

- Do not attempt to reset the VFD immediately. Allow 5 minutes for the drive to cool if an overtemperature fault is suspected.

- Check the following before resetting: confirm the motor is not stalled or jammed; confirm the supply voltage is stable; confirm the VFD cooling fan is running and the VFD ventilation slots are clear.

- If the fault is a simple overtemperature after a long run and conditions are now normal, press the VFD Reset button and re-start per Section 6.3.

- If the VFD fault code indicates overcurrent, phase loss, or an internal VFD error, do not reset. Complete the LOTO procedure in Section 7.5 and call the New Craft Technology specialist.

- Record all VFD fault codes, reset attempts, and outcomes in the machine log.

## 8.5  Machine log requirements

The machine log is the official operational record of the LE Test Bench. The following entries must be made in the machine log:

- Pre-start checklist sign-off at the start of each shift

- Test start: date, time, test file name, operator name

- Cycle count at start and end of each shift

- Any abnormal condition, fault, or alarm observed — including fault codes, times, and actions taken

- Any E-stop event — cause, investigation findings, and clearance confirmation

- Inspection results at 6,000 cycles — wear depth, bedding percentage, friction disc decision (continue or scrap)

- Maintenance activities performed — date, activity, and technician name

- Calibration check results — date, results, and pass/fail status

[IMAGE PLACEHOLDER — Insert machine log page template. To be designed and printed as a physical log book for the R&D cell.]

# 9  Maintenance and Calibration

## 9.1  Maintenance overview

Regular maintenance of the Low Energy Test Bench is essential to ensure accurate test results, continued safe operation, and long service life of the machine and its components. Maintenance activities are divided into two categories — routine maintenance performed by Operator 1 or Operator 2, and technical maintenance performed by the maintenance person or third-party specialist.

All maintenance activities must be preceded by completion of the LOTO procedure in Section 7.5 unless the activity is an external inspection that does not require access to any hazardous energy zone. All maintenance activities must be recorded in the machine log.

OPEN ACTION OA-005: Formal maintenance frequency intervals have not been confirmed by the customer. The intervals shown in this section are based on standard industrial practice and the site observations made during the risk assessment. Valeo Friction Materials India Private Limited must review and formally confirm or adjust all maintenance intervals before this manual is finalised. Responsible party: Valeo Friction Materials India Private Limited. Target close date: [Insert].

## 9.2  Maintenance schedule summary

| Activity | Frequency | Who performs | LOTO required? | Section reference |
| --- | --- | --- | --- | --- |
| Pre-start verification checklist | Every shift | Operator 1 or 2 | No | Section 5 |
| Air filter inspection | Weekly | Operator 1 or 2 | No (visual external) | Section 9.6 |
| Pneumatic filter bowl drain | Weekly | Operator 1 or 2 | No (external drain point) | Section 9.5 |
| General housekeeping — external machine clean | Weekly | Operator 1 or 2 | No | Section 9.8 |
| Air filter replacement | Monthly or as required (when blocked) | Operator 2 with maintenance person | Yes | Section 9.6 |
| Torque calibration check | Every 6 months (or before any new test series) | Maintenance person + NABL calibration vendor | No (with machine stopped) | Section 9.7.1 |
| Temperature calibration check | Every 6 months | Maintenance person + NABL calibration vendor | No (with machine stopped) | Section 9.7.2 |
| RPM calibration check | Every 6 months | Maintenance person + NABL calibration vendor | No (with machine stopped) | Section 9.7.3 |
| Dust collection duct and fan inspection | Every 6 months | Maintenance person | Yes | Section 9.5 |
| Electrical cabinet inspection — connections, insulation, component condition | Every 6 months | New Craft Technology specialist (third party) | Yes | — |
| Pilz PNOZ X3 safety relay function test — full dual-channel test | Every 12 months | New Craft Technology specialist (third party) | Yes | — |
| Pneumatic cylinder and actuator inspection — seals, fittings, hose condition | Every 12 months | Maintenance person / New Craft Technology | Yes | — |

## 9.3  Spare parts list

| Part description | Part number / specification | Recommended quantity on hand | Source | Notes |
| --- | --- | --- | --- | --- |
| Air filter — dust collection system | [Insert filter make, model, and dimensions from filter housing label] | 2 | OEM / local industrial supplier | Replace when blocked or monthly |
| Thermocouple — K-type, 4.8 mm diameter | [Insert thermocouple specification from NI card datasheet] | 2 | NI / local instrument supplier | Replace if damaged during drilling or fitting |
| M8 bolt — cover plate assembly | [Insert grade and length specification] | 10 | Local industrial supplier | Replace after each test if worn |
| M5 bolt — flywheel to adapter | [Insert grade and length specification] | 10 | Local industrial supplier | Inspect after each test |
| M10 slinckey fixing bolt — bed | [Insert specification] | 2 | New Craft Technology / machine OEM | Keep one spare |
| Pilz PNOZ X3 safety relay | Pilz PNOZ X3 (Part No. 777303 — verify against installed unit) | 1 | Pilz India distributor | Keep one spare for rapid replacement |
| NI USB-6001 DAQ controller | National Instruments USB-6001 | 1 | NI India / authorised reseller | One spare — lead time may be long |
| VFD — main drive | [Insert VFD make, model, and kW rating from nameplate — refer OA-002] | 1 (where budget allows) | VFD manufacturer Indian distributor | Check lead time |
| Pneumatic cylinder seals kit | [Insert make and bore size from cylinder nameplate] | 1 kit | Pneumatic cylinder manufacturer | Replace seals at 12-month inspection |

## 9.4  Dust collection system inspection

The dust collection exhaust system must be inspected every 6 months by the maintenance person under LOTO. The inspection must cover the following:

- Exhaust ductwork — inspect for cracks, loose joints, or blockages

- Exhaust fan motor — check bearing condition, listen for abnormal noise, check current draw against nameplate rating

- Duct connection points at machine enclosure — confirm tight seals, no air leakage

- Exhaust outlet — confirm discharge is unobstructed and directed away from personnel areas

## 9.5  General housekeeping

The following housekeeping activities must be performed weekly:

- Wipe down the external surfaces of the machine enclosure with a dry or slightly damp cloth. Do not use water spray on the machine.

- Clear the machine area of any accumulated test dust on the floor. Use a vacuum cleaner — do not use compressed air to blow floor dust.

- Drain any accumulated water from the pneumatic filter bowl using the drain point at the bottom of the filter unit.

- Inspect the 1 m clearance zone around the machine and remove any tools, components, or obstructions.

## 9.6  Air filter replacement procedure

⚠ CAUTION: Complete the LOTO procedure in Section 7.5 before performing this procedure. Isolate both electrical and pneumatic supplies before accessing the filter housing.

- Complete the full LOTO procedure. Confirm zero energy state — no electrical supply, no pneumatic pressure.

- Locate the filter housing access panel on the machine enclosure.

[IMAGE PLACEHOLDER — Insert photograph showing filter housing access panel location on machine exterior. Source: Site photograph to be taken during next maintenance visit.]

- Open the filter housing access panel.

- Remove the used air filter. Handle carefully — the filter will contain glass fibre and metal-infused dust. Place the used filter immediately into a sealed plastic bag for disposal. Do not shake or blow out the used filter.

- Insert the new filter in the correct orientation. Refer to the directional arrow marked on the filter frame.

- Close and secure the filter housing access panel.

- Complete the LOTO tag-in procedure per Section 7.5.3 to restore power and pneumatic supply.

- Start the machine per Section 6.3 and confirm the exhaust system is running correctly.

- Record the filter replacement in the machine log including the date, filter reference, and operator name.

Disposal of used filters: Used air filters containing glass fibre and metal dust must be disposed of as industrial waste via the Valeo EHS team. Do not dispose of used filters in general waste.

## 9.7  Calibration procedures

Calibration of the LE Test Bench measurement systems must be performed every 6 months by a NABL-accredited third-party calibration vendor. The calibration covers three parameters: torque, temperature, and RPM. The actual calibration must be performed by a qualified calibration engineer using certified reference instruments.

Before calibration begins, open the MORPHEE file, click Moniteurs, and select the Consignes file to access the calibration verification screens.

### 9.7.1  Torque calibration

[IMAGE PLACEHOLDER — Insert photograph of torque calibration setup showing 1M arm bar mounted on machine spindle with calibration masses. Source: Excel Quality Instruction sheet, calibration procedure image 1.]

- Fix the 1 metre arm bar to the machine spindle as per the approved calibration setup photograph.

- Apply calibration masses in the following sequence: 5 kg, 15 kg, 25 kg, 35 kg, and 45 kg.

- For each mass, record the force value displayed on the MORPHEE calibration screen.

- Compare with the calculated reference value: mass (kg) × 9.81 N/kg × arm length (1 metre), converted to DaN.

- The MORPHEE display value must be within the acceptable tolerance for each mass. Tolerance: [Insert — to be confirmed with calibration vendor].

- Record all readings in the calibration log.

[IMAGE PLACEHOLDER — Insert calibration data recording sheet for torque calibration. Source: Excel Quality Instruction sheet, calibration data sheet image.]

### 9.7.2  Temperature calibration

[IMAGE PLACEHOLDER — Insert photograph of temperature calibration setup showing Fluke reference instrument connected to calibration heat source. Source: Excel Quality Instruction sheet, calibration procedure image 2.]

- Connect the calibrated Fluke reference thermometer to the thermocouple input point.

- Apply reference temperatures in the following sequence: 0°C, 40°C, 80°C, 100°C, 200°C, and 300°C using the approved calibration heat source.

- For each reference temperature, record the temperature displayed on the MORPHEE calibration screen.

- Compare each MORPHEE reading with the reference thermometer reading.

- The MORPHEE display value must be within ±2°C of the reference thermometer at each test point.

- Record all readings in the calibration log.

### 9.7.3  RPM calibration

- Using the calibrated reference tachometer, measure the actual machine spindle speed at the following setpoints: 500 RPM, 1000 RPM, and 1500 RPM.

- For each setpoint, compare the MORPHEE speed display with the tachometer reading.

- The MORPHEE RPM display must be within ±10 RPM of the tachometer reading at each test point. Enter the manual RPM in the CVM field in MORPHEE calibration screen to verify.

- Record all readings in the calibration log.

## 9.8  Third-party maintenance specialist

The following activities must only be performed by New Craft Technology or another approved third-party electrical specialist. These activities must not be performed by Valeo operators or the in-house maintenance person.

- Any work inside the electrical control cabinet

- VFD parameter modification or VFD replacement

- NI DAQ card replacement or MORPHEE software reinstallation or re-configuration

- Safety relay (Pilz PNOZ X3) replacement

- Any modification to the control system wiring or logic

Before the third-party specialist arrives, the maintenance person must prepare: the machine log showing recent fault history, any MORPHEE non-conformance reports, any VFD fault codes recorded, and the electrical drawings (NT1639-NEW CRAFT WIRING, Rev R0, December 2025). A work permit must be issued by the Valeo EHS Lead before the third-party specialist commences any maintenance activity.

# 10  Commissioning and Decommissioning

## 10.1  Purpose

This section provides guidance for the initial commissioning of the Low Energy Test Bench at its current installation at Valeo Friction Materials India Private Limited, Chennai, and for the formal decommissioning and shutdown of the machine when it reaches end of service life or requires relocation.

OPEN ACTION OA-006: A formal commissioning protocol was not provided by the customer at the time of manual preparation. The checklist in Section 10.2 has been developed based on the known control system architecture, safety requirements, and site observations. Valeo Friction Materials India Private Limited must review, validate, and formally approve this commissioning checklist before the machine is put into production service. Responsible party: Valeo Friction Materials India Private Limited / New Craft Technology. Target close date: [Insert].

## 10.2  Commissioning checklist

| Item | Activity | Responsible | Witness | Verified | Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirm machine is mechanically complete and all guards are in position | New Craft Technology | Valeo EHS Lead | ☐ |  |
| 2 | Confirm all electrical connections are tight and correctly terminated per NT1639 drawings | New Craft Technology | Valeo EHS Lead | ☐ |  |
| 3 | Confirm Guard G004 (acrylic busbar guard) is fabricated and installed — OA-003 | New Craft Technology / Valeo | Valeo EHS Lead | ☐ |  |
| 4 | Confirm overvoltage surge protection device is installed — OA-004 | Valeo / New Craft Technology | Valeo EHS Lead | ☐ |  |
| 5 | Confirm motor nameplate data has been recorded — OA-002 | Valeo | R&D Deputy Manager | ☐ |  |
| 6 | Confirm machine asset number has been confirmed — OA-001 | Valeo | R&D Deputy Manager | ☐ |  |
| 7 | Confirm compressed air supply is available at 4 to 6 bar | New Craft Technology | Valeo EHS Lead | ☐ |  |
| 8 | Confirm dust collection exhaust system is connected and operational | New Craft Technology | Valeo EHS Lead | ☐ |  |
| 9 | Confirm all three E-stop buttons function correctly — Pilz PNOZ X3 trips on each button press | New Craft Technology | Valeo EHS Lead | ☐ |  |
| 10 | Confirm all three door interlocks function correctly — machine will not start with any door open | New Craft Technology | Valeo EHS Lead | ☐ |  |
| 11 | Confirm VFD parameters are set correctly and motor runs at correct speeds | New Craft Technology | R&D Deputy Manager | ☐ |  |
| 12 | Confirm NI DAQ cards are communicating with MORPHEE — all channels reading correctly | New Craft Technology | R&D Deputy Manager | ☐ |  |
| 13 | Confirm torque calibration — perform calibration check per Section 9.7.1 | Calibration vendor | R&D Deputy Manager | ☐ |  |
| 14 | Confirm temperature calibration — perform calibration check per Section 9.7.2 | Calibration vendor | R&D Deputy Manager | ☐ |  |
| 15 | Confirm RPM calibration — perform calibration check per Section 9.7.3 | Calibration vendor | R&D Deputy Manager | ☐ |  |
| 16 | First test run validation — refer Section 10.3 | Operator 2 + New Craft Technology | R&D Deputy Manager + EHS Lead | ☐ |  |

## 10.3  First test run validation

After all commissioning checklist items are completed, a first test run must be performed using a reference clutch assembly to validate all machine systems before any R&D test data is recorded.

- Load a reference clutch assembly as per Section 6.4.

- Set displacement as per Section 6.5.

- Launch a short reference test program (minimum 500 cycles) in MORPHEE.

- Monitor torque, temperature, and RPM readings throughout — confirm all readings are within the expected range.

- At the end of the reference test, confirm that the MORPHEE data file has been correctly recorded and saved.

- Confirm that door interlocks functioned correctly throughout the test — the machine stopped immediately when the door was tested (under controlled condition) during a separate E-stop test performed before the first run.

- Record the first test run results in the commissioning sign-off form.

- R&D Deputy Manager and EHS Lead must both sign the commissioning checklist before the machine is cleared for production R&D testing.

## 10.4  Decommissioning and isolation procedure

When the LE Test Bench is to be permanently decommissioned, relocated, or taken out of service for an extended period, the following procedure must be followed.

- Complete the normal machine stopping procedure per Section 6.10.

- Complete the full LOTO procedure per Section 7.5 and apply padlocks and tags to all energy isolation points.

- Disconnect the compressed air supply line at the machine inlet. Bleed all residual pressure.

- At the main distribution panel SSB-02, isolate and lock off the LE Test Bench supply breaker.

- Post a 'Machine Decommissioned — Do Not Operate' sign on the machine and at the cabin workstation.

- Notify the EHS Lead, R&D Deputy Manager, and New Craft Technology.

- Arrange for the removal of the machine by a qualified rigger or millwright. Confirm lifting capacity before using any lifting equipment.

[IMAGE PLACEHOLDER — Insert machine lifting point diagram or photograph showing correct sling positions, if available from New Craft Technology or original machine records.]

## 10.5  Long-term shutdown procedure

If the machine is to be shut down for a period exceeding 30 days without decommissioning — for example during a plant shutdown, facility upgrade, or extended maintenance period — the following long-term shutdown procedure must be followed.

- Complete the normal machine stopping procedure per Section 6.10.

- Complete the full LOTO procedure per Section 7.5 and leave the padlock and tags in place for the duration of the shutdown.

- Disconnect the compressed air supply line at the machine inlet.

- Post a 'Machine Under Shutdown — Do Not Operate' sign on the machine and at the cabin workstation.

- Notify the EHS Lead and record the shutdown in the machine log including the expected restart date.

Before restarting after a long-term shutdown, complete the full pre-start verification checklist in Section 5 and perform a calibration check per Section 9.7 before any test data is recorded.

## 10.6  Disposal of test components

Clutch assembly components that are scrapped after testing must not be disposed of in general waste. The friction disc is manufactured from impregnated glass fibre yarn infused with metal and must be treated as industrial waste. All scrapped test components must be collected and handed to the Valeo EHS team for disposal in accordance with the Factories Act 1948 and applicable Tamil Nadu state environmental regulations.

A disposal record must be maintained by the EHS team showing the component type, quantity, test reference, and disposal date and method.

# Open Actions Register

This register consolidates all open actions identified during the preparation of this manual. Each open action must be formally closed by the responsible party before this manual is considered complete and ready for distribution. The status of each open action must be reviewed at the commissioning sign-off meeting (Section 10.2).

| OA No. | Section ref. | Description | Responsible party | Target close date | Status |
| --- | --- | --- | --- | --- | --- |
| OA-001 | 2.1 | Machine nameplate data not available — serial number, model number, and original manufacturer not confirmed. Machine is a retrofit from another Valeo facility with no nameplate. | Valeo Friction Materials India Pvt. Ltd. | [Insert] | Open |
| OA-002 | 2.2 | Main drive motor electrical ratings (kW, voltage, full load current, RPM) and gear motor ratings not available in any provided documentation. Must be verified from motor nameplate or electrical panel records. | Valeo Friction Materials India Pvt. Ltd. | [Insert] | Open |
| OA-003 | 2.6, 10.2 | Busbar acrylic guard G004 dimensions recorded as 'xxx mm' in risk assessment. Fabrication status and actual dimensions not confirmed. Must be confirmed at commissioning. | Valeo Friction Materials India Pvt. Ltd. / New Craft Technology | [Insert] | Open |
| OA-004 | 2.6, 10.2 | Overvoltage surge protective device recommended in risk assessment but not confirmed as installed. Valeo must confirm installation status or formally accept risk. | Valeo Friction Materials India Pvt. Ltd. | [Insert] | Open |
| OA-005 | 9.2 | Formal maintenance frequency intervals have not been confirmed by the customer. Intervals shown in Section 9 are based on standard practice. Valeo must review and formally confirm all intervals. | Valeo Friction Materials India Pvt. Ltd. | [Insert] | Open |
| OA-006 | 10.1 | Formal commissioning protocol was not provided by the customer. Section 10.2 checklist has been developed from available data. Valeo must review and formally approve the commissioning checklist. | Valeo Friction Materials India Pvt. Ltd. / New Craft Technology | [Insert] | Open |
| OA-007 | 1.6 | Document number format not confirmed. Valeo must assign a formal document number in accordance with Valeo's document control procedure. | Valeo Friction Materials India Pvt. Ltd. | [Insert] | Open |

# Abbreviations

| Abbreviation | Full form |
| --- | --- |
| AI | Analogue Input |
| AO | Analogue Output |
| B11 LMSS | ANSI B11 — Machine Safety Standards (USA) |
| DaN | Deca-Newton (1 DaN = 10 Newtons) |
| DAQ | Data Acquisition |
| DI | Digital Input |
| DO | Digital Output |
| EHS | Environment, Health, and Safety |
| E-stop | Emergency Stop |
| FFP2 | Filtering Face Piece — protection level 2 (respirator classification) |
| HMI | Human–Machine Interface |
| IEC | International Electrotechnical Commission |
| IS | Indian Standard |
| ISO | International Organization for Standardization |
| LE Test Bench | Low Energy Test Bench |
| LOTO | Lockout/Tagout |
| MCB | Miniature Circuit Breaker |
| MPCB | Motor Protection Circuit Breaker |
| NABL | National Accreditation Board for Testing and Calibration Laboratories |
| NI | National Instruments |
| OA | Open Action |
| PLC | Programmable Logic Controller |
| PPE | Personal Protective Equipment |
| R&D | Research and Development |
| RPM | Revolutions Per Minute |
| SMPS | Switch Mode Power Supply |
| SSB | Sub-switchboard |
| USB | Universal Serial Bus |
| VFD | Variable Frequency Drive |